3GPP TSG-RAN WG2 Meeting #130	R2-2503599

St Julian’s, Malta, 19th – 23th May, 2025

Agenda Item:	8.9.3

Source: 	TCL

Title:  	Discussion on UL Capability Enhancement for IOT NTN

Document for:	Discussion and Decision

- Introduction

In Rel-19 IoT NTN WID [1], UL Capability enhancement is one of the objectives.

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

    - Study and specify RRM requirement, if identified [RAN4]

This contribution would like to provide our considerations on the pending issues of UL Capability enhancement.

- Discussion on UL Capability Enhancement

During R2#129b, the details of msg4 for CB-Msg3 procedure has been discussed [2] and some issues are still pending.

The first pending issue is the rapporteur proposed that a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution. The reason is Like legacy EDT, for some CB-Msg3 transmission, especially CP solution, the RRC message in Msg4 is not mandatory. In this case, a Msg4 with only MAC CE(s) should be allowed. It is referred to as L2 ACK.

In the legacy 4-step RACH procedure, Msg4 must contain both the Contention Resolution Identity MAC CE and an RRC message (e.g., RRCConnectionSetup). The Contention Resolution MAC CE resolves contention conflicts among multiple UEs, ensuring that only the legitimate UE proceeds with subsequent procedures. It operates at the MAC layer to complete contention resolution without involving UE resource configuration. In contrast, the RRCConnectionSetup allocates necessary configuration parameters for the UE to transition to the RRC_CONNECTED state, such as activating ciphering and integrity protection, defining SRBand DRB  and configuring neighbor cell measurements or handover parameters. This ensures the UE transitions from RRC_IDLE or RRC_INACTIVE to RRC_CONNECTED.

In the PUR procedure, if no downlink data is present, the (ng-)eNB may terminate the procedure by transmitting a Layer 1 acknowledgement (optionally including a Time Advance Command), a MAC Time Advance Command, or an RRCEarlyDataComplete without user data. This implies that Layer 1 signalling, MAC CE, or RRC messages can individually terminate the procedure. One rationale is that the EDT procedure does not require the UE to enter the RRC_CONNECTED state, thus eliminating the need for RRC messages to deliver full configuration parameters—a simple acknowledgement suffices.

Similarly, the CB-Msg3 procedure may not strictly require RRC messages for termination. However, since CB-Msg3 is contention-based, the Contention Resolution MAC CE just like the 4-step RACH procedure remains necessary to resolve potential contention among multiple UEs.

Therefore, it is proposed that the Contention Resolution MAC CE is mandatory for Msg4, while the RRC message is not mandatory for Msg4. This ensures contention resolution reliability across scenarios while allowing flexibility in Msg4 content adaptation (e.g., using MAC CE or Layer1 signalling for termination) depending on service requirements.

- For CB- Msg3 procedure, the Contention Resolution MAC CE is mandatory for Msg4, while the RRC message is not mandatory for Msg4.

The second pending issus is the rappoteur proposed that “UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. FFS on the condition for the UE to initiate the fallback to legacy 4-step RA.” The reason is that If the CB-Msg3 cannot succeed, UE can try the legacy 4-step RA. Or when the network find that the Msg3 cannot be decoded due to the bad timing alignment, network can indicate the correspond UEs to stop current procedure, and initiate legacy 4-step RA instead. A new MAC CE in Msg4 can be introduced for this.

When the UE consistently fails to complete the CB-Msg3 procedure due to issues such as timing misalignment, falling back to a reliable fallback procedure remains effective. Therefore, it is recommended to support the capability for the UE to initiate the legacy 4-step Random Access upon CB-Msg3 procedure failure.

- RAN2 to confirm that UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- For CB- Msg3 procedure, the Contention Resolution MAC CE is mandatory for Msg4, while the RRC message is not mandatory for Msg4.

- RAN2 to confirm that UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails.

4. Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek

[2] R2-2503059， Report of [AT129bis][305][R19 IoT NTN] CB-msg3-EDT (Mediatek)