3GPP TSG-RAN WG2 #130	R2-2503601

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.4

Source:	Ericsson

Title:	A-IoT Data Transmission Aspects

Document for:	Discussion/Decision

# 1	Introduction

This paper discusses some of remaining aspects of data transmission functionality and other general aspects required for A-IoT devices, which are listed in the email discussion [POST129bis][017][AIoT] 38.391 Running CR (Huawei) [4]. In particular, the discussion covers open aspects related to message content and MAC PDU format, data (re)transmission, segmentation/reassembly, and AS ID handling.

# 2	Discussion

## 2.1	MAC PDU format and message content

Regarding the open issue 3-5 in the OI list [4], whether to support D2R message type, we think Msg1 in CBRA contains RN16 and thus can be distinguished from other D2R message including first D2R message in CFRA; given that all other D2R messages are larger in size. Meanwhile, other D2R message rather than CBRA Msg1 can have the same format considering their content of having upper layer data, UL control information (e.g., segmentation support). We note that Msg3 for inventory report also includes upper layer data (device ID), similar to the other D2R message after Msg3 that includes a NAS container. Thus, we think there is no need of the message type field in the MAC header for D2R. If this construct is possible, the message size would be small.

No need of message type field in D2R message.

RAN2 assumed that segmentation is only needed for D2R messages other than Msg3 and Msg1, e.g., Msg5. However, it is unclear if Msg3 needs to carry security information in support of the challenge-response authentication mechanism if defined (as in SA3 TR33.713). In case Msg3 needs to carry also security information in support of authentication procedure, RAN2 needs to wait for progress from SA3 to decide if segmentation is needed for Msg3 or not.

Msg3 D2R upper layer data transfer may need to carry security information in support of authentication procedure thus may require segmentation.

Depending on progress from SA3, RAN2 may need to revisit the segmentation support for Msg3 D2R upper layer data transfer, i.e., in case additional security information is included.

Figure 1 shows an example of MAC PDU content for CBRA Msg1 and other D2R messages, respectively. Note that this example does not comprehensively list options, e.g. for where a smaller optimized size could be made, due to e.g. a fixed size payload part or where a random ID is not needed.

Figure 1: Example of MAC PDU format for D2R messages.

RAN2 already agreed that the 1-bit indication is sufficient to indicate whether more D2R data will be sent or not. However, to enable the reader to estimate the remaining size of next segment(s) to allocate radio resources, especially in case expected D2R message size (e.g., Msg5 in case of read command) is not provided by CN, we think the 1-bit indication may not be that practical. Currently, for length field we foresee two options and the comparison including pros and cons in provided the table 2 below.

Table 1: Length field options

| Option 1 | Option 2 |
| --- | --- |
| - Length field indicates the size of the MAC SDU with variable size. - In case of segmented transmission, the reporting over length field must be provided in first segment; this enables reader to allocate resources efficiently for successive segments - The length field would be practically not useful for remaining segments if first segmented had indicated total upper layer data size - The reader needs to derive resources for various as per indicated total upper data size from device | - Length field indicates the size of included payload in current D2R transmission - In case of segmented transmission, the information carried in the length field for last segment is very crucial, as the PDU may contain padding in addition to payload, so the length field reporting enable network to identify payload in PDU - The length field usage in other non-last segments may not be practically important, as the PDU expected to be filled with payload without padding as there is remaining data (to be transmitted over next segment(s)) - The device needs to calculate value of length especially for last segment |

Regarding the open issue 3-4 in [4], i.e., FFS on how length field in the D2R with variable MAC SDU is used. We think the length field can be used to indicate the size of MAC SDU. Given that the granted size for the D2R is provided by reader in the preceding R2D message, together with the value of the length, reader should be able to infer if/how many padding bits are included. In addition, we think 7 bits are sufficient for the length given that D2R message is 1000 bits or less.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

The length field indicates the size of the D2R MAC SDU with variable size

Use 7 bits for the length field in MAC PDU for D2R.

On aspects of retransmission, there is an open issue 3-2 in [4], i.e., FFS whether offset is always included.

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

We believe the behavior of segmented and non-segmented D2R transmission should be similar, and in both cases the offset is indicated. A dynamic inclusion of offset, e.g., only in segmented transmission may result in increased complexity compared to a unified behavior. For non-segmented transmission or retransmission, offset can be set to zero. This applies also to the case of first segment retransmission.

An unsegmented transmission can be retransmitted in similar manner as segmented transmission by indicating a zero offset.

Moreover, it is FFS whether reader always include the command for retransmission of segment (open issue 3-1 in [4]). We think upper layer command needs to be included in the R2D message scheduling the retransmission for first segment or unsegmented D2R. However, non-first segments, there is no need to include upper layer command in the R2D message.

R2D message scheduling non-first segment (re)transmission does not include upper layer command.

On handling erroneous cases where a segment fails, the reader can trigger retransmission for the failed segment or re-access. For re-access, the device always begins the transmission from the scratch, i.e., with the first segment if device passes contention. For segmented transmission, if re-access is triggered, the device begins transmission from the scratch (or with a first segment if segmentation needed) in a new occasion after passing contention.

In case of segmentation, 1-bit indication from device is sufficient to manage resources except for allocation resources of last segment.

The device ID selection behavior in Msg3 is not settled in SA3. This may pose difficulty related to offset based mechanism for segmented transmission.

For segmented transmission, if re-access is triggered, the device begins transmission from the scratch (or with a first segment if segmentation needed) in a new occasion after passing contention.

In RAN2-129 meeting, there was an FFS for multicast R2D messages other than Msg2, which needs to be addressed. We think R2D messages after Msg2 are all in the nature of unicast messages, i.e., targeting only one device. However, reader can also send a R2D message in multicast manner to reduce overhead, e.g., a common acknowledgement in Msg4 to multiple devices after their Msg3, or a common command request to multiple devices. However, in this case, the size of such R2D message might be very large considering control information in Msg4, e.g., AS ID assignment, RN16 for addressing, security information for authentication. In addition, a device that has sent Msg3 may have to wait long until receiving a common Msg4. For simple design, we think there is no need of multiplexing of information for multiple devices in R2D message after Msg2.

Multiplexing of information for multiple devices in R2D message after Msg2 is not supported.

Four R2D message types have been identified: A-IoT Paging message, Access Occasion Trigger message, Random ID Response message (Msg2), R2D Upper Layer Data Transfer message.

Aim is to restrict to fewer message types, and utilize a given message for multiple purposes. E.g., Random ID Response message Msg2 can be utilized to perform/indicate initial or re-transmission of Msg3 with or without segments, or NACK to trigger re-access.

RAN2-129bis has agreed an explicit 1-bit indication to indicate whether it is CFRA or CBRA per paging message. It is our understanding that the same paging message format is used for both CBRA and CFRA. Regarding the open issue 1-5 [4] on whether CFRA paging can omit some CBRA related fields such as transaction ID, indication of paging ID present or absence, number of access occasion [4]. Considering only single-device CFRA, we think there are potential fields of a paging message, as listed in the table 2 below. As the reader determines which particular device(s) to send CFRA paging message, there is no need of having transaction ID to allow target device(s) to avoid duplicate response. Similarly, the number of access occasions is only relevant for CBRA, and thus not needed in CFRA paging message. In addition, in case of CFRA, there is always device ID (paging identifier), and thus the indication of paging ID present or absent is not needed.

A-IoT paging message may need to include security information to support device authentication, pending SA3 progress.

RAN2 should wait for SA3 progress on whether paging message needs to include security information for authentication.

A-IoT paging message for CFRA can omit the paging ID presence/absence indicator, transaction ID, and number of access occasions.

We think that the unified message format for both CFRA and CBRA might needs to be revisited if RAN2 agree the support of multi-device CFRA. In this case, the paging message would need to include multiple components each consisting of a paging ID and authentication parameter for an individual device.

In case multi-device CFRA is supported, RAN2 to revisit if paging message format for CFRA and CBRA can be unified or not.

Table 2: Content of A-IoT paging message

| A-IoT paging field | Field length (bits) | Mandatory or optional | Comment |
| --- | --- | --- | --- |
| R2D message type | 3 bits | Mandatory |  |
| Random access type | 1 bit | Mandatory |  |
| Paging ID (PID) presence indicator | 1 bit | Optional | Not present in case of CFRA |
| Length of PID | ? | Optional | Pending SA2 progress, not present in case no PID is indicated |
| Paging ID (PID) | ? | Optional | Not present in case no PID is indicated |
| Transaction ID | ? | Optional | Not present in case of CFRA |
| Number of access occasions | ? | Optional | Not present in case of CFRA |
| Authentication parameter | ? | ? | Pending SA3 progress |

It was agreed in RAN2-129 that MAC PDU for the R2D message with variable size contains message type field, a length field for the variable part. This format is for R2D from Msg4 that carries DL NAS data, i.e., command request received from CN and possibly other NAS information. But it remains unclear the size of this length field. In addition, depending on whether the R2D message contains any security related information, either at NAS level or MAC level, other potential DL control information may be needed, pending SA3 progress. Considering the upper bound of 1000 bits TBS, we think 7 bits are sufficient to indicate the size in bytes of the variable part.

Support 7-bit length field in MAC PDU of R2D message with NAS information of variable size.

Regarding the PDU format for the Random ID response message (Msg2), given that Msg2 can be used to echo multiple devices sending RN16 in Msg1, and there can be AS ID assignment or not for each of the devices. To facilitate the processing/parsing at device, one could assume it is beneficial to have a field indicating how many RN16s are present, but as other parts of the MAC PDU are fixed, these can be parsed one by one (OI in RA). Finally, there should be a flag indicating presence/absence indicator for AS ID. For each of RN16, there is scheduling information for msg3 transmission, as detailed in [8].

MAC PDU for the Random ID Response message (Msg2) includes:

- R2D message type

- One or multiple RN16s subject to message size limitation, for each of RN16 there are associated control information, including

  - RN16 value

  - Presence/absence indicator for AS ID (in case of AS ID assignment)

  - Scheduling information for Msg3

## 2.2	AS ID

There is an open aspect on whether to specify any additional AS ID release method in addition to the reception of paging message and when device triggers a new Msg1 (i.e., issue 3-3 in [4]). We think no need for device and network to keep AS ID after completion of the ongoing inventory + command procedure. Otherwise, the requirement on device side to keep the AS ID across access/paging round would imply significant device complexity unnecessarily. In addition, the need of addressing and scheduling is only valid among the devices accessing to the same access occasion.

One related topic being in the email discussion is how device determines the completion of inventory + command procedure. Given that reader anyway needs to send R2D messages for Msg1 resource determination, e.g., access occasion trigger message. We think that this signalling can be used as an indication for the completion of the inventory + command procedure and thus the device can release AS ID, if available. Note that in case of re-access (upon reception of a NACK on D2R transmission), the procedure is considered completed even though not successfully completed. Therefore, the reception of an access occasion trigger message should be used as the indication for procedure completion.

For CBRA, from a device’s perspective, AS ID is valid until completion of the inventory + command procedure (Option 4b).

Device considers procedure completed upon reception of an access occasion trigger message.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Msg3 D2R upper layer data transfer may need to carry security information in support of authentication procedure thus may require segmentation.

Observation 2	In case of segmentation, 1-bit indication from device is sufficient to manage resources except for allocation resources of last segment.

Observation 3	The device ID selection behavior in Msg3 is not settled in SA3. This may pose difficulty related to offset based mechanism for segmented transmission.

Observation 4	For segmented transmission, if re-access is triggered, the device begins transmission from the scratch (or with a first segment if segmentation needed) in a new occasion after passing contention.

Observation 5	Four R2D message types have been identified: A-IoT Paging message, Access Occasion Trigger message, Random ID Response message (Msg2), R2D Upper Layer Data Transfer message.

Observation 6	Aim is to restrict to fewer message types, and utilize a given message for multiple purposes. E.g., Random ID Response message Msg2 can be utilized to perform/indicate initial or re-transmission of Msg3 with or without segments, or NACK to trigger re-access.

Observation 7	A-IoT paging message may need to include security information to support device authentication, pending SA3 progress.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No need of message type field in D2R message.

Proposal 2	Depending on progress from SA3, RAN2 may need to revisit the segmentation support for Msg3 D2R upper layer data transfer, i.e., in case additional security information is included.

Proposal 3	The length field indicates the size of the D2R MAC SDU with variable size

Proposal 4	Use 7 bits for the length field in MAC PDU for D2R.

Proposal 5	An unsegmented transmission can be retransmitted in similar manner as segmented transmission by indicating a zero offset.

Proposal 6	R2D message scheduling non-first segment (re)transmission does not include upper layer command.

Proposal 7	Multiplexing of information for multiple devices in R2D message after Msg2 is not supported.

Proposal 8	RAN2 should wait for SA3 progress on whether paging message needs to include security information for authentication.

Proposal 9	A-IoT paging message for CFRA can omit the paging ID presence/absence indicator, transaction ID, and number of access occasions.

Proposal 10	In case multi-device CFRA is supported, RAN2 to revisit if paging message format for CFRA and CBRA can be unified or not.

Proposal 11	Support 7-bit length field in MAC PDU of R2D message with NAS information of variable size.

Proposal 12	MAC PDU for the Random ID Response message (Msg2) includes:

	R2D message type

	One or multiple RN16s subject to message size limitation, for each of RN16 there are associated control information, including

o	RN16 value

o	Presence/absence indicator for AS ID (in case of AS ID assignment)

o	Scheduling information for Msg3

Proposal 13	For CBRA, from a device’s perspective, AS ID is valid until completion of the inventory + command procedure (Option 4b).

Proposal 14	Device considers procedure completed upon reception of an access occasion trigger message.

# 4      References

RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #107, Incheon, South Korea, March, 2025.

TR 22.840, V0.4.0, “Study on Ambient power-enabled Internet of Things (Release 19)”

TR 23.700-13, V1.2.1, “Study on Architecture support of Ambient power-enabled Internet of Things (release 19)”.

[POST129bis][017][AIoT] 38.391 Running CR, Huawei, RAN2#129bis, Wuhan, China, 7th – 11th April, 2025.

R2-25XXXX, “Initial Text Proposal for A-IoT MAC specification”, Huawei, RAN2#130, Malta, Italy, 19th – 23rd May, 2025.

R2-25XXXX, “Discussion on DL messages for Ambient IoT UEs”, Ericsson, RAN2#130, Malta, Italy, 19th – 23rd May, 2025.

R2-25XXXX, “UL multiple access for Ambient IoT”, Ericsson, RAN2#130, Malta, Italy, 19th – 23rd May, 2025.

R1-25XXXX, “Other aspects for Ambient IoT”, Ericsson, RAN2#121, Malta, Italy, 19th – 23rd May