3GPP TSG-RAN WG2 Meeting #130		R2-2503627

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on MAC and RRC open issues for UEI BMR

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

In the email discussion on [Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson), some RRC open issues are listed as below [1]:

| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |
| --- | --- | --- |
| UEI BMR | Editor’s note: The fields ul-BWP-Id, servCellIndex and periodicityAndOffset may have further updates. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |

In the email discussion on [Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung), some MAC open issues are listed in [2].

In this contribution, we will discuss the above open issues related to UE-initiated/event-driven beam management, including RRC and MAC impacts.

- Discussion

  - RRC open issues

For the below open issue:

| UEI BMR | FFS whether UEI BM parameters should be moved to CSI-MeasConfig. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |
| --- | --- | --- |

First, the motivation of the FFS needs further clarification. If it means to introduce a new RRC parameter, like UEI-CSI-ReportConfigToAddModList, to provide the configuration of the UE-initiated/event-driven beam management, we think it is unnecessary. Compared to designing a new report configuration framework for UE-initiated/event-driven beam management, it is much simpler to support UE-initiated/event-driven beam management by extending the legacy CSI report configuration framework. Similar to the event-triggered reporting configuration for LTM, a new report type, i.e., UEI-BM can be introduced in the CSI-ReportConfig. Under the parameter of the new report type, the parameters related to UE-initiated/event-driven beam management, including event-related parameters, reporting uplink resource configuration and report content configuration can be included. In summary, compared to moving UEI BM parameters to CSI-MeasConfig, extending the legacy CSI report configuration framework to support UE-initiated/event-driven beam management is preferred.

RAN2 supports to extend the legacy CSI report configuration framework to support UE-initiated/event-driven beam management, i.e. not moving UEI BM parameters to CSI-MeasConfig.

Similar to the event-triggered report configuration framework for LTM, a new report type, i.e., UEI-BM could be introduced in the CSI-ReportConfig and the UEI-BM corresponding parameters can be replaced under the parameter of the new report type.

- For the below open issue:

| UEI BMR | Editor’s note: The fields ul-BWP-Id, servCellIndex and periodicityAndOffset may have further updates. | To be discussed in contributions, but we should wait once the UEI BMR parameters are more stable. |
| --- | --- | --- |

- According to the below agreement achieved in RAN1#118 meeting [3],

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, for at least Mode-B, the beam report should be carried in the second UL channel in the CC where the corresponding CSI report configuration is configured. - Above applies to both cross-CC and same-CC beam report. - Note: Above is applied to the case that second UL channel is PUSCH. - FFS: Whether the first and second channels can be from the same/different CC.

- the pre-configured CG-PUSCH used to carry the beam report in Mode B should be in the same CC where the corresponding CSI report configuration is configured. Therefore, the parameter of servCellIndex used to indicate the CC of the CG-PUSCH resources should be removed. On the contrary, the parameter of ul-BWP-id should be provided. In the current specification, the parameter of ConfiguredGrantConfigIndex-r16 is used to indicate the index of one of multiple UL Configured Grant configurations in one UL BWP. If the parameter of ul-BWP-id is absent, the UE cannot determine the CG-PUSCH resources based on the parameter of ConfiguredGrantConfigIndex-r16. For the parameters of the first PUCCH channel for UE-initiated/event-driven beam management, besides PUCCH resource id, periodicity and offset, UL BWP id should also be provided. Otherwise, UE cannot determine the PUCCH resources in the frequency domain.

For the parameters of the configured grant in Mode B, ul-BWP-id should be configured, but servCellIndex should be removed.

For the parameters of the first PUCCH channel for UE-initiated/event-driven beam management, besides periodicityAndOffset and resource, ul-BWP-id should also be configured for UE to determine the PUCCH resources.

  - MAC open issue

Regarding the measurement RS determination of new beam(s), RAN1 has agreed that the measurement RS(s) for new beam(s) are explicitly configured in one RS resource set associated with a CSI reporting configuration in RAN1#118 meeting [2].  But whether the RS in the RS resource set can be updated by MAC-CE is still being discussed in RAN1.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

In our view, new beam(s) changes as the current beam changes, so it is necessary to update the measurement RS(s) of new beam(s). In addition to dynamic update by MAC CE, RRC reconfiguration is another approach. However, as RRC reconfiguration is a long-time operation, it is not suitable for UE-initiated/event-driven beam reporting, which is intended to reduce TCI switch delay. To avoid the update of measurement RS(s) of new beam(s), all of the current beam(s) applied for transmission and reception, the activated TCI states other than the indicated one, and inactive TCI states should be included in the measurement resource set. However, it may result in a large measurement overhead and increase the UE computation complexity. In summary, the dynamic update of measurement RS within the new beam resource set by MAC CE is more reasonable. But anyway, the final decision should be up to RAN1 discussion.

RAN2 assumes to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

- Conclusion

In this contribution, we discuss the RRC and MAC open issues on UE-initiated/event-driven beam management, identified in the email discussion. Based on the above discussion, we have the following proposals.

RRC open issues:

- RAN2 supports to extend the legacy CSI report configuration framework to support UE-initiated/event-driven beam management, i.e. not moving UEI BM parameters to CSI-MeasConfig.

Similar to the event-triggered report configuration framework for LTM, a new report type, i.e., UEI-BM could be introduced in the CSI-ReportConfig and the UEI-BM corresponding parameters can be replaced under the parameter of the new report type.

For the parameters of the configured grant in Mode B, ul-BWP-id should be configured, but servCellIndex should be removed.

For the parameters of the first PUCCH channel for UE-initiated/event-driven beam management, besides periodicityAndOffset and resource, ul-BWP-id should also be configured for UE to determine the PUCCH resources.

MAC open issue:

(MAC-4) RAN2 assumes to support dynamic update of the measurement RS in the measurement resource set for new beam(s) by MAC CE, and final decision is up to RAN1.

- References

- R2-2504185, Open issues for MIMO on 38.331,	Ericsson

- R2-2504210, Report of MAC open issues for MIMO, Sumsung

- Draft_Minutes_report_RAN1#118