3GPP TSG-RAN WG2 Meeting #130	                  	                  R2-2503646

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.11.2

Source: 	OPPO

Title:  	Remaining issues of SBFD RACH procedure

Document for:	Discussion and Decision

# Introduction

According to the previous RAN2 discussion, RAN2 made the following agreements regarding the RACH procedure for SBFD operation:

RAN2#127: Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

RAN2#127bis: RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2#128: CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

RAN2#129: When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

RAN2#129bis: On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

In this contribution, we provide some analysis on the remaining issues of the SBFD RACH procedure

# Discussion

## SBFD RO for LTM

RAN2#129bis agreement: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

TS 38.300: ### 9.2.6 Random Access Procedure The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure (see clause 18.0) is ongoing, when UL synchronisation status is "non-synchronised"; - UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; - Handover, except for when RACH-less HO is configured; - SR failure; - Explicit request by RRC upon synchronous reconfiguration; - RRC Connection Resume procedure from RRC_INACTIVE; - To establish time alignment for a primary or a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell; - SDT in RRC_INACTIVE (see clause 18); - Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning; - Early UL synchronization with an LTM candidate cell; - RACH-based LTM cell switch.

TS 38.473:

For the early sync RACH, in order to support the RACH in SBFD symbol, extra standard effort is needed to include the SBFD RACH configuration in EarlyUL-SyncConfig, and the SBFD indication used together with the early sync PDCCH order should be coordinated between the source node and the target node, and the target node needs to provide the SBFD PRACH resource to the source node for the early sync RACH (as quoted above from TS 38.473).

For the RACH-based LTM cell switch, the SBFD indication needs to be added in the LTM Cell Switch Command MAC CE, and the SBFD indication via the LTM Cell Switch Command MAC CE needs to be coordinated between the source node and the target node, and the target node needs to provide the SBFD PRACH resource of CFRA to the source node for the early sync RACH (as quoted above from TS 38.473).

Thus, to avoid the cross WG standard efforts including RAN1/RAN2/RAN3, we think that RACH in SBFD symbol should not be supported for LTM in Rel-19.

Proposal 1: RACH in SBFD symbols is not supported for LTM in Rel-19, including both LTM early sync and LTM cell switch.

## Power ramping offset for the fallback between legacy RO and additional RO

RAN2#128 meeting agreements: The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

Running 38.331 CR of SBFD:

According to the RAN2#128 meeting agreements and the running 38.331 CR of SBFD item as quoted above, for the Option 2 of SBFD RACH configuration, the UE will have two different powerRampingStep values, one for the legacy RO and one for the SBFD RO.

Observation 1: For SBFD RACH configuration Option 2, the power ramping step of additional RO can be different from the power ramping step of the legacy RO.

According to TS38.321: … …

According to the TS 38.321, if two different power ramping steps are used during a RACH procedure (e.g. due to the fallback from 2-step RACH to 4-step RACH), the UE needs to add the power offset to compensate the difference between different power ramping steps, so that the transmission power at the switch between 2 different power ramping steps can be guaranteed.

Observation 2: The power ramping offset (i.e. POWER_OFFSET_2STEP_RA) for the fallback between 2-step RACH and 4-step RACH is introduced, as the power ramping step of 2-step RACH can be different from the power ramping step of 4-step RACH.

Thus, we think the power offset needs to be introduced to ensure the coverage for the fallback between the additional RO and the legacy RO

Proposal 2: Alike the fallback between 2-step RACH and 4-step RACH, power ramping offset is introduced for the fallback between legacy RO and additional RO.

## Msg3 indication for SBFD

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

Firstly, we consider that if the gNB is able to know the UE capability of supporting SBFD earlier, it would be beneficial for the gNB to use the SBFD symbols for the UE immediately after RACH. Some companies consider that as the UE could select the legacy RO, the gNB is not able to know the UE capability of supporting SBFD during RACH. Then indicating the UE capability (e.g. 1 bit) of supporting SBFD in Msg3 can be useful. However, from our understanding, the UE would have several different capability bits on how to support different SBFD features (e.g. the support of different types of SBFD RACH configurations, the support of different SBFD Tx/Rx/measurement procedures and the support of different CLI handlings). Then indicating only 1 bit on the SBFD capability via Msg3 is not able to help the gNB to provide the proper SBFD configurations to the UE. Apart from the RACH procedure, the gNB would anyway need the full UE capability bits of SBFD (e.g. via Msg5 of the UECapabilityInformation message) to configure the SBFD function to the UE. Thus, we think that the 1-bit Msg3 indication for SBFD is not needed.

Observation 3: Using 1-bit Msg3 indication of SBFD cannot help the gNB to provide the correct SBFD configuration, as a few UE capability bits will be introduced for indicating which sub-feature of SBFD function is supported.

Proposal 3: Msg3 indication for SBFD is not supported.

## RA-RNTI collision between legacy RO and additional RO

RAN1#120 meeting agreements: Conclusion For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt-1: - Alt-1: do not optimize for this case

TS 38.321: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id MSGB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 × 2

According to the RAN1#120 meeting agreements, the RACH configuration Option 2 of SBFD supports the overlapping between the additional RO and the legacy RO. As the f_id of both the additional RO and the legacy are overlapped (i.e. both f_id(s) start from “0”), when some UEs are using additional RO and other UEs are using legacy RO in the same symbol, the RA-RNTI for the Msg2 reception could be collided.

However, we think that even though the f_id(s) of both the additional RO and the legacy are overlapped, the RA-RNTI may/may not be collided, depending on the number of RACH procedures triggered at the same time. On the other hand, when the number of legacy ROs and the number of additional ROs are large, the collision probability of RA-RNTI can be further reduced. Even though in some extreme cases when the RA-RNTI is collided, the UE can still re-access to send a new preamble, which will encounter less RA-RNTI collision due to the randomization of BI. Moreover, if the gNB considers that the RA-RNTI collison issue is severe, the gNB can configured the legacy RO for 2-step RACH, in which the MSGB-RNTI of legacy RO is not collided with the RA-RNTI of additional RO.

Observation 4: When additional RO overlaps with legacy RO, the RA-RNTI may/may not be collided, depending on the number of RACH procedures triggered at the same time.

Observation 5: When the number of legacy ROs and the number of additional ROs are large, the probability of RA-RNTI collision is low.

Observation 6: When the RA-RNTI is collided, the UE can still re-access by sending a new preamble, which encounters less RA-RNTI collision due to the randomization of BI.

Observation 7: The configuration of the legacy RO for 2-step RACH and the additional RO for 4-step RACH in the same symbol can be supported. The MSGB-RNTI of legacy RO is not collided with the RA-RNTI of additional RO.

Proposal 4: Whether/how to handle the RA-RNTI collision for the case that additional RO overlaps with legacy RO is up to the network implementation.

# Conclusion

According to the analysis provided above, we have the following observations and proposals:

Observation 1: For SBFD RACH configuration Option 2, the power ramping step of additional RO can be different from the power ramping step of the legacy RO.

Observation 2: The power ramping offset (i.e. POWER_OFFSET_2STEP_RA) for the fallback between 2-step RACH and 4-step RACH is introduced, as the power ramping step of 2-step RACH can be different from the power ramping step of 4-step RACH.

Observation 3: Using 1-bit Msg3 indication of SBFD cannot help the gNB to provide the correct SBFD configuration, as a few UE capability bits will be introduced for indicating which sub-feature of SBFD function is supported.

Observation 4: When additional RO overlaps with legacy RO, the RA-RNTI may/may not be collided, depending on the number of RACH procedures triggered at the same time.

Observation 5: When the number of legacy ROs and the number of additional ROs are large, the probability of RA-RNTI collision is low.

Observation 6: When the RA-RNTI is collided, the UE can still re-access by sending a new preamble, which encounters less RA-RNTI collision due to the randomization of BI.

Observation 7: The configuration of the legacy RO for 2-step RACH and the additional RO for 4-step RACH in the same symbol can be supported. The MSGB-RNTI of legacy RO is not collided with the RA-RNTI of additional RO.

Proposal 1: RACH in SBFD symbols is not supported for LTM in Rel-19, including both LTM early sync and LTM cell switch.

Proposal 2: Alike the fallback between 2-step RACH and 4-step RACH, power ramping offset is introduced for the fallback between legacy RO and additional RO.

Proposal 3: Msg3 indication for SBFD is not supported.

Proposal 4: Whether/how to handle the RA-RNTI collision for the case that additional RO overlaps with legacy RO is up to the network implementation.

# Reference

[1] 3GPP TS38.331, “NR; Radio Resource Control (RRC) protocol specification”