3GPP TSG-RAN WG2 Meeting #130                    	                  R2-2503647

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.11.3

Source: 	OPPO

Title:  	Discussion on the SBFD related issues

Document for:	Discussion and Decision

# Introduction

According to the discussion in RAN1 and RAN4, the UE requirements on the L3 measurement and the BFR procedure for SBFD symbol were discussed. According to the latest agreements made in the Rel-19 NES, the NES function will introduce the Rel-19 NES RACH adaptation configuration. In this contribution, we provide our understandings on the RAN2 related aspects of the SBFD operation for the L3 measurement, the BFR procedure and the NES.

# Discussion

## Measurement gap

RAN1#118bis: Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

According to the RAN1 agreement as quoted above, SSB can be configured in the SBFD symbol, with the restriction that the SSB is within DL suband. The case that SSB outside of the DL suband of SBFD symbol has not been agreed yet. Form our understanding, the UE may/may not require a measurement gap when the SSB is outside of the DL subband of the SBFD symbol, depending on the UE capability and the measurement requirements defined in RAN4. RAN2 can wait for more inputs from RAN4 on whether a separate gap is needed for the SBFD symbol.

Observation 1: The SSB in SBFD symbol is within DL subband.

Proposal 1: Wait for RAN1/RAN4 inputs on whether a separate measurement gap for SBFD symbol is needed.

## RLM/BFR

RAN4#114bis meeting agreements: - For RLM/BFD/CBD, further discuss the following options - RAN4 to define requirements based on the assumption that occasions of a single CSI-RS resource are all in the same symbol type - Existing measurement period apply - FFS whether to add Note: For the potential power difference between SBFD symbol and non-SBFD symbol, no RAN4 enhancement on power difference handling in Rel-19

According to the RAN4#114bis meeting agreements as quoted above, the RLM/BFD requirements can be based on the single CSI-RS resource in the same symbol type. This means that the RLM/BFD requirements of the SBFD symbol can be different from that of the non-SBFD symbol.

TS 38.331:

According to the TS 38.331 as quoted above, one RLM or BFR procedure can use multiple RS resources, which mean that some RS resources can be in the SBFD symbol and some RS resources can be in the non-SBFD symbol.

Observation 2: The legacy RLM or BFR procedure can make use of multiple RS resources, e.g. some RS resources in SBFD symbols and others in non-SBFD symbols.

TS 38.321: The following UE variables are used for the beam failure detection procedure: - BFI_COUNTER (per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets): counter for beam failure instance indication which is initially set to 0. … 2> if beam failure instance indication has been received from lower layers: 3> start or restart the beamFailureDetectionTimer; 3> increment BFI_COUNTER by 1;

According to the BFR procedure defined in TS38.321, the UE uses the beam failure instance indication to calculate the BFI_COUNTER, and the beam failure instance indication is provided by PHY based on the PHY measurement on the BFD reference signals. The BFI_COUNTER is per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets, i.e. does not differentiate between different beams or different reference signal configuration in different symbols. On the other hand, as the signal transmission from/to a specific beam should also be allowed in both the SBFD symbol and the non-SBFD symbol, the failure of one beam should be the combination of the channel conditions of both the SBFD symbol and the non-SBFD symbol. Thus, there is no need to differentiate the BFD/BFD procedure for the SBFD symbol and the non-SBFD symbol. As the BFD procedure, the same principle of using both SBFD symbol and non-SBFD symbol (i.e. no need to differentiate SBFD symbol and non-SBFD symbol) can be applied to the per-cell RLM procedure.

Observation 3: The current BFD procedure in MAC is per serving cell or per BFD-RS set of serving cell configured with two BFD-RS sets, i.e. BFI_COUNTER does not reset when the serving beam changes.

Proposal 2: The RLM/BFR procedure does not differentiate between the SBFD symbol and the non-SBFD symbol.

## SBFD configuration for NES

RAN2#129bis meeting agreements: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

According to the discussion in the RAN2#129bis meeting, RAN2 agreed not to support RACH in SBFD symbol for SI request.

Running 38.300 CR of NES:

According to the Rel-19 NES discussion/running CR as quoted above, the OD-SIB1 (on-demand SIB1) is introduced by configuring the PRACH resources for the request of the SIB1. According to the TS 38.331, as SIB1 is not part of the SI (SystemInformation) message, the RAN2#129bis agreement of not support SBFD symbol PRACH for SI request cannot be applied to the OD-SIB1.

From our understanding, RAN2 can adopt the same principle (i.e. using legacy RO is sufficient for SI request) as discussed for the SI request, and the OD-SIB1 can use the legacy RO for the OD-SIB1, so that extra the standard effort of using SBFD symbol for OD-SIB1 can be saved.

Proposal 3: The additional RO of SBFD symbol is not supported for the OD-SIB1 request.

Running 38.300 CR of NES:

According to the Rel-19 NES discussion and the running CR as quoted above, adaptation of PRACH configurations in time domain is supported for 4-step RACH CBRA. From our understanding, in order to support the additional RO of the SBFD symbol, extra standard efforts (e.g. SBFD RO configuration in the additional PRACH configuration) would be needed. However, as the details on how the adaptation of PRACH configurations is configured are still open in the Rel-19 NES item, we think that we can wait for more inputs of the PRACH configuration adaptation from the Rel-19 NES discussion, before deciding whether/how to support SBFD RO for the PRACH configuration adaptation.

Proposal 4: Wait for more inputs from the Rel-19 NES discussion before deciding whether/how to support SBFD RO for the PRACH configuration adaptation.

# Conclusion

According to the analysis provided above, we have the following observations and proposals:

Observation 1: The SSB in SBFD symbol is within DL subband.

Observation 2: The legacy RLM or BFR procedure can make use of multiple RS resources, e.g. some RS resources in SBFD symbols and others in non-SBFD symbols.

Observation 3: The current BFD procedure in MAC is per serving cell or per BFD-RS set of serving cell configured with two BFD-RS sets, i.e. BFI_COUNTER does not reset when the serving beam changes.

Proposal 1: Wait for RAN1/RAN4 inputs on whether a separate measurement gap for SBFD symbol is needed.

Proposal 2: The RLM/BFR procedure does not differentiate between the SBFD symbol and the non-SBFD symbol.

Proposal 3: The additional RO of SBFD symbol is not supported for the OD-SIB1 request.

Proposal 4: Wait for more inputs from the Rel-19 NES discussion before deciding whether/how to support SBFD RO for the PRACH configuration adaptation.

# Reference

[1] R2-2500554 Other aspects of SBFD Nokia discussion Rel-19 NR_duplex_evo-Core

[2] R2-2500607 Discussion on L3 and L1 measurement in SBFD ZTE Corporation discussion Rel-19 NR_duplex_evo-Core

[3] R2-2501170 Other Aspects of SBFD Samsung discussion Rel-19 NR_duplex_evo-Core

[4] R2-2500885 CSI-RS measurements for RLM/BFD/CBD in SBFD Ericsson discussion Rel-19 NR_duplex_evo-Core

[5] 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”.