3GPP TSG-RAN WG2 Meeting #130                           R2-2503664

St.Julians, Malta, May 19th – 23rd , 2025

Source:	China Telecom

Title:	Further discussions on A-IoT random access

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

In the last RAN2#129-bis meeting, RAN2 discussed random access related issues and achieved the following agreements [1].

Agreements on msg1 1.In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes 2.RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. Agreements on new R2D message 1.A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. 2.Assumption: The R2D message does not include slot number/count down number. Agreements on msg 2 1.A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. 2.Same Msg2 format is used for initial transmission and retransmission of Msg2. Agreements on NACK 1For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion). 2For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging 3For CFRA, NACK feedback and re-access is not supported. FFS how to achieve 4FFS on end of procedure

In this contribution, the details of A-IoT random access procedure are further discussed.

- Discussion

For CBRA, the transmission resource information of Msg1 is indicated by a paging message. If an A-IoT device needs to respond with some data (such as in response to a read command), it is meaningful for the device to report some assistance information, including the size of the data volume, to enable the reader to accurately allocate transmission resources. Regarding the transmission resource of Msg3, the reader can indicate the time-frequency resource information through Msg2.

Proposal 1: The device may include assistance information (e.g., data size) to indicate the resource for Msg3 via Msg1.

Proposal 2: The reader allocates the time-frequency resource for Msg3 via Msg2.

Based on the existing agreements, a new R2D message other than the paging message is introduced to indicate the start of a set of Msg1 resources that were configured in the paging message. The terminology of this R2D message can be “Access Occasion Trigger message”. Since the resources for Msg1 have already been configured in the paging message, this message only needs to include a message type field.​

Proposal 3: Access Occasion Trigger message only needs to include a message type field.

For the Msg2 of CBRA, whether to assign a new AS ID depends on the reader implementation. Thus, this AS ID field is optional. To indicate if the AS ID is assigned by the reader, we can set one bit field to indicate the presence of it, e.g., 1 means AS ID is present and 0 means AS ID is not present. When Msg2 carries multiple AS IDs for the different devices, the order of the above one-bit field and AS ID field are the same as the order of resources occupied by Msg1.​

Proposal 4: A one-bit field can be used to indicate the presence of the AS ID assigned by the reader through Msg2.

Moreover, when the one-bit indicator indicates that the AS ID is absent, the reader can schedule the device via the random ID in the subsequent R2D message. This approach adds no extra complexity to the device side. Therefore, from our perspective, the above one-bit indicator can also be used to indicate whether to reuse the random ID as the AS ID.​

Proposal 5: If Proposal 4 is supported, one bit indication can also indicate whether to reuse random ID as AS ID or not. (e.g., when it indicates the AS ID is absent, it also indicates the reuse of the random ID as the AS ID).

Regarding the NACK mechanism, we have agreed that, as a baseline for CBRA, the NACK-based mechanism is applied only to Msg3. During the D2R data transmission stage (e.g., for command response messages), transmission failures may occur. The NACK mechanism is also useful in this case to achieve a unified design. In some situations, multiple devices may fail and need to be indicated by NACK. To reduce the overhead of the message type in the A-IoT MAC header, it is beneficial to have a NACK indication that can include the ID information of multiple failed devices.​

Proposal 6: The NACK mechanism can also be applied to command response messages for subsequent re-access.

Proposal 7: A NACK indication can include the ID information of multiple failed devices.

In the last meeting, we agreed that NACK feedback and re-access are not supported for CFRA. When the reader does not receive Msg1 at the allocated resource location, it means that the Msg1 transmission has failed. In this case, the reader can trigger a new CFRA procedure for this device with new resource allocation. The reader will not re-trigger a new CFRA until it receives Msg1 at the dedicated resource. This is a very simple and efficient design.​

Proposal 8: For CFRA, the reader may trigger a new CFRA procedure when the previous Msg1 transmission fails.

As for the re-access procedure, the subsequent paging message is used to trigger re-access. Different from the initial paging message with the same service request, the subsequent paging message may include a one-bit indication in the A-IoT MAC header. This bit indicates that this is the message used to trigger re-access. In this way, those devices that have accessed successfully will not process this subsequent paging message. This reduces redundant signaling transmission and improves communication efficiency.

Proposal 9: A one-bit re-access indication may be included in the subsequent paging message.

Moreover, in addition to the subsequent paging message used for re-access, an explicit R2D message can also be defined to trigger the re-access procedure. To make use of the unoccupied opportunities in the initial access round, this explicit R2D message can trigger the failed devices to re-access, which is an efficient way to fully utilize the remaining resources.

Proposal 10: A new R2D message should be defined to trigger re-access during the initial access round.

RAN2 has agreed to use a 16-bit random ID in CBRA and determine whether the access is successful or not based on whether the echoed random ID returned in Msg2 is consistent with the one sent in Msg1. Additionally, the decision to use a 16-bit random number can minimize conflicts as much as possible. However, when the reader successfully receives and decodes the same random ID occupying different Msg1 resources from different devices, the reader returns two identical random IDs and assigns different AS IDs. In such a case, devices cannot identify which AS ID corresponds to themselves. Therefore, it is crucial to define the mapping relationship between the random ID and the selection of Msg1 resource.​

Proposal 11: RAN2 to define the mapping relationship between the random ID and the Msg1 resource selection.

For the Msg2 format of the 3-step CBRA, the draft has captured that the R2D message Type field, Echoed Random ID field, Assigned AS ID field, and D2R Scheduling Info field are included in Msg2, but the details are FFS. From our perspective, we propose to group {Echoed Random ID, Assigned AS ID, and D2R scheduling info} for the same device, as these elements are interrelated. When a device decodes the relevant information pertaining to itself, it can proceed to the next operation without having to decode all the information (except for the last device), thereby achieving an energy-saving effect. Additionally, this design obviates the need to indicate the number of random IDs, which can reduce signaling overhead.​

Proposal 12: Regarding the Msg2 format, it is proposed to group {Echoed Random ID, Assigned AS ID, D2R scheduling info} for the same device.

- Conclusion

Based on the above analysis, the following proposals are given for A-IoT random access procedure.

Proposal 1: The device may include assistance information (e.g., data size) to indicate the resource for Msg3 via Msg1.

Proposal 2: The reader allocates the time-frequency resource for Msg3 via Msg2.

Proposal 3: Access Occasion Trigger message only needs to include a message type field.

Proposal 4: A one-bit field can be used to indicate the presence of the AS ID assigned by the reader through Msg2.

Proposal 5: If Proposal 4 is supported, one bit indication can also indicate whether to reuse random ID as AS ID or not. (e.g., when it indicates the AS ID is absent, it also indicates the reuse of the random ID as the AS ID).

Proposal 6: The NACK mechanism can also be applied to command response messages for subsequent re-access.

Proposal 7: A NACK indication can include the ID information of multiple failed devices.

Proposal 8: For CFRA, the reader may trigger a new CFRA procedure when the previous Msg1 transmission fails.

Proposal 9: A one-bit re-access indication may be included in the subsequent paging message.

Proposal 10: A new R2D message should be defined to trigger re-access during the initial access round.

Proposal 11: RAN2 to define the mapping relationship between the random ID and the Msg1 resource selection.

Proposal 12: Regarding the Msg2 format, it is proposed to group {Echoed Random ID, Assigned AS ID, D2R scheduling info} for the same device.

# References

[1] 3GPP RAN2#129bis Chair notes.