3GPP TSG-RAN WG2 Meeting #130	R2-2503680

St Julian, Malta, 19th – 23rd May 2025

Agenda item:	8.6.1

Source: 	China Telecom

Title:	List of 37.340 open issues for R19 mobility

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

This paper provides the open issue list for 37.340 based on the following post meeting discussion on 37.340 running CR.

[POST129b][120][MOB] (China Telecom)

Scope: Update 37.340 running CR based on RAN2#129bis progress.

Intended outcome: Updated 37.340 running CR.

Deadline: Long email discussion

- Discussion

Open issues related to RAN2

Open issue 37.340-1: whether to have the restriction on only supporting MCG LTM with SN addition within the same SN.

Issue description:

As per the comments received during the offline discussion, it seems that there are different understandings on the following agreement made in RAN2#128.

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

Understanding 1: Only MCG LTM with the same SN addition is supported, which means the SN is released before execution of LTM and then added during the MCG LTM.

Understanding 2: MCG LTM with SN addition is supported with no restriction on the same SN before LTM.

In the latest 37.340 running CR, it is implemented with Understanding 2 and an Editor’s Note is added for further discussion.

Inter-MN or Intra-MN MCG LTM with SCG addition, SCG release or configuration change without SN change is supported. Inter-MN or intra-MN MCG LTM with SN changed to another SN is not supported. Editor’s Note: FFS on whether to have the restriction on only supporting MCG LTM with SN addition within the same SN.

Proposed resolution:

According to the agreement made in RAN2#128, inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. In the current running CR, there is no restriction on whether to only support MCG LTM with SN addition within the same SN. If we go for Understanding 1, it will bring extra spec impact and more complexity to gNB implementation to guarantee the same SN is involved. Thus, from rapporteur’s point of view, we suggest keep the current version and delete the Editor’s note. However, since only a few comments are received during the offline discussion, it is better to confirm a common understanding in next meeting based on companies’ contribution.

Proposal 1: Regarding MCG LTM with SN addition, RAN2 to discuss and confirm whether there is no restriction on only supporting MCG LTM with SN addition within the same SN.

Open issues related to RAN3

Open issue 37.340-2: FFS up to RAN3 discussion

Issue description:

In the latest version of running CR, there are a few open issues up to RAN3 discussion. Some of them have already been fixed by RAN3. After the coordination with RAN3 rapporteur of 37.340 BLCR (CATT), they suggest RAN2 keep the FFS for now, since RAN3 is also checking the FFS listed by RAN2.

Proposed resolution:

Regarding the remaining FFS related to RAN3 in the running CR, RAN2 keeps the FFS for now and waits for RAN3’s progress.

Proposal 2: Regarding the remaining FFS related to RAN3 in the running CR, RAN2 keeps the FFS for now and waits for RAN3’s progress.

- Conclusion

In this contribution, we summarize the open issues in the 37.340 running CR. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding MCG LTM with SN addition, RAN2 to discuss and confirm whether there is no restriction on only supporting MCG LTM with SN addition within the same SN.

Proposal 2: Regarding the remaining FFS related to RAN3 in the running CR, RAN2 keeps the FFS for now and waits for RAN3’s progress.

- Reference

R2-2503679	Introduction of NR mobility enhancements Phase 4 in TS 37.340