3GPP TSG-RAN WG2 #130	R2-2503687

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	   8.3.3

Source:		Lenovo

Title:                            Configuration and Report of Inference Input to NW-sided model

Document for:		Discussion and Decision

# Introduction

As instructed by the SID, the AI for mobility SI will study the RRM measurement prediction and measurement event prediction. In this paper, we discuss essential LCM aspects of network-sided model, for AI/ML mobility.

# Discussion

In R19 AI/ML for NR air interface work item, the fundamental framework of AI/ML functionality has been discussed considering the following modules for the beam management and positioning use cases:

- Life Cycle Management (LCM)

  - LCM for NW-sided model

  - LCM for UE-sided model

- Data collection

  - Data collection for NW-sided model

  - Data collection for UE-sided model

The general aspects of LCM have focused on the procedures to enable AI/ML functionality and eventually provide an inference outcome for an intended use case or a sub-use case.  The procedures that were addressed include network configuration for AI/ML-related functionalities, determination of applicability of functionality, signaling mechanisms for applicability report, inference reporting from the UE, etc. The agreed AI/ML framework should be extendable to other use cases, such as AI/ML for mobility: RRM measurement prediction and RRM event prediction.

The AI for mobility SID suggests studying both UE-sided and NW-sided models for RRM measurement prediction. Whether RRM measurement prediction is conducted by the UE or the network, it is, in either case beneficial in determining the mobility-related decision and configuration by the network. For example, it can be used by the network for configuring the beams, in making handover decisions, or configuring thresholds for mobility-related measurements.

Observation: Both UE-sided and NW-sided RRM measurement predictions can be equally beneficial to improve mobility performance.

RRM measurement predictions performed by the NW-sided model can be implemented with a least specification impact other than data collection procedures. As agreed in AI/ML for air interface WI, UE does not need to be informed of and involved in any gNB-sided functionality management decision (e.g., selection, de(activation), switching, fallback, etc), except being configured to provide the required measurement/data. Thus, the network can configure the UE for RRM measurements and reporting required for inputs (e.g., L1-RSPR, L3-RSRP) to the NW-sided models. As per the progress of LCM for the NW-sided model for beam management (BM) in AI/ML for air interface, RAN2 can assume that the following agreements can be directly extended to the RRM measurement prediction use case.

Observation: The following agreements can be reviewed and extended to RRM measurement prediction:

LCM and data collection for NW-sided model

- RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

For the BM use case, as the input and predictions are mainly dependent on the L1 measurements, the following agreement indicates the use of L1 signaling. However, for the case of RRM measurement prediction, since cell-level measurement is the final inference outcome, and the input for inference depends on which case of RRM measurement prediction is selected, along with L1 signaling, L3 signaling is also required for the input.

For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

Proposal 1: For the network-sided model for AI/ML Mobility, RAN2 confirms that L1/L3 signaling can be used for UE inputs for inference.

The input from UE for inference can include any legacy L1/L3 beam measurement results or legacy L3 cell measurement results. At the same time, we believe the L1 beam prediction result by a UE supporting the AI for air interface beam prediction use case could also be considered as possible input for further L3 beam/cell level result prediction at NW.

Proposal 2: The input from UE for inference at the network side could be either legacy L1/L3 beam measurement, L3 cell measurement, or L1 beam prediction results (if UE is capable of L1 beam prediction in the AI for air interface).

As described in the SID, the AIML model used for RRM measurement prediction can also be deployed at NW. compared to using a UE-sided model, using a NW-sided model for RRM measurement prediction may have the following advantages:

- NW has more powerful computing capability

- NW can easily collect enough training data from UEs under its coverage via legacy L1/L3 measurement reports. The complexity of data collection to train a UE-sided model can be avoided.

- NW can start the NW-sided AIML inference without additional prediction-related configuration towards UE.

- For the sake of model performance monitoring, NW node can easily configure the UE to do necessary L1/L3 measurements and report, or even request the neighbour NW node to provide the necessary UE measurements after a handover execution.

From RAN2 point of view, how NW requests input to support RRM measurement prediction using NW-sided model would be the focus. As a starting point assumption, NW can request input from UE in one-shot (aperiodic), periodic, or event-triggered manner.

Proposal 3: NW can configure UE to report in a periodic, one-shot (aperiodic), or event-triggered manner for RRM measurement prediction for NW-sided model. FFS details.

# 3	Conclusion

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

Observation: The following agreements can be reviewed and extended to RRM measurement prediction:

LCM for NW-sided model

- RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

- RAN2 to consider an RRC configuration to configure radio measurements and the related reporting to enable data collection for NW-side training

- For AI/ML based beam management, RAN2 assumes the L1 measurement framework shall be used for configuring the input data of the NW side AI/ML model inference.  FFS if further enhancements are needed

Proposal 1: For the network-sided model for AI/ML Mobility, RAN2 confirms that L1/L3 signaling can be used for UE inputs for inference.

Proposal 2: The input from UE for inference at the network side could be either legacy L1/L3 beam measurement, L3 cell measurement, or L1 beam prediction results (if UE is capable of L1 beam prediction in the AI for air interface).

Proposal 3: NW can configure UE to report in a periodic, one-shot (aperiodic), or event-triggered manner for RRM measurement prediction for NW-sided model. FFS details.

# Reference

[1] RP-234055, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#102.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)