3GPP TSG-RAN WG2 #130	R2-2503688

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	   8.3.5

Source:		Lenovo

Title:	 Support of model monitoring in AI/ML for mobility

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the monitoring for both NW NW-sided model and UE UE-sided model in AI for mobility.

# Discussion

## 2.1	Performance Monitoring for NW-sided model

For RRM measurement prediction with NW-sided model, the monitoring of the performance can be simply based on UE’s measurement report according to the gNB’s configuration.

Actually, similar as discussed in AI for PHY, the LCM (including monitoring) for NW-sided model can be totally transparent to UE, meaning no additional UE impacts.

For RRM measurement prediction with NW-sided model, the performance monitoring is performed at gNB based on UE’s measurement report according to gNB’s configuration.

For NW-sided model performance monitoring, there is no additional impacts on the UE.

## 2.2	Performance Monitoring for UE-sided model

For performance monitoring with the UE-sided model, the involvement of NW needs to be discussed.

For RRM measurement prediction or indirect RRM event prediction (based on RRM measurement prediction), the monitoring will be essentially about comparing the predicted measurement result and the actual measurement result, which can be similar as discussed in the AI for beam management use case as captured in TR 38.843:

- Type 1: NW centric performance monitoring: configuration is sent from gNB to UE for measurement and reporting. In addition, there are two variations:

  - Option 1 (NW-side performance monitoring): UE sends measurement reporting to NW (e.g., for the calculation of performance metric at NW)

  - Option 2 (UE-assisted performance monitoring): UE calculates performance metric(s), either reports it to NW or reports an event to NW based on the performance metric(s).

- Type 2: UE centric performance monitoring: UE monitors the performance with the help from gNB’s configuration, e.g., gNB configures UE to perform measurements on interested beams cells.

For RRM measurement prediction and indirect RRM event prediction with UE-sided model, RAN2 considers the following options:

  - gNB centric performance monitoring: 1) gNB monitors based on measurement report from UE 2) gNB configures UE to calculate and report the performance metric.

  - UE centric performance monitoring: based on gNB’s configuration, UE measures interested beams/cells without reporting the performance metric to gNB.

For direct RRM event prediction on the other hand, we are not sure if gNB centric performance monitoring is practical, considering:

- The RRM event will not occur or be predicted to occur very frequently when UE is under a cell coverage. There may not be enough samples for gNB to calculate the monitoring metric.

- The UE may be handed over to another cell after reporting a predicted RRM event. There may not be enough time for gNB to determine and make use of the monitoring metric before handover.

For direct RRM event prediction, gNB centric performance monitoring may not be practical considering:

  - The RRM event will not occur or be predicted to occur very frequently when UE is under a cell coverage. There may not be enough samples for gNB to calculate the monitoring metric.

  - The UE may be handed over to another cell after reporting a predicted RRM event. There may not be enough time for gNB to determine and make use of the monitoring metric before handover.

Comparatively, since the same UE sided model for direct RRM event prediction will be maintained at UE, and may be used crossing cells within an area, it will be certainly helpful for UE to keep monitoring based on statistics when UE stays in different cells.

For direct RRM event prediction with UE-sided model, UE centric performance monitoring is supported.

For direct RRM event prediction with UE-sided model, RAN2 considers gNB centric performance monitoring is not practical.

One challenge for both direct and indirect RRM event prediction is that it could happen that a handover is triggered before a predicted event (e.g., A3/A5) really occurs. In this case, the UE cannot effectively monitor if a previous made RRM event prediction is accurate or not, since UE will probably not continue measuring the old cell after the handover. RAN2 is suggested to study if/how UE can continue monitoring RRM event prediction upon handover.

To monitor RRM event prediction with UE-sided model, RAN2 studies the scenario wherein a handover is triggered before the time point a RRM event is predicted to occur.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	For direct RRM event prediction, gNB centric performance monitoring may not be practical considering:

a.	The RRM event will not occur or be predicted to occur very frequently when UE is under a cell coverage. There may not be enough samples for gNB to calculate the monitoring metric.

b.	The UE may be handed over to another cell after reporting a predicted RRM event. There may not be enough time for gNB to determine and make use of the monitoring metric before handover.

Based on the discussion above, we propose:

Proposal 1	For RRM measurement prediction with NW-sided model, the performance monitoring is performed at gNB based on UE’s measurement report according to gNB’s configuration.

Proposal 2	For NW-sided model performance monitoring, there is no additional impacts on the UE.

Proposal 3	For RRM measurement prediction and indirect RRM event prediction with UE-sided model, RAN2 considers the following options:

a.	gNB centric performance monitoring: 1) gNB monitors based on measurement report from UE 2) gNB configures UE to calculate and report the performance metric.

b.	UE centric performance monitoring: based on gNB’s configuration , UE measures interested beams/cells without reporting the performance metric to gNB.

Proposal 4	For direct RRM event prediction with UE-sided model, UE centric performance monitoring is supported.

Proposal 5	For direct RRM event prediction with UE-sided model, RAN2 considers gNB centric performance monitoring is not practical.

Proposal 6	To monitor RRM event prediction with UE-sided model, RAN2 studies the scenario wherein a handover is triggered before the time point a RRM event is predicted to occur.