3GPP TSG-RAN WG2 Meeting #130	R2-2503702

St. Julians, Malta, 19 – 23 May 2025

Agenda item:	8.7.6

Source:	Apple

Title:	Views on Remaining Issues of XR Uplink Rate Control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

In RAN2 #129bis, we have made the following agreements:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

This paper aims to present some of our views relating to the open issues of XR uplink rate control, including:

- Rate control MAC CE format

- Operations of Bit Rate Query Prohibit Timer.

# Discussions

## UL Rate Control MAC CE Formats

Regarding the QFI indication in the MAC CE, we have observed that N:1 mapping between QoS Flows and DRBs is a much more common configuration in the field, which means quite often multiple QoS flows are mapped to the same DRB. On the other hand, we think it is more scalable and efficient if the recommended bit rates for multiple QoS flows can be provided via one MAC CE jointly. With this in mind, we think the MAC CE can have the following elements:

- One Logical Channel ID

- One bitmap indication of concerned QoS flow IDs that are mapped to the indicated Logical Channel

- A list of recommended rates corresponding to the concerned QoS flow IDs

In particular, the bitmap indicate which QoS flows mapped to the the logical channel are targeted by this MAC CE, and the list of recommended rates respectively correspond to the concerned QoS flow IDs. An example MAC CE structure is illustrated below:

Figure 1 An exemplary MAC CE structure for XR rate control

In the example MAC CE structure shown in Figure 1, the QoS flow bitmap takes one byte, which means it is applicable to cases where up to 8 QoS flows are mapped to this LCH. The value of Qi (0 or 1) indicates whether the content of this MAC CE is applicable to the i-th QoS flow mapped to this LCH.

Proposal 1: The rate control MAC CE for Rel-19 XR can include the following fields:

- A LCID corresponding to the DRB where this MAC CE is applicable

- A bitmap indicating the concerned QoS flows mapped to the applicable DRB

- A list of bit rates respectively applicable to the concerned QoS flows.

## Operations of Bit Rate Query Prohibit Timer

We have agreed that the UE may send rate query MAC CE, which allows the UE to ask for suitable bit rates for different QoS flows. An open issue is related to the bit rate query prohibit timer. In particular, whether the configuration granularity of prohibit timer should be per QoS flow or per LCH.

According to the offline discussion on the running CR of TS 38.331 [POST129][510][XR], it seems a majority of companies believe the prohibit timer configuration granularity should be aligned the reporting granularity of the MAC CE design. Since we have now agreed to support per QoS flow indication in the MAC CE, we think RAN2 can also confirm that rate query prohibit timer can be configured at a per QoS flow basis.

Proposal 2: RAN2 can confirm that rate query prohibit timer can be configured per QoS flow.

Furthermore, the prohibit timer procedure should be very similar to legacy. In particular, when the query MAC CE for a specific QoS flow is transmitted, the UE should start the prohibit timer associated to that QoS flow. While the prohibit timer is running, the UE is not allowed transmit another rate query MAC CE that includes the query for the same QoS flow, until the prohibit timer is expired.

Proposal 3: RAN2 can confirm the following UE behaviours related to rate query prohibit timer:

- The UE starts the prohibit timer for a QoS flow when a rate query MAC CE includes the query of this QoS flow.

- The UE does not transmit another rate query MAC CE that includes the query of this QoS flow until the associated prohibit timer expires.

# Conclusions

This paper provides some of our views about UL rate control for Rel-19 XR. We have the following proposals:

Proposal 1: The rate control MAC CE for Rel-19 XR can include the following fields:

- A LCID corresponding to the DRB where this MAC CE is applicable

- A bitmap indicating the concerned QoS flows mapped to the applicable DRB

- A list of bit rates respectively applicable to the concerned QoS flows.

Proposal 2: RAN2 can confirm that rate query prohibit timer can be configured per QoS flow.

Proposal 3: RAN2 can confirm the following UE behaviours related to rate query prohibit timer:

- The UE starts the prohibit timer for a QoS flow when a rate query MAC CE includes the query of this QoS flow.

- The UE does not transmit another rate query MAC CE that includes the query of this QoS flow until the associated prohibit timer expires.

# References

- 3GPP RAN2 #129bis Chair’s Notes, Wuhan, China, April 2025.