3GPP TSG-RAN WG2 Meeting #130	R2-2503720

St. Julians, Malta, 19-23 May 2025

Agenda item:	8.2.2

Source:	Apple

Title:	Discussion on Ambient IoT Paging

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#129 meeting [1], the following agreements about paging has been reached:

- Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID. - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer - the A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In RAN2#129bis meeting [2], the following agreements about paging and R2D trigger message has been reached:

- FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to tak this into account for their design. - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number.

In this paper, we discuss some remaining issues for the normative work for Ambient IoT paging:

# 2 Discussion

### 2.1 Contents of R2D trigger message

RAN2 129bis [2] has agreed to introduce a new R2D trigger message to indicate the start of the set of Msg1 resource. Those Msg1 resource may be TDM/FDMed. So far, RAN2 has not determine any contents of this message, but only assume the R2D message does not include slot number/count down number.  However, we think besides the message type indicating this is a R2D trigger, the triggering message need contain something to associate it with the respective paging transaction. Otherwise, a R2D trigger for a different paging transaction (either from the same reader or different reader) may misleading the device. Therefore, we think a “transaction ID” needs to be included in the trigger.

Proposal 1 	R2D trigger message includes the same “transaction ID” as the one in Paging message.

Note that based on the low-cost nature of type.1 A-IoT device and channel conditions in PRDCH, it is very likely that the device may fail to decode every R2D trigger message. Then, we think it is problematic for a device to first determine a slot # for slotted Aloha access, but then lost track of the slot number due to Rx failure. In this case, the device may mis-count the slot numbers, and lost the access opportunities. Although this is an error case, but we think RAN2 should check if there is anything can be done to help the device in this misfortune. One possible enhancement is to add the slot number in R2D trigger message, so that the device, even after missing a few R2D triggers, can still be correctly triggered based on the slot number information.

Proposal 2 	RAN2 discuss whether the slot# needs to be included in the R2D trigger message.

### 2.2 Handling multiple or new paging requests

The A-IOT device is supposed to be very low-cost simple device. So, the logic regarding the handling of multiple paging requests should be also simple and straight-forward.  It has been agreed that “a A-IoT device is expected to preform one procedure at one time”, but somehow the relationship between the “procedure” and “transaction ID” is still unclear. We think it would be better clarified that the device will only has one ongoing “transaction ID” at a time. This will ensure that the device has no need to store multiple transaction IDs in its memory.

It is very challenging or infeasible for device to understand or track the NW implementation or reader-coordination circumstances from area to area. So, from the device design perspective, it has to assume parallel paging requests may occur.  Second, the device and reader may be out of sync about the what is the “current” transaction due to some intermittent in-and-out coverage issues, so we cannot assume they are always in sync about the current ongoing transaction. It is also equally hard for device to track the readers and prioritize one request over another based on complex conditions. So, we think regardless of whether a new paging transaction is provided by the same reader or different reader, the device should always just follow the “new” paging transaction and abort the “old” one, as long as its device ID matches the Paging ID in the new Paging message. Note that this may create some challenges in NW coordination. Nonetheless, we think the primary goal of A-IoT system design is to reduce device complexity, so it is worth adding NW complexity to ensure simple and robust A-IoT device behavior

Proposal 3 	A device shall abort the current transaction and respond to the matching Paging request with new “Transaction ID”, regardless of whether this new paging transaction is initiated from the same reader or different reader.

### 2.3 Paging ID Visibility in AS layer

One consequence of the introducing AS ID in A-IoT MAC layer design is that the AS ID has to be reliably associated with an upper layer ID (e.g., permanent device ID or temp Device ID), so that the reader can encapsulate the follow-up upper layer A-IoT NAS message(s) to the A-IoT MAC PDU(s) with the matching AS ID in A-IoT MAC header. It is worth noting that any A-IoT NAS messages will not be address to an AS-layer ID such as “AS ID or RN16”, so it is necessary to maintain such an association for any A-IOT device discovered by the reader.

From this perspective, we think the paging ID visibility is at least needed for CFRA cases. The reader knows the paging ID to be used in CFRA. Once the device has responded in inventory procedure, the reader associate an AS ID to this Paging ID. Some may argue that for CBRA case, the reader can learn each device’s individual device ID from Msg 3 and then associate this device ID to a new AS ID assigned to the device so that the reader does not need to know the group paging ID. But in CFRA, the paging ID itself is the device ID and there is no “Msg3” for the reader to learn the device ID from the D2R message. So, we think the paging ID needs to be exposed to the reader’s MAC layer at least for CFRA case.

Proposal 4	At least for CFRA paging, Paging ID is visible to reader’s MAC layer.

### 2.4 Transaction ID in other R2D messages

For CFRA, the paging message is likely not to be repeated. The reader will determine that the targeted device is not in the coverage if CFRA fails, so there is no need to have a transaction ID.

Proposal 5	Paging message for CFRA does not include transaction ID.

As the command-only procedure is not supported, so each command is part of “inventory + command” procedure, then there is no need to have a distinctive transaction identifier for the command procedure. Basically, the device is still in the same procedure as CBRA paging when it receives the command. Different command procedures for different devices can be distinguished by AS ID.

Proposal 6	A transaction ID is not included in R2D data/command message.

### 2.4 Size of Transaction ID

We assume the device will not need to remember multiple transaction IDs at the same time. But s very small-size transaction ID may wrap up too quickly and create some misunderstanding the device side.

For example, the device responds to a paging message, records its transaction ID, moves out-of-coverage (thereby missing the ensuing transactions from the same reader), then moves back in the prior reader coverage and detects the appearance of the same “transaction ID” although this is a completely new transaction. During this period, the device may still be able to maintain power due to a CW node is radiating the carrier-waves to charge the devices. So, the device will be able to maintain its “current” transaction ID. Hence, in this case, the device falsely believes the old transaction is still ongoing and no need to respond. Thus, if the transaction ID is just 1-bit or 2-bit, such a risk of error is really high.

So, we suggest to have a reasonably long transaction ID, such as 4-6 bit to mitigate this risk.

Proposal 7	4-6 bit “Transaction ID” is used for Paging message.

# 3 Conclusion

In this contribution, we discuss the paging issue for Ambient IoT, and have the following proposals:

Proposal 1 	R2D trigger message includes the same “transaction ID” as the one in Paging message.

Proposal 2 	RAN2 discuss whether the slot# needs to be included in the R2D trigger message.

Proposal 3 	A device shall abort the current transaction and respond to the matching Paging request with new “Transaction ID”, regardless of whether this new paging transaction is initiated from the same reader or different reader.

Proposal 4 	At least for CFRA paging, Paging ID is visible to reader’s MAC layer.

Proposal 5 	Paging message for CFRA does not include transaction ID .

Proposal 6 	A transaction ID is not included in R2D data/command message.

Proposal 7 	4-6 bit “Transaction ID” is used for Paging message.

# 4 References

[1] Chairman’s note R2-129

[2] Chairman’s note R2-129-bis