3GPP TSG-RAN WG2 Meeting #130	R2-2503724

St.Julians, Malta, 19-23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0039 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of egress Uu RLC channel determination for L2 U2N Relay UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If an U2N relay UE’s outgoing Uu SRAP PDU to gNB contains the UE ID matching the NW configuration of SRAP mapping, the UE will only determine egress RLC channel for SRB0 and SRB1 case, all other cases are excluded, but wrongly described in an “else if” branch. However, as the Level-1 “if” condition is already satisfied, it will never enter the Level-1 “else if” branch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Section 5.3.3.2 Folded the ”else if” branch which determines the egress Uu relay RLC channel for bearers other than SRB0/1 under the Level-1”if” branch. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Egress RLC Channel Determination for L2 U2N Relay UE Inter-operability: - If the UE is implemented according to the CR and the NW is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | U2N relay UE won’t relay any traffic other than SRB0/1 to gNB as it fails to determine the egress Uu Relay RLC channel for those traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

#### 5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU:

-	If the SRAP Data PDU is for SRB0:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB0 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB1 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if there is an entry in sl-RemoteUE-ToAddModList which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1):

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

End of Change