3GPP TSG-RAN WG2 Meeting #130	                                                                                   R2-2503741

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on Asymmetric DL sTRP UL mTRP

Agenda Item:    8.12.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on asymmetric DL sTRP UL mTRP [1]. In details, the follows issues are covered in Section 2,

- Further details on PL offset

- TCI state activation for asymmetric TRP

Our proposals are summarized in Section 3.

Discussion

Further details on PL offset

RRC open issue: How to capture PL offset behaviour in RRC for handover and resume case.

In the RRC open issue list, it needs to further discuss how to handle the cases for handover or RRCResume for the PL offset update,

| Asymmetric DL sTRP/UL mTRP | How to capture PL offset behaviour in RRC for handover and resume case. | To be discussed in contributions |
| --- | --- | --- |

The companies’ concern is how to perform delta configuration if the PL offset is updated by the PL offset update MAC CE. From our point of view, this is not a critical issue as it can be solved by gNB implementation.

Firstly, upon sending the PL offset update MAC CE to UE, the gNB knows the latest value used by UE for path loss calculation of UL-only TRP, so the source/anchor gNB can send the latest RRC configuration including the updated PL offset to the target cell for delta configuration.

Secondly, the gNB needs to include the PL offset in the RRCReconfiguration or RRCResume messages due to the pathlossOffset-r19 field with need R.

So we think there is no further spec impact for this issue. Therefore, it is proposed that,

Proposal 1: No further spec impact is needed for delta configuration upon handover or RRCResume procedure even if the PL offset had been updated by the PL offset update MAC CE.

MAC open issue suggested by rapporteur summary in Proposal 7

In the post email, the rapporteur proposed the potential problem for asymmetric TRP in LTM cell switch. The intention is to use the PL offset associated with the TCI state indicated in the LTM cell switch command for the first PUSCH transmission during RACH-less LTM.

However, companies had further concern on whether to pre-configure the PL offset associated with the TCI state in the candidate configuration, so the following issues are suggested by the rapporteur,

Proposal 7: Regarding PL offset in LTM, discuss - how to apply the PL offset in intra-DU LTM when PL offset is provided in LTM configuration (e.g., the PL offset for the indicated TCI state in the LTM cell switch command is applied for the first PUSCH transmission in RACH-less LTM cell switch); - whether to support PL offset in intra-DU LTM when PL offset is not provided in LTM configuration for the target cell.

Before discuss above issues, one more thing needs to be clarified. In the current RRC running CR, the PL offset is associated with the TCI-State/TCI-UL-State-r17 in the ServingCellConfig but not associated with the CandidateTCI-State-r18/ CandidateTCI-UL-State-r18 configured in ltm-TCI-Info-r18 per LTM candidate. However, the TCI state in the LTM command MAC CE refers to the one configured in ltm-TCI-Info-r18 per LTM candidate. So how does UE get the PL offset associated with the TCI state indicated in the LTM command MAC CE?

TCI-UL-State-r17 ::= SEQUENCE { tci-UL-StateId-r17 TCI-UL-StateId-r17, … [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]], [[ pathlossOffset-r19 ENUMERATED { dB-12, dB-8, dB-4, dB0, dB4, dB8, dB12, dB16, dB20, dB24, dB28, dB32, dB36, dB40, dB44, dB48, dB52, dB56, dB60} OPTIONAL -- Need R ]] } CandidateTCI-UL-State-r18 ::= SEQUENCE { tci-UL-StateId-r18 TCI-UL-StateId-r17, referenceSignal-r18 CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId }, pathlossReferenceRS-Id-r18 PathlossReferenceRS-Id-r17 OPTIONAL, -- Need R tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL, -- Cond 2TA ul-PowerControl-r18 Uplink-powerControlId-r17 OPTIONAL, -- Cond separateTCI ... }

One straighfoward way is to add the associated PL offset for the candidate UL/Joint TCI state so that upon receiving the LTM command MAC CE, the UE can determine the corresponding PL offset for the first PUSCH transmission during RACH-less LTM.

Another way is after UE completes the application of candidate RRCReconfiguration, the UE can find the UL TCI state in the ServingCellConfig of target configuration which shares the same source RS with the TCI state indicated in the  LTM command MAC CE. By this way, UE can determine the corresponding PL offset for the first PUSCH transmission during RACH-less LTM.

From our view, the second way has no spec impact, and therefore it is suggested clarifying that,

Proposal 2: Upon receiving the LTM command MAC CE, it’s up to UE implementation to determine the PL offset used for first PUSCH of RACH-less LTM by the UL/Joint TCI state in the ServingCellConfig of target configuration which shares the same source RS with the TCI state indicated in the of LTM command MAC CE. There is no spec impact.

Then if the PL offset is pre-configured in the LTM candidate RRCReconfiguration, as suggested by our Proposal 2, it is straightforward to use the PL offset associated with the indicated TCI state in the LTM command for the first PUSCH transmission of RACH-less LTM.

If the NW cannot estimate the PL offset for the UL-only TRP of the candidate cell, it’s up to NW implementation to only configure the L1 measurement with the beams associated with the DL/UL TRP. By this way, the NW can only trigger UE to perform LTM to the DL/UL TRP of a candidate cell, and then re-configure UE with the PL offset (if acquired by the NW) for the UL-only TRP in the current serving cell.

Based on above analysis, we proposed that,

Proposal 3: For the co-existence of asymmetric TRP with intra-DU LTM, it’s up to NW implementation to only configure the L1 measurement with the beams associated with the DL/UL TRP, if the candidate cell cannot estimate the PL offset for the UL-only TRP.

TCI state activation for asymmetric TRP

MAC open issue suggested by rapporteur summary in Proposal 8

RAN1 reached the following agreement for the TCI state activation in asymmetric TRP,

Conclusion RAN1#117 For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

Then companies have different views on whether to enhance the TCI state activation framework to support the asymmetric DL sTRP/UL mTRP deployment. Therefore, rapporteur suggested discussing this issue by companies’ contribution as summarized in the post email discussion,

Proposal 8: Discuss whether/how Rel-18 TCI state activation MAC CEs for sDCI mTRP is applied for asymmetric DL sTRP/UL mTRP.

For the asymmetric TRP configured with R17 unified TCI framework, companies share the common understanding that the R17 Unified TCI States Activation/Deactivation MAC CE can map one joint TCI state or one DL TCI state + one UL TCI state to one DCI codepoint by the current format, which can satisfy the RAN1 requirement.

Proposal 4a: RAN2 to confirm the Unified TCI States Activation/Deactivation MAC CE can satisfy the RAN1 requirement. No further MAC impact is needed for the asymmetric TRP with configured R17 unified TCI state framework.

For the asymmetric TRP configured with R18 unified TCI framework, i.e., sDCI mTRP with unified TCI state framework, when the UE is configured with separated TCI state mode, it is easy to support activating one DL TCI state + up to two UL TCI states as the Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States has the Fi,j field to indicate the present of each DL TCI state mapped to one codepoint.

For the joint TCI state mode, the only RAN1 requirement is to activate up to two joint TCI states, which is also supported by the current Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States as there is also Fi,j field to indicate whether the codepoint i is mapped with one or two joint TCI states.

In summary, from the MAC perspective, both the Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States and the Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States do not need to be enhanced. So we suggest just following RAN1 conclusion. Therefore, it is proposed that,

Proposal 4b: RAN2 to confirm the current format of the Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States and the Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States can satisfy the RAN1 requirement for the asymmetric TRP configured with R18 unified TCI framework. No further MAC impact is needed.

Conclusion

Based on the previous analysis in Section 2, our contributions are summarized as follows:

Further details on PL offset

Proposal 1: No further spec impact is needed for delta configuration upon handover or RRCResume procedure even if the PL offset had been updated by the PL offset update MAC CE.

Proposal 2: Upon receiving the LTM command MAC CE, it’s up to UE implementation to determine the PL offset used for first PUSCH of RACH-less LTM by the UL/Joint TCI state in the ServingCellConfig of target configuration which shares the same source RS with the TCI state indicated in the of LTM command MAC CE. There is no spec impact.

Proposal 3: For the co-existence of asymmetric TRP with intra-DU LTM, it’s up to NW implementation to only configure the L1 measurement with the beams associated with the DL/UL TRP, if the candidate cell cannot estimate the PL offset for the UL-only TRP.

TCI state activation for asymmetric TRP

Proposal 4a: RAN2 to confirm the Unified TCI States Activation/Deactivation MAC CE can satisfy the RAN1 requirement. No further MAC impact is needed for the asymmetric TRP with configured R17 unified TCI state framework.

Proposal 4b: RAN2 to confirm the current format of the Enhanced Unified TCI States Activation/Deactivation MAC CE for Separate TCI States and the Enhanced Unified TCI States Activation/Deactivation MAC CE for Joint TCI States can satisfy the RAN1 requirement for the asymmetric TRP configured with R18 unified TCI framework. No further MAC impact is needed.

Reference

[1] R2-2502982, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others