3GPP TSG-RAN WG2 Meeting #130	                                                                                   R2-2503742

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on UE-initiated Beam Reporting

Agenda Item:    8.12.3

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issue related to the UE initiated beam reporting [1]. In details, the follows issues are covered in Section 2. Our proposals are summarized in Section 3.

Discussion

MAC open issue suggested by rapporteur summary in Proposal 3

During last meeting discussion, RAN2 achieved common understanding that,

Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. FFS if any other MAC impact for UL skipping

It still remains open whether to have MAC impact if UL skipping is configured to UE.

As suggested by the rapporteur summary in the post email discussion,

Proposal 1: UL skipping is not applicable to mode-B type-1 CG event-triggered beam report. Proposal 2: For Rel-16 UL skipping (enhancedSkipUplinkTxDynamic), the UCI for mode-A DG-based UE-initiated report follows the existing procedure (i.e., MAC PDU is generated), there is no MAC impact. Proposal 3: For Rel-15 UL skipping (skipUplinkTxDynamic is configured), discuss whether to generate MAC PDU for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH.

It is common understanding (Proposal 1) that the UL skipping is not applicable to the mode-B type-1 CG event-triggered beam report, because the current MAC running CR captured that the type-1 CG for event-triggered beam report is transparent to the MAC layer,

The MAC entity shall not include the configured grant Type 1 provided by RRC for UE-initiated reporting as an uplink grant to generate MAC PDU in procedures specified in this clause and in clause 5.4.

For the mode-A DG-based UE-initiated report, all companies agreed (Proposal 2) that the current MAC spec has covered the RAN1 requirement to generate the MAC PDU for UCI reporting in DG PUSCH even if enhancedSkipUplinkTxDynamic-r16 is configured,

The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and … 3> not generate a MAC PDU for the HARQ entity.

Then the discussion (Proposal 3) is whether to enhance to ensure the generation of MAC PDU if R15 UL skipping skipUplinkTxDynamic is configured. As in the current MAC spec, it only captures the case of “if” condition on whether the aperiodic CSI transmitted on PUSCH is triggered by DCI,

1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and … 3> not generate a MAC PDU for the HARQ entity.

From our view, this sentence is enough as according to the RAN1 agreement,

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A - Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2 - FFS: DCI format 0_3 - FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

That means RAN1 considers the step 2 of mode-A to trigger the UCI reporting is the same as legacy DCI triggered aperiodic CSI reporting. So we think no further change is needed in MAC for this case. Therefore, it is proposed that,

Proposal 1: To support the mode-A UCI reporting in DG PUSCH multiplexed with the MAC PDU if the R15 skipUplinkTxDynamic is configured, no further change is needed in the current MAC spec, i.e., it is already covered by the “if” condition of no aperiodic CSI requested for this PUSCH transmission.

Conclusion

Based on the previous analysis in Section 2, our contributions are summarized as follows:

Proposal 1: To support the mode-A UCI reporting in DG PUSCH multiplexed with the MAC PDU if the R15 skipUplinkTxDynamic is configured, no further change is needed in the current MAC spec, i.e., it is already covered by the “if” condition of no aperiodic CSI requested for this PUSCH transmission.

Reference

[1] R2-2502982, Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others