3GPP TSG-RAN WG2 Meeting #130	R2-2503747

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.2

Source: 	Fujitsu

Title:	Discussion on inter-CU LTM

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to provide our considerations on open issues for intra-CU and inter-CU LTM.

- Discussions

  - Fast LTM recovery for inter-CU LTM

In RAN2#129bis meeting, RAN2 agreed partially on fast LTM recovery for inter-CU LTM [1].

- Agreement in RAN2#129bis meeting

At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Regarding this, there are two FFSs according to the open issue list [2] and we will discuss each by each.

- FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration.

- FFS if fast failure recovery with different Rel-19 IDs is allowed.

    - When does the UE revert back to the source PCell configuration after inter-CU LTM failure?

In RAN2#129bis meeting, two options were raised in the discussion.

- After inter-CU LTM failure, UE does not revert back to the source PCell configuration and performs fast LTM recovery. If UE fails the fast LTM recovery, then UE reverts back to the original source PCell configuration and performs legacy RRC re-establishment procedure.

- After inter-CU LTM failure, UE reverts back to the source PCell configuration and performs fast LTM recovery.

When considering option (i), the following two points should be taken into account.

- If the UE selects a cell which is not one of the LTM candidate cells, the UE performs legacy RRC re-establishment procedure, and this requires AS security information at the source PCell. This means that reverting back procedure must be performed before performing legacy RRC re-establishment procedure. However, if this is captured, specification would be complicated.

- If the UE fails fast LTM recovery, the UE reverts back to the configuration two steps back (i.e., before inter-CU LTM failure before fast LTM recovery). Such UE behaviour is never required so far, and it may have an impact on UE implementation.

Observation 1: If the UE does not revert back to the source PCell configuration before performing fast LTM recovery, this leads to increasing complexity of the specifications and impact on UE implementation.

About option (ii), keystream reuse issue explained below happens if the UE selects the same target cell again for fast LTM recovery, but this can be avoided by UE implementation.

After inter-CU LTM failure and reverting back to source PCell configuration, the UE has the KgNB used in the source PCell. If the UE selects the same target cell for fast LTM recovery, the same key is derived from the KgNB, the same PCI and the same frequency ARFCN-DL as used in the failed inter-CU LTM. In addition, because the PDCP entity for SRB1 is re-established both during failed inter-CU LTM and fast LTM recovery, RRCReestablishmentComplete message is transmitted with COUNT = 0 both in inter-CU LTM and fast LTM recovery. The same key and the same COUNT value in the same bearer would lead to keystream reuse issue. However, if the UE does not perform fast LTM recovery when it selects the same target cell as before LTM failure, this issue can be avoided.

Observation 2: If the UE reverts back to the source PCell configuration before performing fast LTM recovery, keystream reuse issue happens if the UE selects the same target cell again for fast LTM recovery. However, this can be avoided by UE implementation.

Considering the concerns in option (i) and the fact that the issue in option (ii) can be solved, we propose that UE revert back to source PCell config after inter-CU LTM cell switch failure.

Proposal 1: The UE reverts back to the source PCell configuration after inter-CU LTM cell switch failure.

If this is agreed, to avoid keystream reuse issue, the UE shall ensure that it does not perform fast LTM recovery if it selects the same target cell.

Proposal 2: The UE shall ensure that it does not perform fast LTM recovery if it selects the same target cell.

    - Is fast LTM recovery allowed when the selected cell has different Rel-19 ID from the target cell?

In RAN2#129bis meeting, it was agreed that fast LTM recovery is performed after inter-CU LTM failure when the selected cell has the same Rel-19 ID as the target cell.

In addition to that, we need to consider the following two cases.

- UE selects a cell which has the same Rel-19 ID as the source cell

- UE selects a cell which has the different Rel-19 ID from the source cell or the target cell

Case (i) is considered as intra-CU LTM because the UE configuration is of the source PCell when the UE performs the fast LTM recovery. Although it is intra-CU LTM, security key refresh should be performed to re-establish PDCP and RLC entities. As the source gNB already has the NCC value used in the failed inter-CU LTM, no additional signalling is needed. The source gNB and the UE can compute a new KgNB for the selected cell by vertical key derivation or horizontal key derivation based on the NCC. Therefore, case (i) seems to work, but RAN2 should discuss whether there exist problems or not.

About case (ii), the gNB to which the selected cell belongs (hereinafter called “the selected gNB”) may be another gNB than the source gNB or the target gNB (hereinafter called “the third gNB”). If the selected gNB is the third gNB, as it does not know the NCC value used in the failed inter-CU LTM, additional signalling would be needed to transfer the NCC value from the source gNB to the third gNB.

Observation 3: If the UE selects a cell which does not belong to the source gNB or the target gNB, additional signalling for transferring the NCC value from the source gNB to the selected gNB is needed to perform fast LTM recovery.

This will have the RAN3 impact, and considering the time frame RAN2 should avoid this case.

Proposal 3: RAN2 not to support fast LTM recovery after the UE fails LTM cell switch which requires security key refresh if the selected cell has a different Rel-19 ID from the target cell or the source cell.

Proposal 4: RAN2 should discuss whether it supports the fast LTM recovery if the selected cell has the same Rel-19 ID as the source cell.

  - Issues on fast LTM recovery to intra-DU cell after inter-DU LTM failure

Since Rel-18 LTM, fast LTM recovery after LTM failure is supported, including fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure. However, there is a potential data loss issue in case of fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure. We explain the issue with Figure 1.

Figure 1

1. The UE receives LTM CSC MAC CE and switches the serving cell from Cell-1 (ltm-ServingCellNoResetId = 1) to Cell-3 (ltm-NoReserId = 2). As ltm-NoResetId in the target cell is different from ltm-ServingCellNoResetId, RLC re-establishment is performed and RLC buffer becomes empty. Also, PDCP data recovery for AM DRB is performed. Then, the LTM cell switch fails. PDCP data recovery also fails.

2. The UE reverts back to the source PCell (Cell-1) configuration. In this reverting back operation, RLC SDUs, SDU segments and PDUs cannot be retrieved as explained in [3]. Therefore, RLC buffer remains empty.

3. During RRC re-establishment procedure, the UE selects Cell-2 (ltm-NoResetId = 1) and performs fast LTM recovery to Cell-2. As ltm-NoResetId in the target cell is the same as ltm-ServingCellNoResetId, RLC re-establishment is not performed. Also, PDCP data recovery for AM DRB is not performed even though RLC buffer is empty. This causes data loss issue.

Observation 4: In the case of fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure, data loss issue will be caused. This is because:

1. At the failed inter-DU LTM cell switch, RLC re-establishment and PDCP data recovery for AM DRB is performed. But PDCP data recovery also fails due to LTM cell switch failure.

2. When the UE reverts back to the source PCell configuration, RLC SDUs, SDU segments and PDUs cannot be retrieved as they are already discarded.

3. If the UE performs fast LTM recovery to an intra-DU cell, PDCP data recovery is not performed even though RLC SDUs, SDU segments and PDUs are already discarded.

We see that the issue is not a corner case. Also, data loss on AM DRB will affect application services. Thus, we need to discuss a solution in Rel-19.

Proposal 5: RAN2 to discuss a solution to solve the data loss issue when the UE performs fast LTM recovery to an intra-DU LTM candidate cell after inter-DU LTM cell switch failure.

- Conclusion

We have the following observations and proposals.

Observation 1: If the UE does not revert back to the source PCell configuration before performing fast LTM recovery, this leads to increasing complexity of the specifications and impact on UE implementation.

Observation 2: If the UE reverts back to the source PCell configuration before performing fast LTM recovery, keystream reuse issue happens if the UE selects the same target cell again for fast LTM recovery. However, this can be avoided by UE implementation.

Observation 3: If the UE selects a cell which does not belong to the source gNB or the target gNB, additional signalling for transferring the NCC value from the source gNB to the selected gNB is needed to perform fast LTM recovery.

Observation 4: In the case of fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure, data loss issue will be caused. This is because:

1. At the failed inter-DU LTM cell switch, RLC re-establishment and PDCP data recovery for AM DRB is performed. But PDCP data recovery also fails due to LTM cell switch failure.

2. When the UE reverts back to the source PCell configuration, RLC SDUs, SDU segments and PDUs cannot be retrieved as they are already discarded.

3. If the UE performs fast LTM recovery to an intra-DU cell, PDCP data recovery is not performed even though RLC SDUs, SDU segments and PDUs are already discarded.

Proposal 1: The UE reverts back to the source PCell configuration after inter-CU LTM cell switch failure

Proposal 2: The UE shall ensure that it does not perform fast LTM recovery if it selects the same target cell.

Proposal 3: RAN2 not to support fast LTM recovery after the UE fails LTM cell switch which requires security key refresh if the selected cell has a different Rel-19 ID from the target cell or the source cell

Proposal 4: RAN2 should discuss whether it supports the fast LTM recovery if the selected cell has the same Rel-19 ID as the source cell.

Proposal 5: RAN2 to discuss a solution to solve the data loss issue when the UE performs fast LTM recovery to an intra-DU LTM candidate cell after inter-DU LTM cell switch failure.

# References

[1] R2-2502981, “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung)

[2] R2-2504126, “Open issue list for inter-CU LTM and CLTM”, Ericsson

[3] R2-2402235, “Fast Recovery with LTM Candidates”, MediaTek Inc.