3GPP TSG-RAN WG2 Meeting #130											R2-2503757

St Julian, Malta, 19th– 23rd May 2025

Agenda item:		8.5.4

Title: 	Remaining issues of common signal/channel adaption

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the remaining issues of common signal/channel adaption.

- Discussion

  - Paging occasion adaptation

The following agreements have been made in the recent meeting on paging adaptation [1]:

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

If we introduce N with new value T/32 and Ns with new value 8, the following new parameters are needed for firstPDCCH-MonitoringOccasionOfPO:

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359)

Proposal 1: The above new values for firstPDCCH-MonitoringOccasionOfPO should be introduced in 38.331.

  - PRACH adaptation in time domain

RAN1 have agreed to support DCI based adaptation for additional PRACH resources with the LS [1] sent to RAN2 to confirm the feasibility:

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

From RAN2’s perspective, after checking the Short Message field as follows, we understand it is feasible.

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 | systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period. |
| 5 – 8 | Not used in this release of the specification, and shall be ignored by UE if received. |

Proposal 2: RAN2 confirm that it is feasible to use 1 bit from the 5-8 within the Short Message field to indicate PRACH adaption.

Also, we need to consider the configuration of the additional PRACH resources. Since the common adaption is applicable for UE in idle/inactive and connected, we understand it is more appropriate to configure the additional PRACH resources as cell specific parameters and broadcast via system information.

Considering that the SIBxx is introduced for on demand SIB1, which is a separate feature from PRACH adaption, we understand it is better to include the additional PRACH resources in SIB1.

Proposal 3: The additional PRACH resources should be cell specific and broadcast via system information, e.g. SIB1.

The following parameters for PRACH adaption are included in the latest higher layers parameter list from RAN1 [2], and should be included in system information.

| PRACH-Config Index | PRACH configuration index for additional RACH configuration. | FFS: Value range |
| --- | --- | --- |
| msg1-FrequencyStart | Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0, at least for 4-step RACH. | INTEGER (0..maxNrofPhysicalResourceBlocks-1), |
| addl-ssb-Periodicity | Additional SSB burst periodicity for the Scell. | same as ssb-periodicity X≤2 additional SSB burst periodicities from SSB burst periodicities in ssb-periodicity |
| prach-SubsetMask-Index-Adaptation | Single PRACH mask to identify the subset of the additional PRACH resources applicable at least for adaptation for DCI 1_0 with P-RNTI. | Value range {0,1,2,3} |
| msg1-FDM | The number of PRACH transmission occasions FDMed in one time instance, at least for 4-step RACH. | same as msg1-FDM |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | Configuration of number of SSB per RO and CB preambles per SSB. | same as ssb-perRACH-OccasionAndCB-PreamblesPerSSB |

Proposal 4: Based on the latest input from RAN1, the following parameters should be included in system information for PRACH adaption:

- PRACH-Config Index

- msg1-FrequencyStart

- addl-ssb-Periodicity

- prach-SubsetMask-Index-Adaptation

- msg1-FDM

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- Conclusion and proposals

Based on the analysis in previous sections, the following proposals are given:

Proposal 1: The following new values for firstPDCCH-MonitoringOccasionOfPO should be introduced in 38.331.

sCS15KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS30KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS60KHZoneThirtysecondT											SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17920),

sCS120KHZoneThirtysecondT											 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneThirtysecondT                    						SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..143359)

Proposal 2: RAN2 confirm that it is feasible to use 1 bit from the 5-8 within the Short Message field to indicate PRACH adaption.

Proposal 3: The additional PRACH resources should be cell specific and broadcast via system information, e.g. SIB1.

Proposal 4: Based on the latest input from RAN1, the following parameters should be included in system information for PRACH adaption:

- PRACH-Config Index

- msg1-FrequencyStart

- addl-ssb-Periodicity

- prach-SubsetMask-Index-Adaptation

- msg1-FDM

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- Reference

- R1-2503086 LS on DCI-based PRACH adaptation

- R1-2503155 Consolidated Rel-19 higher layers parameter list Post RAN1#120bis