3GPP TSG-RAN WG2 #130	R2-2503768

St. Julians, Malta, May 19th – 23rd, 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on Store & Forward operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2		Support of Store & Forward

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for last meeting RAN2#129bis[2], the following agreements are made for S&F operation:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

This paper attempts to investigate about consideration on S&F operation based on the above objectives and agreements.

- Discussion

2.1 Time information for mode transition

In the last meeting, it was agreed to introduce the time information for the normal mode to S&F mode transition. However, it is still FFS on signaling details including a perspective whether we can link the time information to other legacy information (i.e. t-Service). For the signaling details, 3 options are mentioned in the open issue list [3] as follows:

Option 1: It is up to NW implementation to set the legacy t-Service as the transition time from normal mode to S&F mode.

Option 2: Introduce a new indication for the transition time from normal mode to S&F mode for the S&F UEs.

Option 3: Using the agreed time information in SIB31 for both directions of transition. UE determines which direction it is based on whether the S&F indication is present.

We prefer Option 3 for the following reasons. Firstly, we think t-Service should not be linked to the time information. We think there would be cases where UEs supporting normal mode and S&F mode would want to continue (or resume) communication even when the communicating satellite transitions from normal mode to S&F mode. If the time information is informed by using t-Service, the UE may not be able to distinguish whether satellite is about to stop NTN service or going to S&F mode. Secondly, Option 3 does not need any additional IEs in the message other than already agreed IEs. Therefore, we would like to propose to adopt Option 3 to use the agreed time information for both directions of transition, and UE distinguishes the direction according to the S&F indication.

Proposal 1: (RRC-1) RAN2 to use the agreed time information for both directions of transition, and UE distinguishes the direction according to the S&F indication

Additionally, in the open issue list, it is also mentioned about S&F related information (current operation mode, transition time) for neighbour cells (RRC-2). We think that current operation mode of neighbour cells is useful for UE to select a cell in normal mode when serving satellite is going to S&F mode. Furthermore, transition time information of neighbour cells is also useful to select a cell with a longer remaining time in normal mode. Therefore, we would like to propose that S&F related information for neighbour cells (current operation mode, transition time) could be provided to UE.

Proposal 2: (RRC-2) S&F related information for neighbour cells (current operation mode, transition time) could be provided to UE

However, there might be cases where it is difficult for a satellite to update the S&F related information for neighbour cells frequently. Therefore, we think that it can be up to network implementation whether S&F related information for neighbour cells is included in system information.

Proposal 2a: (RRC-2) It can be up to network implementation whether S&F related information for neighbour cells is included in system information

2.2 Satellite ID List

In the last meeting, it was discussed to use satellite ID list provided by MME for prioritization of cell reselection. We agree to use satellite ID list provided by MME for the prioritization to avoid unnecessary signaling so that UE power consumption and communication latency could be reduced.

Proposal 3: Satellite ID list provided by MME could be used for prioritization of cell reselection

Additionally, to use satellite ID list for cell reselection, UE would need to monitor SIB33 to acquire neighbour cells’ ID. However, current SIB33 does not include frequency(-es) information. We think that it is beneficial to include frequency(-es) information associated to satellite ID in SIB33 to optimize cell reselection procedure with satellite ID list.

Proposal 4: RAN2 to include frequency(-es) information associated to satellite ID in SIB33

- Summary and proposal

This paper discussed about consideration on S&F operation and proposed the following:

Proposal 1: (RRC-1) RAN2 to use the agreed time information for both directions of transition, and UE distinguishes the direction according to the S&F indication

Proposal 2: (RRC-2) S&F related information for neighbour cells (current operation mode, transition time) could be provided to UE

Proposal 2a: (RRC-2) It can be up to network implementation whether S&F related information for neighbour cells is included in system information

Proposal 3: Satellite ID list provided by MME could be used for prioritization of cell reselection

Proposal 4: RAN2 to include frequency(-es) information associated to satellite ID in SIB33

- References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2502983, “Report from Break-out session on NR-NTN and IoT-NTN”

[3] R2-250xxxx, “RRC open issues for IoT NTN”