3GPP TSG-RAN WG2 Meeting # 130	R2-2503771

St Julian’s, Malta, May. 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2078 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the power ramping indicated to PHY |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Qualcomm Incorporated, Huawei, MediaTek Inc, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to section 7.1.1 of TS 38.213 as quote below, the transmission power of Msg3 PUSCH is based on the “total power ramp-up”. However, in section 5.1.4 of TS 38.321, the text used for indicating the power ramping value to PHY was specified differentlly since Rel-15, which caused two different UE implements, i.e. the transmission power of Msg3 PUSCH is calculated according to the “total power ramp-up” as specified in TS 38.213 or the text in TS 38.321. To allow all UE implentantions without NBC impact, the example of calculating power ramping in 5.1.4 in the MAC specification should be removed. TS 38.213: ## 7.1 Physical uplink shared channel … 7.1.1 UE behaviour … - If the UE receives a random access response message in response to a PRACH transmission or a MsgA transmission on active UL BWP b of carrier f of serving cell c as described in clause 8 - fb,f,c0,l=∆Prampup,b,f,c+δmsg2,b,f,c, where l=0 and - δmsg2,b,f,c is a TPC command value indicated in a random access response grant of the random access response message corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant of the random access response message corresponding to a MsgA transmission according to Type-2 random access procedure with RAR message(s) for fallbackRAR, on active UL BWP b of carrier f of serving cell c, and - and ∆Prampup_requested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers [11, TS 38.321] for carrier f in the serving cell c, MRB,b,f,cPUSCH(0) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP b of carrier f of serving cell c, and ∆TF,b,f,c0 is the power adjustment of first PUSCH transmission on active UL BWP b of carrier f of serving cell c. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.1.4, the example of calculating the power ramping value indicated to PHY is removed. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: 2-step RACH Inter-operability: If the network implements the CR and the UE does not, there is no interoperability issue. If the UE implements the CR and the network does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether or not two different UE implementations are allowed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	if the Random Access Preamble was transmitted on a non-terrestrial network:

3>	if the Random Access Preamble is transmitted with repetitions:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6].

3>	else:

4>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion as specified in TS 38.213 [6].

2>	else if the Random Access Preamble is transmitted with repetitions:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6].

2>	else:

3>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission.

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission on the search space indicated by recoverySearchSpaceId is received from lower layers on the Serving Cell where the preamble was transmitted; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.

1>	else if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the RA-RNTI and the received TB is successfully decoded:

2>	if the Random Access Response contains a MAC subPDU with Backoff Indicator:

3>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

2>	else:

3>	set the PREAMBLE_BACKOFF to 0 ms.

2>	if the Random Access Response contains a MAC subPDU with Random Access Preamble identifier corresponding to the transmitted PREAMBLE_INDEX (see clause 5.1.3):

3>	consider this Random Access Response reception successful.

2>	if the Random Access Response reception is considered successful:

3>	if the Random Access Response includes a MAC subPDU with RAPID only:

4>	consider this Random Access procedure successfully completed;

4>	indicate the reception of an acknowledgement for SI request to upper layers.

3>	else:

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see clause 5.2);

5>	indicate the preambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers;

5>	if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

6>	ignore the received UL grant.

5>	else:

6>	process the received UL grant value and indicate it to the lower layers.

4>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

5>	consider the Random Access procedure successfully completed.

4>	else:

5>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

5>	if this is the first successfully received Random Access Response within this Random Access procedure:

6>	if the transmission is not being made for the CCCH logical channel:

7>	indicate to the Multiplexing and assembly entity to include a C-RNTI MAC CE in the subsequent uplink transmission.

6>	if the Random Access procedure was initiated for SpCell beam failure recovery and spCell-BFR-CBRA with value true is configured:

7>	if there is at least one Serving Cell of this MAC entity configured with two BFD-RS sets:

8>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

7>	else:

8>	indicate to the Multiplexing and assembly entity to include a BFR MAC CE or a Truncated BFR MAC CE in the subsequent uplink transmission.

6>	else if the Random Access procedure was initiated for beam failure recovery of both BFD-RS sets of SpCell:

7>	indicate to the Multiplexing and assembly entity to include an Enhanced BFR MAC CE or a Truncated Enhanced BFR MAC CE in the subsequent uplink transmission.

6>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity and store it in the Msg3 buffer.

NOTE 1:	If within a Random Access procedure, an uplink grant provided in the Random Access Response for the same group of contention-based Random Access Preambles has a different size than the first uplink grant allocated during that Random Access procedure, the UE behavior is not defined.

1>	if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

1>	if ra-ResponseWindow configured in RACH-ConfigCommon expires, and if the Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX has not been received:

2>	consider the Random Access Response reception not successful;

2>	increment PREAMBLE_TRANSMISSION_COUNTER by 1;

2>	if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1:

3>	if the Random Access Preamble is transmitted on the SpCell:

4>	indicate a Random Access problem to upper layers;

4>	if this Random Access procedure was triggered for SI request:

5>	consider the Random Access procedure unsuccessfully completed.

3>	else if the Random Access Preamble is transmitted on an SCell:

4>	consider the Random Access procedure unsuccessfully completed.

2>	if the Random Access procedure is not completed:

3>	if the Random Access Preamble is transmitted with repetitions and neither contention-free Random Access Resources nor Random Access resources for SI request have been provided for this Random Access procedure:

4>	if PREAMBLE_TRANSMISSION_COUNTER = [preambleTransMax-Msg1-Repetition] + 1; or

4>	if PREAMBLE_TRANSMISSION_COUNTER = 2 × [preambleTransMax-Msg1-Repetition] + 1:

5>	if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available:

6>	select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure;

6>	initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

3>	select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF;

3>	if the criteria (as defined in clause 5.1.2) to select contention-free Random Access Resources is met during the backoff time:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2).

3>	else if the Random Access procedure for an SCell is performed on uplink carrier where pusch-Config is not configured:

4>	delay the subsequent Random Access transmission until the Random Access Procedure is triggered by a PDCCH order with the same ra-PreambleIndex, ra-ssb-OccasionMaskIndex, and UL/SUL indicator TS 38.212 [9].

3>	else:

4>	perform the Random Access Resource selection procedure (see clause 5.1.2) after the backoff time.

The MAC entity may stop ra-ResponseWindow (and hence monitoring for Random Access Response(s)) after successful reception of a Random Access Response containing Random Access Preamble identifiers that matches the transmitted PREAMBLE_INDEX.

HARQ operation is not applicable to the Random Access Response reception.

NOTE 2:	For the case that RAR PDSCH bandwidth is larger than the bandwidth the eRedCap UE can receive or process per slot, and the UL grant in RAR indicates that the time is not enough for Msg3 transmission, as specified in TS 38.213 [6], it is up to UE implementation, e.g. either to consider the Random Access Response reception not successful, or transmit Msg3.

END OF CHANGE