3GPP TSG-RAN WG2 Meeting #130	R2-2503800

# St.Julians, Malta, May 19th – 23rd, 2025

Title		Discussion on error causes for AI/ML positioning

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	8.1.2.3 LCM for Positioning use case

WID/SID	NR_AIML_air-Core

# 1		Introduction

In RAN2#128 post email discussion [1], RAN2 discussed UE’s error causes for AI/ML positioning method. Some candidate causes were listed as below;

-- ASN1START NR-AI-ML-TargetDeviceErrorCauses-r19 ::= SEQUENCE { cause-r19 ENUMERATED {assistanceDataInconsistentBetweenTrainingAndInference, thereWereNotEnoughSignalsReceivedForUeBasedAI-ML, resourceOrProcessingCapacityIssueForAIML, battery-low, ... }, ... } -- ASN1STOP

In RAN2#129 meeting [2], the content of error cause was postponed to stage-3 discussion.

Agreements

4: 	The content of error cause is discussed while drafting stage3 CRs.

In RAN2#129bis post email discussion [3], companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-LocationServerErrorCauses and NR-DL-AI-ML-TargetDeviceErrorCauses.

Open issue LPP-13: Location server error causes Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-LocationServerErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.) Open issue LPP-14: Target device error causes Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-TargetDeviceErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

In this paper, we provide our view on error causes for AI/ML positioning Case 1.

# 2		Discussion

The following observations are based on the current specification.

According to clause 5.4.3 (LPP Error Detection) of TS 37.355, upon receiving any LPP message, the receiving entity (e.g., LMF or UE) must attempt to decode the message and verify the presence of any errors. The current specification defines specific behaviors for the receiving entity–for instance, if a decoding error is encountered, the receiving entity returns an LPP Error message (e.g., lppMessageHeaderError or lppMessageBodyError) and discards the received message. However, no explicit actions are defined for the transmitting entity, and such actions are left to implementation. Nevertheless, expected behavior may be inferred—for example, upon receiving a lppMessageHeaderError, the transmitter may attempt to retransmit the message.

Similarly, during LPP Assistance Data and Location Information procedures, the UE and LMF may include error elements specific to the positioning method in use (e.g., NR-DL-TDOA-Error-r16 in the DL-TDOA method). While no explicit procedures are defined for handling such errors, expected behavior may be assumed–for instance, if the LMF receives an error cause such as assistance-data-missing, it is expected to retransmit the relevant assistance data.

Observation 1.	Legacy (non-AI/ML) positioning error causes can provide useful guidance regarding appropriate next actions.

Based on previous meeting agreements, we observe the following.

As agreed in RAN2#127bis, RAN2#129 and RAN2#129bis, the UE may report applicable functionality to the LMF using the LPP ProvideCapabilities message when there is a change in its capabilities, without requiring additional control from the LMF.

Agreements (RAN2#127bis meeting)

1: 	The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline:

Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP 		request capabilities message.

Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at 	the UE side.

Step 3: LMF sends the LPP provide assistance data message (which may contain network side 	additional condition).

Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities 		message.

Step 5: The LMF requests the inferred location information using the LPP request location 		information message.

Step 6: UE reports the inferred location using LPP provide location information message.

Agreements (RAN2#129 meeting)

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

Agreements for positioning case 1 (RAN2#129bis meeting)

- UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

Before receiving an LPP RequestLocationInformation message, the UE is allowed to proactively send the LPP ProvideCapabilities message upon any change in functionality (e.g., AI/ML-based positioning). This capability reporting facilitates the support of AI/ML-based positioning in subsequent LPP Location Information procedure.

After receiving an LPP RequestLocationInformation message, as agreed in RAN2#128 meeting, an AI/ML-based positioning method is considered non-applicable when the LMF requests location estimation with AI/ML positioning method but the UE is unable to execute the AI/ML method. In such cases, the UE responds with an LPP ProvideLocationInformation message including an appropriate error cause. This error reporting allows the LMF to fallback to a legacy (non-AI/ML) positioning method.

Agreements (RAN2#128 meeting) 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

Observation 2.	AI/ML positioning error causes should enable fallback to a legacy (non-AI/ML) positioning method when AI/ML positioning is not applicable.

We propose adding two error causes to existing NR-DL-TDOA-TargetDeviceErrorCauses-r16 structures.

Unlike legacy (non-AI/ML) methods, AI/ML-based positioning can impose additional burdens on the UE, such as low battery, limited processing power, or memory exhaustion. While these low-level causes could, in theory, be reported individually, doing so would require the LMF to interpret and map each cause to appropriate fallback or retry actions. However, such detailed reporting increases complexity and leads to implementation-specific behavior divergence across vendors. Moreover, these internal issues offer little actionable guidance to the LMF. A single abstracted error cause enables the LMF to apply a consistent fallback policy without needing to interpret internal UE conditions.

AI/ML-based positioning also depends heavily on runtime conditions such as memory availability, thermal status, and signal quality — all of which can vary rapidly. Reporting each transient limitation separately may result in excessive signaling and fragile error handling logic. Grouping such cases under a stable, abstracted error cause promotes more robust and adaptable system design.

Additionally, detailed low-level error causes may lead LMF implementations to adapt behavior to specific internal UE states, which can lead to overfitting to device-specific behaviors. A higher-level error abstraction supports a cleaner separation between network-side policy control and UE-side capability management.

We therefore propose introducing a single abstracted error cause: “AI/ML functionality not applicable.” Upon receiving this error cause, the LMF may fallback to a legacy (non-AI/ML) method. This abstraction is both sufficient and practical for handling internal UE limitations.

Proposal 1.	Use of abstracted error cause “AI/ML functionality not applicable” for UE-side limitations in AI/ML positioning.

One specific scenario arises when the assistance data provided by the LMF does not match the input features expected by the UE's pre-trained AI/ML model. This type of mismatch is not typical in traditional deterministic positioning methods but is a realistic and frequent issue in AI/ML-based approaches, due to the decoupled nature of model training and runtime inference. Inference accuracy depends on alignment between the assistance data and the model’s expected input—such as TRP configurations, reference signal (PRS) parameters, environmental conditions, network side additional condition (e.g., info #7), inference configuration—depending on UE’s model implementation. A mismatch in any of these may prevent successful inference, even if the UE nominally supports the AI/ML method. Since the LMF cannot detect or resolve such inconsistencies autonomously, the UE must signal an error cause.

We propose introducing a dedicated error cause, “Assistance data incompatible with model,” to indicate that the provided assistance data does not match the requirements of the UE’s trained AI/ML model. This enables the LMF to distinguish between:

- UE-internal issues (e.g., low battery, processing limitations, etc.), and

- externally resolvable issues by LMF, i.e., incorrect or mismatched assistance data, which may be resolved through reconfiguration by LMF.

In addition to incorrect or mismatched assistance data, other types of externally resolvable issues may also arise as AI/ML positioning matures. While this proposal focuses on assistance data inconsistencies, additional error causes may be considered in the future as new types of recoverable input-related failures are identified (e.g., inference parameter misconfiguration, incompatible model context, etc.).

Future extensions may consider additional error causes, such as modelVersionNotSupported or modelTemporarilyUnavailable, to support model version management via Model-ID-based LCM, if required in future.

Proposal 2.	Use of dedicated error cause “Assistance data incompatible with model” for externally resolvable issues, i.e., incorrect or mismatched assistance data.

As presented in [1], the condition thereWereNotEnoughSignalsReceivedForUeBasedAI-ML is analogous to the existing thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA. Since the current AI/ML positioning (Case 1) operates based on DL-TDOA measurements, such cases are already covered by existing DL-TDOA error causes and do not require the introduction of new error fields.

Proposal 3.	Do not introduce a new error cause for conditions already covered by existing DL-TDOA error causes since current AI/ML positioning (Case 1) operates based on DL-TDOA measurements.

As a result, we recommend adding the proposed two error causes under existing NR-DL-TDOA-TargetDeviceErrorCauses-r16 structures.

Proposal 4.	Add “AI/ML functionality not applicable (aimlFunctionalityNotApplicable)” and “Assistance data incompatible with model (assistanceDataIncompatibleWithModel)” to existing NR-DL-TDOA-TargetDeviceErrorCauses-r16 structure.

-- ASN1START NR-DL-AI-ML-TargetDeviceErrorCauses-r19 ::= SEQUENCE { cause-r19 ENUMERATED { undefined, assistance-data-missing, unableToMeasureAnyTRP, attemptedButUnableToMeasureSomeNeighbourTRPs, thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA, locationCalculationAssistanceDataMissing, aimlFunctionalityNotApplicable, assistanceDataIncompatibleWithModel, ... }, ..., } -- ASN1STOP

We do not propose adding any error causes to NR-DL-AI-ML-LocationServerErrorCauses.

Since AI/ML positioning Case 1 is based on DL-TDOA measurements and the inference is performed on the UE side, we consider that the existing NR-DL-TDOA-LocationServerErrorCauses structure can be reused for any applicable location server-side errors. Therefore, no new location server error causes are proposed for AI/ML positioning.

Proposal 5.	Reuse the existing NR-DL-TDOA-LocationServerErrorCauses structure for AI/ML positioning Case 1, and do not introduce additional error causes in NR-DL-AI-ML-LocationServerErrorCauses.

-- ASN1START NR-DL-AI-ML-LocationServerErrorCauses-r19 ::= SEQUENCE { cause-r19 ENUMERATED { undefined, assistanceDataNotSupportedByServer, assistanceDataSupportedButCurrentlyNotAvailableByServer, notProvidedAssistanceDataNotSupportedByServer, ..., on-demand-dl-prs-NotSupportedByServer-v1700, on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-v1700 }, ... } -- ASN1STOP

# 3		Conclusion

The following observations are based on the current specification.

Observation 1.	Legacy (non-AI/ML) positioning error causes can provide useful guidance regarding appropriate next actions.

Based on previous meeting agreements, we observe the following.

Observation 2.	AI/ML positioning error causes should enable fallback to a legacy (non-AI/ML) positioning method when AI/ML positioning is not applicable.

We propose adding two error causes to existing NR-DL-TDOA-TargetDeviceErrorCauses-r16 structures.

Proposal 1.	Use of abstracted error cause “AI/ML functionality not applicable” for UE-side limitations in AI/ML positioning.

Proposal 2.	Use of dedicated error cause “Assistance data incompatible with model” for externally resolvable issues, i.e., incorrect or mismatched assistance data.

Proposal 3.	Do not introduce a new error cause for conditions already covered by existing DL-TDOA error causes since current AI/ML positioning (Case 1) operates based on DL-TDOA measurements.

Proposal 4.	Add “AI/ML functionality not applicable (aimlFunctionalityNotApplicable)” and “Assistance data incompatible with model (assistanceDataIncompatibleWithModel)” to existing NR-DL-TDOA-TargetDeviceErrorCauses-r16 structure.

-- ASN1START NR-DL-AI-ML-TargetDeviceErrorCauses-r19 ::= SEQUENCE { cause-r19 ENUMERATED { undefined, assistance-data-missing, unableToMeasureAnyTRP, attemptedButUnableToMeasureSomeNeighbourTRPs, thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA, locationCalculationAssistanceDataMissing, aimlFunctionalityNotApplicable, assistanceDataIncompatibleWithModel, ... }, ..., } -- ASN1STOP

We do not propose adding any error causes to NR-DL-AI-ML-LocationServerErrorCauses.

Proposal 5.	Reuse the existing NR-DL-TDOA-LocationServerErrorCauses structure for AI/ML positioning Case 1, and do not introduce additional error causes in NR-DL-AI-ML-LocationServerErrorCauses.

-- ASN1START NR-DL-AI-ML-LocationServerErrorCauses-r19 ::= SEQUENCE { cause-r19 ENUMERATED { undefined, assistanceDataNotSupportedByServer, assistanceDataSupportedButCurrentlyNotAvailableByServer, notProvidedAssistanceDataNotSupportedByServer, ..., on-demand-dl-prs-NotSupportedByServer-v1700, on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-v1700 }, ... } -- ASN1STOP

# 4		References

- 3GPP R2-2500968

- 3GPP R2-2501702

- 3GPP [POST129bis][015][AI PHY] 37.355 Running CR (Qualcomm)