3GPP TSG-RAN WG2 #130                                             	    R2-2503803

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	   8.6.4

Source: 		Qualcomm Incorporated

Title: 	Discussion on conditional intra-CU LTM

Document for:	Discussion

# 1	Introduction

RAN2#129b agreed the following regarding conditional LTM [1]:

Agreements on MAC issues: - Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA Agreements on C-LTM: - Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running. - C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer. - RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE. - For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

This paper discusses further aspects related to intra-CU conditional LTM.

# 2	PTAG TA timer handling upon RACH-less CLTM

In RAN2#129b, the following agreement was made for the CLTM TA timer handling upon CLTM execution.

C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

Since the CLTM execution timing is sorely based on the configured execution condition for the target cell, it is possible that the CLTM execution timing is close to the CLTM TA timer expiry for the selected CLTM target cell. In this situation, the target cell may not have enough time to send the TA command MAC CE to UE, hence UE may end up going through the TA timer expiry procedure which includes HARQ buffer flush, PUCCH/SRS resource release, and DL/UL resource assignment/grant clear, unnecessarily. This could significantly diminish the cell switch latency gain for RACH-less CLTM. Therefore, it would be beneficial to guarantee the minimum amount of PTAG TA timer so that the target cell has enough time to send TA command MAC CE to RACH-less CLTM UE. The following options can be considered when TA timer for PTAG carries over the remaining CLTM TA value:

- Option 1: Extend TA timer for PTAG by a timer offset

  - This timer offset value can be configured by RRC

  - FFS whether this timer offset is applied only when TA timer for PTAG is below threshold.

- Option 2: Determine TA timer for PTAG as max(TA timer for PTAG, minimum TA timer value)

  - The minimum TA timer value can be configured by RRC.

  - This guarantees TA timer for PTAG would be at least minimum TA timer value.

Figure 1a. [current MAC spec] PTAG TA timer from remaining CLTM TA timer value being too short, which leads to PTAG TA timer expiry even after successful RACH-less CLTM

Figure 1b. [Proposal] PTAG TA timer from remaining CLTM TA timer value being extended, which allows the target cell to send TA command MAC CE

Observation 1: Due to the independent operations between CLTM TA timer counting and CLTM execution condition evaluation, UE could run into a situation where PTAG TA timer, which is based on the remaining CLTM TA timer value, expires before UE receives TA command MAC CE from the target cell. And the recovery procedure upon TA expiry would diminish the latency improvement by RACH-less CLTM operation.

Proposal 1: RAN2 to discuss how to avoid unnecessary TA timer expiry upon RACH-less CLTM when the remaining CLTM TA timer value is not long enough for the target cell to send TA command MAC CE and to adopt one of options below.

- Option 1: Extend TA timer for PTAG by a timer offset

  - This timer offset value can be configured by RRC

  - FFS whether this timer offset is applied only when TA timer for PTAG is below threshold.

- Option 2: Determine TA timer for PTAG as max(TA timer for PTAG, minimum TA timer value)

  - The minimum TA timer value can be configured by RRC.

# 3	RACH-less CLTM recovery

In RAN2#129, it was agreed to reuse Rel-18 LTM failure recovery mechanism, i.e., CBRA-based LTM switch procedure. While Rel-18 CBRA-based LTM fast recovery can be naturally supported for CLTM, the possibility of utilizing CG resources for RACH-less CLTM fast recovery should be considered.

Rel-18 LTM fast recovery had to be RACH-based LTM switch because only the target candidate cell in LTM CSC MAC CE activates the reception of CG resources and all the other candidate cells wouldn’t know about LTM switch of UE.

In Rel-19 CLTM, however, the serving cell would have to inform several candidate cells (which UE may try CLTM switch to) of activating CG resource receptions because it doesn’t know which candidate cell will be chosen by UE. As also agreed in RAN2#129, UE would try CG-based RACH-less CLTM when at least one candidate beam fulfils the event condition and UE has a valid TA (either through TA MAC CE or UE-measured TA). Therefore, it is possible that one or more candidate cells still monitor CG resources when UE tries CLTM fast recovery, in which case utilizing the CG resource could further reduce the recovery latency compared to RACH-based CLTM fast recovery.

Observation 2: Unlike Rel-18 LTM, there could be more than one candidate cell that are indicated by serving cell to monitor CG resources for potential CLTM execution by UE and they have to keep monitoring CG resources until they receive UL signal or another indication from original or new serving cell. Then it would be beneficial to utilize those CG resources for RACH-less CLTM fast recovery.

Proposal 2: RAN2 agree to support RACH-less CLTM recovery for Rel-19 intra-CU CLTM when the following conditions are met:

- The selected cell is one of CLTM candidate cell AND

- The selected CLTM candidate cell has configured CG resource AND

- UE has a valid TA for the selected CLTM candidate cell by either (i) TA from TA MAC CE for which TA timer is running or (ii) UE-measured TA.

# 4	Beam selection for L3-based CLTM

In the MAC running CR email discussion, there is an FFS on the beam selection for L3 event-based CLTM execution.

Editor’s NOTE: FFS how to determine the selected beam in case the CLTM is triggered by L3 event.

Since L3 event evaluation for CLTM has been agreed to be performed in RRC and L3 event condition(s) reuse CHO events (i.e., CondEvent3 and CondEvent5), the procedure for CHO can be considered as a baseline. In conditional reconfiguration execution, the SSB selection (e.g., for RACH) is up to UE implementation after the configured event condition(s) is evaluated to be fulfilled, hence the same procedure can be followed for CLTM. Since MAC performs the CLTM execution, RRC would need to send the selected candidate cell ID and the SSB index to MAC.

Observation 3: For the beam selection in L3-based CLTM, CHO procedure can be used a baseline.

Proposal 3: After detecting L3 event condition(s) fulfilled, the SSB selection for the target candidate cell is up to UE implementation, and RRC informs MAC of the target candidate cell ID and the SSB index.

# 4	Early CLTM target cell notification for RACH-less CLTM

When it comes to CG-based first UL transmission on target cell for RACH-less CLTM, due to the unknown CLTM cell switch timing to both serving cell and multiple candidate cells, CG time/frequency resources would have to be reserved earlier than actual CLTM timing and most of them would be left unused, i.e., those CG resources cannot be used for other PUSCH allocations due to the potential collision with CLTM UE’s first UL transmission. As more potential target candidate cells configure such CG resources, the overall resource inefficiency could become even more severe.

Therefore, it would be beneficial to introduce a new signaling procedure (“early target cell notification”) for UE to notify the potential target cell(s) and beam(s) for CLTM to the serving cell such that the candidate cells can start monitoring CG resources only for a certain time period after receiving the notification message from the serving cell. This procedure would allow other candidate cells (which were not selected) to reuse the configured CG resources for other PUSCH allocations without any collision issue.

Observation 4: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 4: For RACH-less CLTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target CLTM cell(s) and beam(s).

# 5	Early TA signalling and maintenance

## 5.1	Candidate cell TA group

For the successful RACH-less CLTM, it is necessary for UE to have a valid TA value for the target candidate cell. Through measurement reports, serving cell would be able to roughly know which candidate cells are potentially target candidate cells for CLTM. However, if the number of those candidate cells is not small enough, these multiple PDCCH ordered RACH procedures would become overhead such that DL and/or UL performance would degrade due to interruptions.

To alleviate the PDCCH ordered RACH overhead issue, the candidate cell TA group can be considered where UE can assume an acquired TA value from one candidate cell (either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells within the TA group. By adopting this TA group, serving cell can trigger only a few PDCCH ordered RACH procedures for all candidate cells within the TA group, hence the RACH overhead could be significantly reduced.

Observation 5: For successful RACH-less CLTM, serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could become huge overhead and interruptions in data transmission and reception at UE.

Proposal 5: RAN2 to support the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

## 5.2	TA validation procedure

In RAN2#129, it was agreed the TA timer-based TA validation procedure, i.e., if CLTM TA timer starts/re-starts when UE receives TA value for the candidate cell via MAC CE, and as long as the TA timer associated with the candidate cell is running UE considers the TA for the candidate cell is valid.

Since the purpose of TA adjustment in uplink transmissions is to compensate for the additional propagation delay, the TA validation procedure should be able to track the propagation delay change, which can be also identified by RSRP change (reflecting a pathloss change).

The TA validation procedure at UE has been discussed in other topics before such as CG-SDT transmission or SRS transmission for positioning, and the existing validation procedure can be used as a baseline. According to Section 5.72.2 in TS 38.321 MAC specification, the following procedure is defined for the TA validation for CG-SDT.

The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and 1> cg-SDT-TimeAlignmentTimer is running

From above three conditions, the second (RSRP change threshold) and the third (TA timer) would be relevant for CLTM. The first condition (valid RSRP) is to cover UE in RRC_INACTIVE state, so it is not necessary for CLTM.

In addition to the TA timer that has been already agreed, checking the RSRP change (L1 or L3) can be used to track UE’s pathloss change which impacts the propagation latency.

Observation 6: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in CLTM execution.

Proposal 6: RAN2 to agree the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

# Conclusion

This contribution discussed aspects related to conditional intra-gNB LTM. The following observations and proposals have been made:

Observation 1: Due to the independent operations between CLTM TA timer counting and CLTM execution condition evaluation, UE could run into a situation where PTAG TA timer, which is based on the remaining CLTM TA timer value, expires before UE receives TA command MAC CE from the target cell. And the recovery procedure upon TA expiry would diminish the latency improvement by RACH-less CLTM operation.

Proposal 1: RAN2 to discuss how to avoid unnecessary TA timer expiry upon RACH-less CLTM when the remaining CLTM TA timer value is not long enough for the target cell to send TA command MAC CE and to adopt one of options below.

- Option 1: Extend TA timer for PTAG by a timer offset

  - This timer offset value can be configured by RRC

  - FFS whether this timer offset is applied only when TA timer for PTAG is below threshold.

- Option 2: Determine TA timer for PTAG as max(TA timer for PTAG, minimum TA timer value)

  - The minimum TA timer value can be configured by RRC.

Observation 2: Unlike Rel-18 LTM, there could be more than one candidate cell that are indicated by serving cell to monitor CG resources for potential CLTM execution by UE and they have to keep monitoring CG resources until they receive UL signal or another indication from original or new serving cell. Then it would be beneficial to utilize those CG resources for RACH-less CLTM fast recovery.

Proposal 2: RAN2 agree to support RACH-less CLTM recovery for Rel-19 intra-CU CLTM when the following conditions are met:

- The selected cell is one of CLTM candidate cell AND

- The selected CLTM candidate cell has configured CG resource AND

- UE has a valid TA for the selected CLTM candidate cell by either (i) TA from TA MAC CE for which TA timer is running or (ii) UE-measured TA.

Observation 3: For the beam selection in L3-based CLTM, CHO procedure can be used a baseline.

Proposal 3: After detecting L3 event condition(s) fulfilled, the SSB selection for the target candidate cell is up to UE implementation, and RRC informs MAC of the target candidate cell ID and the SSB index.

Observation 4: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 4: For RACH-less CLTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target CLTM cell(s) and beam(s).

Observation 5: For successful RACH-less CLTM, serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could become huge overhead and interruptions in data transmission and reception at UE.

Proposal 5: RAN2 to support the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

Observation 6: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in CLTM execution.

Proposal 6: RAN2 to agree the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

# References

- Chairman Notes; TSG RAN WG2 Meeting #129b