3GPP TSG-RAN WG2 #130                                             	    R2-2503804

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	   8.6.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on inter-CU LTM

Document for:	Discussion

# 1	Introduction

RAN2#129b has agreed on the following for inter-CU LTM [1]:

Agreements (Organizational): - A list of sk-counters is linked to a Rel-19 ID configured by the SN. - At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed. - How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration). - The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3. - Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase. Agreements on inter-CU LTM: - For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE. - NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included. - NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG. - From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4. - MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement. - If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA. - An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

This paper discusses further aspects related to inter-CU LTM failure recovery.

# 2	Inter-CU LTM failure recovery with different Rel-19 ID

In RAN2#129b, the following agreements were made for RLF and LTM failure recovery:

- At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

To discuss the feasibility of the fast recovery to a candidate cell with different Rel-19 IDs from both serving cell and target cell, the agreement from the last RAN2#129 meeting on the NCC delivery in inter-CU LTM needs to be recalled.

- Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase. - For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

Since the NCC value for inter-CU LTM cell switch is delivered via the new Rel-19 LTM cell switch command MAC CE, UE would have valid security parameters only for target cell. Therefore, any selected candidate cell whose Rel-19 ID is different from target cell wouldn’t be eligible for LTM recovery due to the lack of the NCC value.

Observation 1: Due to the NCC delivery via the new Rel-19 LTM CSC MAC CE for inter-CU LTM, UE cannot have the valid security parameter for candidate cells whose Rel-19 ID is different from target cell during subsequent inter-CU LTMs. Therefore, LTM recovery towards those candidate cells won’t be possible.

Proposal 1: RAN2 agree to NOT support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

# 3	LTM failure recovery procedure

During RRC running CR email discussion, the following editor’s note has been made w.r.t. the configuration UE needs to revert back to upon T304 expiry and the corresponding RRC re-establishment.

Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration.

In Rel-18 LTM, due to its intra-CU nature, reverting UE configuration back to source PCell would suffice for both LTM recovery and RRC re-establishment. However, as per the last RAN2 meeting agreement copied below,

if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS  if fast failure recovery with different Rel-19 IDs is allowed.

the inter-CU LTM failure can lead to one of the following cases:

- LTM recovery if (i) the selected cell is one of the candidate cells AND (ii) it has the same Rel-19 ID as target cell.

- RRC re-establishment if (i) the selected cell is not one of the candidate cells OR (ii) the selected cell is one of the candidate cells but it has the different Rel-19 ID as target cell.

The problem here is that when T304 expires the cell selection is not yet performed, therefore which configuration UE reverts back to cannot be determined. This should be decided during the RRC re-establishment procedure.

Observation 2: Unlike Rel-18 intra-CU LTM where reverting back to source cell configuration suffices upon T304 expiry, Rel-19 inter-CU LTM has dependency for reverting UE configuration w.r.t. the selected cell being one of candidate cells and its Rel-19 ID.

Then, the simplest option is to perform re-establishment when the UE cannot perform LTM recovery.

Proposal 2: RAN2 decide the following for inter-CU LTM failure (upon T304 expiry and while T311 running):

    - If the selected cell is one of the candidate cells and it has the same Rel-19 ID as target cell, then UE performs LTM recovery.

    - Else, UE reverts back to the source cell configuration and performs RRC re-establishment.

# 3	Conclusion

This contribution discussed aspects related to inter-CU LTM. The following observations and proposals have been made:

Observation 1: Due to the NCC delivery via the new Rel-19 LTM CSC MAC CE for inter-CU LTM, UE cannot have the valid security parameter for candidate cells whose Rel-19 ID is different from target cell during subsequent inter-CU LTMs. Therefore, LTM recovery towards those candidate cells won’t be possible.

Proposal 1: RAN2 agree to NOT support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell.

Observation 2: Unlike Rel-18 intra-CU LTM where reverting back to source cell configuration suffices upon T304 expiry, Rel-19 inter-CU LTM has dependency for reverting UE configuration w.r.t. the selected cell being one of candidate cells and its Rel-19 ID.

Proposal 2: RAN2 decide the following for inter-CU LTM failure (upon T304 expiry and while T311 running):

    - If the selected cell is one of the candidate cells and it has the same Rel-19 ID as target cell, then UE performs LTM recovery.

    - Else, UE reverts back to the source cell configuration and performs RRC re-establishment.

# References

- Chairman Notes; TSG RAN WG2 Meeting #129b