3GPP TSG-RAN2#130 	R2-2503820

St. Julian’s, Malta, 19th May – 23th May, 2025

Agenda item:		8.12.2 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Consideration on Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

1.	Introduction

In the last meeting, RAN2 discussed issues related to asymmetric DL sTRP/UL mTRP deployment and made some agreements.

No need to add a maximum number restriction of the TCI states indicated by the PL offset MAC CE. RAN2 understand the PL offset update MAC CE is at least applicable to PUCCH, PUSCH, SRS, and PDCCH-order CFRA. We will capture in a note to reflect the previous understanding ‘RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset.’. FFS on exact wording. From RAN2 point of view, UE applies the latest PL offset value received in RRC or MAC CE. For 2TA in asymmetric DL sTRP/UL mTRP scenario with pathloss offset configured Rel-18 2TA operation is applied with the following RRC changes: - • remove the restriction that RRC field tag2 is configured only if coresetPoolIndex is configured with more than one value; - • a single n-TimingAdvanceoffset is configured, i.e., n-TimingAdvanceOffset2 is not configured for 2TA in asymmetric DL sTRP/UL mTRP scenario. For PRACH transmission, PL offset is applicable only to PDCCH-order CFRA.

During post e-mail discussion [1], some issues and MAC running CR were discussed, but some issues are remained.

In this contribution, we discuss remaining issues of asymmetric DL sTRP/UL mTRP deployment.

2.	Discussion

During post e-mail discussion [1], two issues related to asymmetric DL sTRP/UL mTRP deployment were discussed, and one of issue is whether to apply Rel-19 2TA (sDCI mTRP) for LTM cell switch.

Currently, Rel-18 2TA (mDCI mTRP) is supported for LTM cell switch. LTM Cell Switch Command MAC CE includes TAC and TCI-state ID, and the TAC is applied to the TAG which is associated with the TCI state ID. Thus, during [1], some companies argued that Rel-19 2TA (sDCI mTRP) can be applied to LTM cell switch directly and this can be determined in MIMO session.

However, in our view, whether to support LTM cell switch with Rel-19 2TA should be determined in LTM session, not in MIMO session.

Firstly, it is not clear whether PL offset introduced in Rel-19 MIMO can be configured for the candidate cell in advance before LTM cell switch. If PL offset can be configured with TCI-state ID for the candidate cell, the UE may apply the PL offset associated with TCI-state ID in LTM Cell Switch Command MAC CE for the first PUSCH transmission. However, there is no agreement on whether/how to configure PL offset with TCI-state ID for the candidate cell and we think that this should be discussed in LTM session.

Secondly, it is not clear how to work LTM cell switch if PL offset is not configured. For asymmetric DL sTRP/UL mTRP, there is a scenario where PL offset is not configured, and the UE may expected to receive SSB from UL-only TRP. However, in LTM session, there is no discussion how LTM cell switch works in asymmetric DL sTRP/UL mTRP deployment with no PL offset.

Lastly, asymmetric DL sTRP/UL mTRP is scoped as intra-DU, but LTM cell switch is scoped as inter-DU. Thus, how to restrict the scenario considering asymmetric DL sTRP/UL mTRP and LTM cell switch should be discussed, and we think this also should be discussed in LTM session.

Therefore, we propose to discuss the inter-operability with Rel-19 2TA and LTM cell switch in LTM session. Note that the inter-operability with Rel-19 2TA and LTM cell switch is already discussed in LTM session, and session chair mentioned that the inter-operability will be discussed later based on RAN1 progress.

Proposal 1. The inter-operability with Rel-19 2TA and LTM cell switch is discussed in LTM session, not MIMO session.

In addition, whether/how to apply Rel-18 Unified TCI States A/D MAC CE for asymmetric DL sTRP/UL mTRP deployment was also discussed in [1].

In RAN1#117, RAN1 concluded to reuse Rel-17 unified TCI/ICBM and Rel-18 unified TCI framework for asymmetric DL sTRP/UL mTRP deployment.

Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In our understanding, if Rel-17 Unified TCI/ICBM is configured, Rel-17 Unified TCI States A/D MAC CE can be reused directly for asymmetric DL sTRP/UL mTRP deployment. The Rel-17 Unified TCI States A/D MAC CE supports one TCI state for joint mode and {one DL TCI state + one UL TCI state} for separate mode, so no enhancement is required and there was no concern from other companies.

However, for the case where Rel-18 unified TCI is configured, some companies argued to introduce new MAC CE or to send LS to ask whether enhancement is needed.

In our view, the cases in RAN1 agreement can be covered by Rel-18 MAC CE. For joint mode, the network can choose to enable only one TCI state or two TCI state by setting Fi, j field. For separate mode, the network can also choose to enable only one DL/UL TCI state or two DL/UL TCI state by setting Fi, j field. Thus, we don’t see no new MAC CE or any enhancement is needed.

Proposal 2. Reuse Rel-18 Unified TCI States A/D MAC CE for asymmetric DL sTRP/UL mTRP deployment.

3.	Conclusion

In this document, we discuss RAN2 impacts from asymmetric DL sTRP/UL mTRP, and made proposals as follows.

Proposal 1. The inter-operability with Rel-19 2TA and LTM cell switch is discussed in LTM session, not MIMO session.

Proposal 2. Reuse Rel-18 Unified TCI States A/D MAC CE for asymmetric DL sTRP/UL mTRP deployment.

4.	Reference

- [Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)

- RP-242394	WID revision: NR MIMO Phase 5	Samsung (Moderator)