3GPP TSG-RAN WG2 Meeting #130			     R2-2503830

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.5 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining open issues on RLC enhancements for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses remaining open issues related to RLC enhancements.

Discussion

Avoid unnecessary RLC retransmissions

Following open issues are identified for unnecessary RLC retransmission avoidance during the RLC running CR discussion.

For the RLC-6, the issue is that when the transmitting entity receives a STATUS report, even though a SDU is indicated by negative acknowledgement in the STATUS report, if stopReTxDiscardedSDU is configured, the SDU is not considered for retransmission and RETX_COUNT for the SDU doesn’t increase because the SDU is already indicated by a discard indication from PDCP. In this situation, RETX_COUNT for the SDU indicated by the discard indication may not be met with the maxRetxThreshold.

To handle this problem, some companies may think that even though the SDU is not considered for retransmission due to a discard indication from PDCP, RETX_COUNT for the SDU is incremented by negative acknowledgement in the STATUS report. We understand that this may occur when stopReTxDiscardedSDU is configured. However, incrementing RETX_COUNT without performing retransmission would be undesirable because RETX_COUNT can be met to the max threshold without further actual retransmissions. We also think that this issue would not be a critical problem since the RLC maximum retransmission is not a sole mechanism to detect RLF in the UE and there are other mechanisms to detect RLF which may detect the RLF properly before reaching the RLC maximum retransmission. Thus, further enhancement is not needed and it is preferred not to change current UE behaviour for detecting RLF based on the maximum RLC retransmission.

Proposal 1. RETX_COUNT handling is not changed even when avoiding unnecessary retransmission is introduced, i.e., if the SDU is not considered for retransmission due to a discard indication from PDCP, RETX_COUNT for the SDU is NOT incremented by negative acknowledgement as the current RLC running CR.

For the RLC-10, the issue is that when the stopReTxObsoleteSDU is configured at the transmitting entity but the t-RxDiscard is not configured at the peer receiving entity, the transmitting window stalling problem may not be resolved properly. To handle this problem, the proponent suggests to move the transmitting window forward based on the discard indication. We have sympathy on the issue if it is possible to configure stopReTxObsoleteSDU at the transmitting entity without configuring t-RxDiscard at the peer receiving entity. However, if the network can ensure that the transmitting entity is configured with stopReTxObsoleteSDU and the peer receiving entity is configured with the t-RxDiscard, there would be no problem, i.e., the network implementation can handle this. Having said that, if companies still have strong concern on it, one possible standard solution would have a configuration guidance in the RRC specification.

Proposal 2. Add RRC configuration guidance for the stopReTxObsoleteSDU and t-RxDiscard, i.e., if stopReTxObsoleteSDU is configured, the t-RxDiscard is configured at the peer receiving entity.

For the RLC-11, the issue is that when t-PollRetransmit timer expires, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with due to green highlight below. In our view, if all SDUs buffered whose transmissions have been stopped due to discard indication from PDCP after t-PollRetransmit timer expires, PDB of outdated SDUs is already passed and it may be useless for an application. This means that there is no need to retransmit the outdated SDU even after t-PollRetransmit timer expires and delaying the poll retransmission until a new SDU is received from the PDCP would be no problem. In addition, according to the current running RLC CR in the below box, if nothing is selected for retransmission after expiry of t-PollRetransmit due to green highlight below, the RLC include a poll in an AMD PDU as described in clause 5.3.3.2 as yellow highlight below. It means that if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP after expiry of t-PollRetransmit, an ongoing polling procedure would be stopped here since there is no more SDU to transmit and the RLC cannot check polling condition until another new SDU is received from upper layers. Therefore, we don’t think that additional condition to stop ongoing polling procedure and further enhancement on this part are needed.

#### 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured); or - consider any RLC SDU which has not been positively acknowledged for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured). - include a poll in an AMD PDU as described in clause 5.3.3.2.

Proposal 3. No enhancement for poll retransmission after t-PollRetransmit timer expires, i.e., when t-PollRetransmit timer expires, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, no SDU to retransmit the poll is selected as in the current RLC running CR.

Timely RLC retransmissions

Following open issues are identified for timely RLC retransmissions during the RLC running CR discussion.

For the RLC-7, when the retransmission is indicated by a STATUS PDU, the RLC entity increments RETX_COUNT value per RLC SDU. If RETX_COUNT value reaches the configured maximum retransmission threshold, the RLC entity indicates to RRC that max retransmission has been reached and then RLF would be declared by RRC. Considering that remaining-time based RLC retransmission is performed to satisfy tight latency requirement, if remaining-time based RLC retransmission increments RETX_COUNT and maximum retransmission threshold is reached, RLF would cause RRC connection re-establishment and this requires long interruption which makes remaining-time based RLC retransmission useless. In addition, RAN2 agreed that “Only a single autonomous retransmission will be triggered per RLC SDU.”, and skipping one time of RETX_COUNT increment by remaining-time based RLC retransmission would not be a big problem. Thus, we prefer to keep the current mechanism and RETX_COUNT increment by remaining-time based RLC retransmission is not needed. Note that RETX_COUNT is not only way to detect RLF and there are other mechanisms to detect RLF in a UE.

Proposal 4. Remaining-time based RLC retransmission does not increment RETX_COUNT, but a simple text is added in the RLC specification as in the following TP.

When an RLC SDU or an RLC SDU segment is considered for retransmission by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

For the RLC-8 and RLC-9, in LTE, concatenation is performed at RLC and only one AMD PDU including one poll bit is submitted for one transmission opportunity. In NR, however, concatenation is performed at MAC and the RLC entity can submit multiple AMD PDUs for one transmission opportunity. In this condition, if pollPDU or pollByte is configured with a lower value, it is already possible to submit multiple AMD PDUs including poll for one transmission opportunity. This means that multiple polling can be already transmitted for one transmission opportunity and there is no problem so far since even if status reporting was triggered several times before receiving the first transmission opportunity indicated by lower layer, one STATUS PDU is constructed and transmitted. In this sense, we think that multiple polling is already possible and it is not an issue. We also don’t see the problem that too early and/or unnecessary retransmission is performed due to polling based on remaining time. Thus, no special handling to avoid multiple polling and too early/unnecessary retransmission is needed.

Observation 1. In 5G NR, multiple polling is already transmitted for one transmission opportunity and it is not a problem so far.

Proposal 5. Even if polling is included based on the remaining time, no special handling to avoid multiple polling and too early/unnecessary retransmission is introduced.

One additional open issue is identified during the PDCP running CR discussion.

- Issue 2: It is FFS for which PDCP SDU the UE shall trigger remaining-time-based RLC retransmission and polling, if pdu-SetDiscard is configured.

The issue is that when pdu-SetDiscard is configured, if remaining time of a SDU in a PDU set becomes equal to or less than the threshold, whether remaining-time based RLC retransmission and polling should be triggered for all SDUs in the PDU set or not. Two options can be considered for this open issue:

- Option 1: Even though pdu-SetDiscard is configured, remaining-time based RLC retransmission and polling are triggered for one SDU, not all SDUs in the PDU set;

- Option 2: If pdu-SetDiscard is configured, remaining-time based RLC retransmission and polling are triggered for all SDUs in the PDU set.

The option 1 is easy and simple to control remaining-time based RLC retransmission, but it is unlikely to satisfy the PDU set delay budget (PSDB) and the option 1 would cause unnecessary radio resource wastes since one of SDUs in the PDU set may not be retransmitted within the PSDB of the PDU set and already transmitted SDUs in the PDU set may be useless.

The option 2 needs more retransmission than the option 1, but this option is likely to satisfy the PDU set latency requirement because SDUs in the PDU set can be retransmitted within the PSDB of the PDU set. in addition, considering RAN2 agreements and RLC running CR, even though the PDCP indicates to lower layers that the condition for remaining-time based RLC retransmission is met for all SDUs in the PDU set, only RLC SDUs which satisfy the following conditions can be actually retransmitted.

- the RLC SDU or the RLC SDU segment has been submitted to lower layers; and

- it has not been positively acknowledged; and

- the RLC SDU or the RLC SDU segment is not pending for retransmission.

This means that if a RLC SDU is not yet submitted to lower layers or is pending for retransmission or is already positively acknowledged, the RLC ignores an indication from PDCP and doesn’t perform remaining-time based RLC retransmission for this RLC SDU. Therefore, we think that the option 2 would be desirable for remaining-time based RLC retransmission when pdu-SetDiscard is configured in normal network condition. However, if the network is congested, we are skeptical of retransmitting all SDUs in the PDU set because this makes network congestion even worse and anyway satisfying PSDB may not be possible in this situation. It would be reasonable that if PSI based SDU discard is activated, remaining-time based RLC retransmission is not triggered for all SDUs in the PDU set which can be controlled by PDCP.

For the remaining-time based polling, as explained above for the proposal 5, multiple polling is not a problem and either option is ok. Thus, it is preferable to have a unified behaviour and the option 2 would be sufficient for the remaining-time based polling.

Proposal 6. If pdu-SetDiscard is configured, remaining-time based RLC retransmission and polling are triggered for all SDUs in the PDU set.

Proposal 7. If PSI based SDU discard is activated, remaining-time based RLC retransmission is not triggered.

Rel-18 leftovers

For the RLC-12, the issue is that SN gap report is lost during handover and the t-Reordering starts unnecessarily at the target RAN node as explained in [1-2]. In RAN2#129, it was concluded that RAN2 doesn’t see a big problem to solve for Rel-18, but discuss whether this problem needs to be solved in Rel-19 XR. In our view, it is still doubtful whether this new triggering condition for a PDCP SN gap report is needed since this is not a critical issue and there is only one t-reordering delay without this enhancement. In addition, as explained below, one proposed solution causes unnecessary PDCP SN gap report retransmission at the target RAN node.

Proposal 8. No enhancement is needed for PDCP SN gap report handling during UE mobility.

That being said, if majority companies want to introduce this enhancement in Rel-19 XR, the following point should be considered to choose a solution for this issue.

As per the contributions on this issue in the last RAN2 meeting, there would be two options for this issue:

- Option 1: The transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers. [1]

- Option 2: UE triggers a PDCP SN gap report when it receives a status report in which an SDU indicated in a previous SN gap report is negatively acknowledged. [2]

In option 1, the UE needs to retransmit a PDCP SN gap report to the target RAN node during PDCP Re-establishment whenever no lower-layer confirmation of successful delivery has been received for the PDCP SN gap report. The problem is that the PDCP SN gap report is successfully received at the source RAN node but just the STATUS report for the PDCP SN gap report is not yet received. In this case, the PDCP SN gap report is not lost and it doesn’t need to retransmit the PDCP SN gap report, but the option 1 retransmits the PDCP SN gap report unnecessarily. Considering that the issue would be a corner case as addressed in [2], the option 1 seems too much. We don’t think that it is desirable to have the option 1 for this issue.

Observation 2. If The transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers, unnecessary PDCP SN gap report retransmission can occur.

The option 2, on the other hand, retransmits the PDCP SN gap report to the target RAN node only when the PDCP SN gap is actually lost and there would be no unnecessary PDCP SN gap report retransmission. Thus, if RAN2 try to solve this issue in Rel-19 XR, we prefer to have the option 2.

Proposal 9. If the proposal 8 is not agreeable, the UE retransmits a PDCP SN gap report when an SDU indicated in a previous SN gap report is negatively acknowledged in a PDCP status report.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. In 5G NR, multiple polling is already transmitted for one transmission opportunity and it is not a problem so far.

Observation 2. If The transmitting PDCP entity shall re-transmit any previously (prior to the PDCP entity re-establishment) transmitted PDCP SN Gap report(s) for which the successful delivery has not been confirmed by lower layers, unnecessary PDCP SN gap report retransmission can occur.

Proposal 1. RETX_COUNT handling is not changed even when avoiding unnecessary retransmission is introduced, i.e., if the SDU is not considered for retransmission due to a discard indication from PDCP, RETX_COUNT for the SDU is NOT incremented by negative acknowledgement as the current RLC running CR.

Proposal 2. Add RRC configuration guidance for the stopReTxObsoleteSDU and t-RxDiscard, i.e., if stopReTxObsoleteSDU is configured, the t-RxDiscard is configured at the peer receiving entity.

Proposal 3. No enhancement for poll retransmission after t-PollRetransmit timer expires, i.e., when t-PollRetransmit timer expires, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, no SDU to retransmit the poll is selected as in the current RLC running CR.

Proposal 4. Remaining-time based RLC retransmission does not increment RETX_COUNT, but a simple text is added in the RLC specification as in the following TP.

Proposal 5. Even if polling is included based on the remaining time, no special handling to avoid multiple polling and too early/unnecessary retransmission is introduced.

Proposal 6. If pdu-SetDiscard is configured, remaining-time based RLC retransmission and polling are triggered for all SDUs in the PDU set.

Proposal 7. If PSI based SDU discard is activated, remaining-time based RLC retransmission is not triggered.

Proposal 8. No enhancement is needed for PDCP SN gap report handling during UE mobility.

Proposal 9. If the proposal 8 is not agreeable, the UE retransmits a PDCP SN gap report when an SDU indicated in a previous SN gap report is negatively acknowledged in a PDCP status report.

References

[1] R2-2502277	RLC enhancements	Nokia, Nokia Shanghai Bell	discussion	Rel-19

[2] R2-2501764	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19