# 3GPP TSG RAN2 Meeging #130  	                                                                     R2-2503849

# Malta, Malta, 19th – 23rd May , 2025

Agenda item:	8.1.3

Title:	Discussion On the NW Side Data Collection RRC Framework

Source:	ZTE Corporation, Apple, MediaTek, Samsung, OPPO, Lenovo, Xiaomi, CMCC, China Telecom, vivo, NTT DOCOMO, Sanechips

Document for:	Discussion and Decision

Introduction

During the RAN2#129bis meeting, the RRC framework for NW-side data collection was preliminarily discussed in the context of AI/ML for PHY and AI/ML for mobility. A consensus was reached that the NW-side data collection framework can be configured for both AI/ML-based beam management and AI/ML-based mobility. This contribution further elaborates on a unified NW-side data collection framework.

Discussion

Analysis

In RAN2#129bis meeting, the following agreement for NW side data collection framework are achieved:

AI/ML based Beam Management

=> Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

AI/ML based Mobility:

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

The NW-side data collection RRC framework for beam management (BM) should be future-proof to accommodate additional use cases (e.g., AI mobility, AI CSI feedback).

Observation 1: According to RAN2#129b agreements in both AI PHY and AI mobility, a unified RRC framework for NW side data collection, which is forward compatible to accommodate future use cases (e.g. AI Mobility), is preferred in RAN2.

For designing the unified RRC framework of NW side data collection, we identify the following three options:

- Option 1a: Introduce a unified 	RRC framework with a same level of MeasConfig

- Option 1b: Introduce a unified cell level RRC framework

- Option 2: Extend the legacy L1 CSI framework (CSI-MeasConfig) with logging configuration [1]

Option 1a

In option 1a, inspired by RRM measurement framework, the general structure can be shown in Fig.1:

Fig.1: The general RRC structure for Option 1a [Note 1]

In above RRC structure, the cell group level NW side data collection (LoggedDataCollectionConfig) contains three key components in general:

- Measurement Resource Configuration (A list of BM-DataMeasConfig)

- References to L1 resource ID of existing measurement resource configurations (i.e., CSI-ResourceConfigId for BM)

- Data Logging Configuration (A list of BM-LoggingConfig)

- Defines logging type (periodic/event-triggered) and measurement quantity (loggingQuantity).

- Linkage Configuration (A list of DataCollectionId)

- Maps measurement resources to logging configurations.

Spec Impact

In option 1a, the spec impact can be generally summarized as in below:

- RAN2: Introduce A new CG level RRC framework for NW side data collection, including the measurement resource configuration, logging related configuration

- RAN3:

- Beam management use case: Potential F1 interface impact can be foreseen in gNB-split case if gNB split case is supported for NW side data collection, that is, CU-DU interaction for CU to obtain the Layer 1 measurement resources from DU.

- RAN1: No RAN1 spec impact is foreseen.

Potential spec work to extend to AI mobility

- RAN2: In option 1a, it can be easily extended to AI mobility by introducing a list of L3 Measurement Resource Configuration, where each one includes one L3 resource ID referencing to existing measurement resource configurations.

- RAN3: There is no obvious RAN3 impact as the measurement resources is just referring to the existed layer 3 measurement configuration.

Note 1: For the convenience, the ASN.1 structure for Fig 1 has been provided in the Annex.

Option 1b

In option 1b, inspired by CSI-MeasConfig, the RRC structure for cell level NW side data collection can be shown in Fig.2:

Fig.2: The general RRC structure for Option 1b [Note.2]

In above RRC structure, the cell group level NW side data collection (LoggedDataCollectionConfig) contains two key components in general:

- Measurement Resource Configuration (A list of DataMeasConfig)

  - Similar as option 1a

- Data logging related configuration (A list of LoggingConfig)

  - Similar as option 1a

Note: On the mapping from measurement resources to logging configurations, we follow the similar way of legacy L1 CSI framework to include DataMeasConfigId in LoggingConfig.

Spec Impact

In option 1b, the spec impact can be generally summarized as in below:

- RAN2: Introduce a new Cell level RRC framework for NW side data collection.

- RAN3:

- Beam management use case: potential F1 interface impact can be foreseen in gNB-split case if gNB split case is supported for NW side data collection, such as,CU-DU interaction regarding the logging related parameter is needed, including logging type(e.g. periodic, or event),logging quantity.

- RAN1: No RAN1 spec impact.

Potential spec work to extend to AI mobility

In option 1b, it can be easily extended to AI mobility via the following two specification changes:

- RAN2: similar to option 1a, introduce a list of L3 Measurement Resource Configuration, where each one includes one L3 resource ID referencing to existing measurement resource configurations (i.e., MeasObjectId for mobility).

- RAN3: if gNB split case is supported for NW side data collection, the CU-DU interaction is needed in order for DU to obtain L3 measurement related configuration for AI/ML based mobility.

Note 2: For the convenience, the ASN.1 structure for Fig 2 has been provided in the Annex.

Option 2

In option 2, the CSI framework is suggested to be extended for NW side data collection for beam management, the general RRC structure from contribution [1] can be shown in Fig.3:

Fig 3: The general RRC structure for option 2 [1]

In above RRC structure, the NW side data collection contains two parts in general:

- Measurement Resource Configuration (reuse existing CSI-resource config)

- For measurement resource configuration, as similar as option 1b, CSI-ResourceConfigId can be used to configure the measurement resource configuration for beam management.

- Data logging related configuration (A list of CSI Logged measurement config, similar to option 1a/1b)

- For data logging relate configuration, it shall generally contain the logging type (e.g. periodic, or event triggered), and loggingQuantity to define the logging measurement quantity.

Spec Impact

The spec impact from option 2 can be generally summarized as in below:

- RAN2:

- Extending CSI-MeasConfig with a logging related configuration.

- RAN3:

- Beam management use case: Potential F1 interface impact can be foreseen if gNB split case is supported for NW side data collection, such as,CU-DU interaction regarding the logging related parameter is needed, including logging type(e.g. periodic, or event),logging quantity..

- RAN1:

- Requires updates to capture the data logging procedures in RAN1 spec (TS 38.214).

- It is not clear whether the reporting procedure needs to be captured in in RAN1 spec (TS 38.214).

Potential spec work to extend to AI mobility

- RAN2:

- Not extensible. It seems that the only feasible way is to touch the base of L3 RRM RRC framework (i.e. MeasObjectNR) because it doesn’t make sense to extend L1 CSI framework (CSI-MeasConfig) to L3 measurement logging.

- RAN3:

- Potential CU-DU impact if gNB split case is supported for NW side data collection

    - CU-DU interaction regarding the layer 3 event related configuration is needed.

    - CU-DU interaction is needed for CU to transfer the L3 measurement related information to the DU in order for DU to generate the measurement resources configuration for AI/ML based mobility.

Summary

Above all, for understanding easily, the following table including the spec impact for all 3 options：

|  | RAN2 Spec Impact | Potential RAN3 Spec Impact(Note 1) | RAN1 Spec Impact | Potential spec impact for AI mobility |
| --- | --- | --- | --- | --- |
| Option 1a | New RRC framework with a same level MeasConfig | F1 interface impact - CU-DU interaction for CU to obtain the Layer 1 measurement resources for data collection from DU. | None. | RAN2: - Introduce a list of L3 Measurement Resource Configuration, where each one includes one L3 resource ID referencing to existing measurement resource configurations |
| Option 1b | New cell-level framework | F1 interface impact: - CU-DU interaction is needed for CU to retrieve the logged data upon the UE has been configured with data logging based data collection - CU-DU interaction is needed for DU to configure the L3 event related parameter for data logging. - CU-DU interaction is needed for CU to obtain the logged data configuration to check the validity of the received logged data reporting. | None | RAN2: - Same as Option 1a. RAN3: F1 interface impact - CU-DU interaction is needed for DU to obtain the L3 measurement resource configuration and related configuration. |
| Option 2 | Extend CSI-MeasConfig | F1 interface impact: - CU-DU interaction is needed for CU to retrieve the logged data upon the UE has been configured with data logging based data collection - CU-DU interaction is needed for DU to configure the L3 event related parameter for data logging. - CU-DU interaction is needed for CU to obtain the logged data configuration to check the validity of the received logged data reporting. | Requires updates to capture the data logging procedures in RAN1 spec (TS 38.214). FFS whether the reporting procedure needs to be captured in in RAN1 spec (TS 38.214). | RAN2: - Not extensible. It seems that the only feasible way is to touch the base of L3 RRM RRC framework (i.e. MeasObjectNR) because it doesn’t make sense to extend L1 CSI framework (CSI-MeasConfig) to L3 measurement logging. RAN3: F1 interface impact: - CU-DU interaction regarding the layer 3 event related configuration is needed. - CU-DU interaction regarding the Layer 3 measurement resources generation. |
| Note 1: It is up to RAN3 to decide whether to support NW side data collection in gNB-split case in rel-19. |  |  |  |  |

Compare among three options, Option 2 (i.e. extend Layer 1 measurement framework) causes the potential RAN1 spec impact, considering there is only one WG meeting left for RAN1 to terminate this WI, it is not suggested to heavier the workload to the RAN1 group.

Observation 2: Option 2 (i.e. extending L1 CSI framework) requires non-trivial spec changes on RAN1 (TS 38.214). However, RAN1 Rel-19 AI PHY will be closed in May meeting. Thus, option 2 will put a risk on successful completion of RAN2 Rel-19 AI PHY.

As for the option 1a and option 1b, For beam management case, option 1b causes more F1 interface impact on the data logging part, while option 1a causes more F1 interface impact on the measurement resource part. but for mobility case, there is no RAN 3 impact from option 1a, and potential RAN3 impact from option 1b is still existed. Which can be seen, the option 1a cause the least impact by considering both AI/ML beam management and AI/ML mobility. So it is proposed:

Proposal 1: The RRC framework with a same level of MeasConfig is adopt for NW side data collection.

If proposal 1 is agreeable, according to the general RRC framework structure as shown in Fig.1, we would like to propose the following:

Proposal 2: Introduce the following new RRC configurations for NW side data collection configuration for AI/ML based Beam management:

- Embed loggedDataCollectionConfig in RRCReconfiguration

- Include bm-DataMeasResourceConfig (measurement resources referring to legacy L1 resource CSI-ResourceConfigId), bm-DataLoggingConfig (logging parameters), and loggedDataCollectionLinkage (resource-logging mapping).

For understanding easily, please find the TP for proposal 2 in the Annex aspect.

If proposal 1 and proposal 2 is agreeable, the agreement shall be informed to RAN3 for them to consider whether there is any potential RAN3 impact. .

Proposal 3: Send an LS to inform RAN3 of RAN2 agreements regarding RRC framework of NW side data collection for AI/ML based beam management.

The draft LS to RAN3 can be found in Annex 2 for your convenience.

Conclusion

In this contribution, we provide our views on RRC framework for NW side data collection . We have the following observations and proposals:

Observation 1: According to RAN2#129b agreements in both AI PHY and AI mobility, a unified RRC framework for NW side data collection, which is forward compatible to accommodate future use cases (e.g. AI Mobility), is preferred in RAN2.

Observation 2: Option 2 (i.e. extending L1 CSI framework) requires non-trivial spec changes on RAN1 (TS 38.214). However, RAN1 Rel-19 AI PHY will be closed in May meeting. Thus, option 2 will put a risk on successful completion of RAN2 Rel-19 AI PHY.

Proposal 1: The RRC framework with a same level of MeasConfig is adopt for the NW side data collection.

Proposal 2: Introduce the following new RRC configurations for NW side data collection configuration for AI/ML based Beam management:

- Embed loggedDataCollectionConfig in RRCReconfiguration

- Include bm-DataMeasResourceConfig (measurement resources referring to legacy L1 resource CSI-ResourceConfigId), bm-DataLoggingConfig (logging parameters), and loggedDataCollectionLinkage (resource-logging mapping).

Proposal 3: Send an LS to inform RAN3 of RAN2 agreements regarding RRC framework of NW side data collection for AI/ML based beam management.

References

- R2-2502904	 NW-side data collection for beam management and positioning	       Ericsson         	discussion

Annex1

Option 1a

–	 RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                                                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs                                           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=         SEQUENCE {

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

sk-Counter                               SK-Counter                                                            OPTIONAL,   -- Need N

nonCriticalExtension                     RRCReconfiguration-v1610-IEs                                          OPTIONAL

}

RRCReconfiguration-v1610-IEs ::=        SEQUENCE {

otherConfig-v1610                       OtherConfig-v1610                                                    OPTIONAL, -- Need M

bap-Config-r16                          SetupRelease { BAP-Config-r16 }                                      OPTIONAL, -- Need M

iab-IP-AddressConfigurationList-r16     IAB-IP-AddressConfigurationList-r16                                  OPTIONAL, -- Need M

conditionalReconfiguration-r16          ConditionalReconfiguration-r16                                       OPTIONAL, -- Need M

daps-SourceRelease-r16                  ENUMERATED{true}                                                     OPTIONAL, -- Need N

t316-r16                                SetupRelease {T316-r16}                                              OPTIONAL, -- Need M

needForGapsConfigNR-r16                 SetupRelease {NeedForGapsConfigNR-r16}                               OPTIONAL, -- Need M

onDemandSIB-Request-r16                 SetupRelease { OnDemandSIB-Request-r16 }                             OPTIONAL, -- Need M

dedicatedPosSysInfoDelivery-r16         OCTET STRING (CONTAINING PosSystemInformation-r16-IEs)               OPTIONAL, -- Need N

sl-ConfigDedicatedNR-r16                SetupRelease {SL-ConfigDedicatedNR-r16}                              OPTIONAL, -- Need M

sl-ConfigDedicatedEUTRA-Info-r16        SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16}                      OPTIONAL, -- Need M

targetCellSMTC-SCG-r16                  SSB-MTC                                                              OPTIONAL, -- Need S

nonCriticalExtension                    RRCReconfiguration-v1700-IEs                                         OPTIONAL

}

RRCReconfiguration-v1700-IEs ::=        SEQUENCE {

otherConfig-v1700                       OtherConfig-v1700                                              OPTIONAL, -- Need M

sl-L2RelayUE-Config-r17                 SetupRelease { SL-L2RelayUE-Config-r17 }                       OPTIONAL, -- Need M

sl-L2RemoteUE-Config-r17                SetupRelease { SL-L2RemoteUE-Config-r17 }                      OPTIONAL, -- Need M

dedicatedPagingDelivery-r17             OCTET STRING (CONTAINING Paging)                               OPTIONAL, -- Cond PagingRelay

needForGapNCSG-ConfigNR-r17             SetupRelease {NeedForGapNCSG-ConfigNR-r17}                     OPTIONAL, -- Need M

needForGapNCSG-ConfigEUTRA-r17          SetupRelease {NeedForGapNCSG-ConfigEUTRA-r17}                  OPTIONAL, -- Need M

musim-GapConfig-r17                     SetupRelease {MUSIM-GapConfig-r17}                             OPTIONAL, -- Need M

ul-GapFR2-Config-r17                    SetupRelease { UL-GapFR2-Config-r17 }                          OPTIONAL, -- Need M

scg-State-r17                           ENUMERATED { deactivated }                                     OPTIONAL, -- Need S

appLayerMeasConfig-r17                  AppLayerMeasConfig-r17                                         OPTIONAL, -- Need M

ue-TxTEG-RequestUL-TDOA-Config-r17      SetupRelease {UE-TxTEG-RequestUL-TDOA-Config-r17}              OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1800-IEs                                   OPTIONAL

}

RRCReconfiguration-v1800-IEs ::=        SEQUENCE {

needForInterruptionConfigNR-r18         ENUMERATED { disabled, enabled }                                   OPTIONAL, -- Need M

aerial-Config-r18                           SetupRelease { Aerial-Config-r18 }                             OPTIONAL, -- Need M

sl-IndirectPathAddChange-r18                SetupRelease { SL-IndirectPathAddChange-r18 }                  OPTIONAL, -- Need M

n3c-IndirectPathAddChange-r18               SetupRelease { N3C-IndirectPathAddChange-r18 }                 OPTIONAL, -- Need M

n3c-IndirectPathConfigRelay-r18             SetupRelease { N3C-IndirectPathConfigRelay-r18 }               OPTIONAL, -- Need M

otherConfig-v1800                           OtherConfig-v1800                                              OPTIONAL, -- Need M

srs-PosResourceSetAggBW-CombinationList-r18 SetupRelease { SRS-PosResourceSetAggBW-CombinationList-r18 }   OPTIONAL, -- Need M

ltm-Config-r18                              SetupRelease {LTM-Config-r18}                                  OPTIONAL, -- Need M

nonCriticalExtension                        RRCReconfiguration-v1830-IEs                                   OPTIONAL

}

RRCReconfiguration-v1830-IEs ::=        SEQUENCE {

otherConfig-v1830                       OtherConfig-v1830                                                  OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1900-IEs                            OPTIONAL

}

RRCReconfiguration-v1900-IEs ::=        SEQUENCE {

loggedDataCollectionConfig-r19					SetupRelease {LoggedDataCollectionConfig-r19}					 OPTIONAL, -- Need M

nonCriticalExtension							SEQUENCE {}													 OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

BAP-Config-r16 ::=                      SEQUENCE {

bap-Address-r16                         BIT STRING (SIZE (10))                                    OPTIONAL, -- Need M

defaultUL-BAP-RoutingID-r16             BAP-RoutingID-r16                                         OPTIONAL, -- Need M

defaultUL-BH-RLC-Channel-r16            BH-RLC-ChannelID-r16                                      OPTIONAL, -- Need M

flowControlFeedbackType-r16             ENUMERATED {perBH-RLC-Channel, perRoutingID, both}        OPTIONAL, -- Need R

...

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

OnDemandSIB-Request-r16 ::=                  SEQUENCE {

onDemandSIB-RequestProhibitTimer-r16         ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30}

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms1000, ms1500, ms2000}

IAB-IP-AddressConfigurationList-r16 ::= SEQUENCE {

iab-IP-AddressToAddModList-r16      SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressConfiguration-r16 OPTIONAL, -- Need N

iab-IP-AddressToReleaseList-r16     SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressIndex-r16         OPTIONAL, -- Need N

...

}

IAB-IP-AddressConfiguration-r16 ::=     SEQUENCE {

iab-IP-AddressIndex-r16                 IAB-IP-AddressIndex-r16,

iab-IP-Address-r16                      IAB-IP-Address-r16                                                OPTIONAL,  -- Need M

iab-IP-Usage-r16                        IAB-IP-Usage-r16                                                  OPTIONAL,  -- Need M

iab-donor-DU-BAP-Address-r16            BIT STRING (SIZE(10))                                             OPTIONAL,  -- Need M

...

}

SL-ConfigDedicatedEUTRA-Info-r16 ::=            SEQUENCE {

sl-ConfigDedicatedEUTRA-r16                    OCTET STRING                                              OPTIONAL,  -- Need M

sl-TimeOffsetEUTRA-List-r16                    SEQUENCE (SIZE (8)) OF SL-TimeOffsetEUTRA-r16             OPTIONAL    -- Need M

}

SL-TimeOffsetEUTRA-r16 ::=        ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1, ms1dot25, ms1dot5, ms1dot75,

ms2, ms2dot5, ms3, ms4, ms5, ms6, ms8, ms10, ms20}

UE-TxTEG-RequestUL-TDOA-Config-r17 ::=  CHOICE {

oneShot-r17                             NULL,

periodicReporting-r17                   ENUMERATED { ms160, ms320, ms1280, ms2560, ms61440, ms81920, ms368640, ms737280 }

}

SRS-PosResourceSetAggBW-CombinationList-r18 ::= SEQUENCE (SIZE(1.. maxNrOfLinkedSRS-PosResSetComb-r18)) OF SRS-PosResourceSetLinkedForAggBW-List-r18

SRS-PosResourceSetLinkedForAggBW-List-r18 ::= SEQUENCE (SIZE(2..maxNrOfLinkedSRS-PosResourceSet-r18)) OF SRS-PosResourceSetLinkedForAggBW-r18

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

/omit for short/

### 6.3.2	Radio resource control information element

#### –	BM-DataLoggingConfig

The IE BM-DataLoggingConfig specifies the data logging related configuration for AI/ML functions.

BM-DataLoggingConfig information element

-- ASN1START

-- TAG-BM-DataLoggingConfig-START

BM-DataLoggingConfig-r19 ::=              SEQUENCE {

bm-DataLoggingConfigId-r19						BM-DataLoggingConfigId-r19,

loggingType-r19                    CHOICE {

eventTriggerdLogging-r19				FFS,

Periodic								BOOLEAN

},

loggingQuantity-r19                                FFS,

...,

}

-- TAG-BM-LoggedDataCollectionConfig-STOP

-- ASN1STOP

#### –	BM-DataLoggingConfigId

The IE BM-DataLoggingConfigId is used to identify the BM-DataLoggingConfig.

BM-DataLoggingConfig information element

-- ASN1START

-- TAG-BM-DataLoggingConfigId-START

BM-DataLoggingConfigId-r19 ::= INTEGER (0..maxNrofBM-DataLoggingConfig-1-r19)

-- TAG-BM-LoggedDataCollectionConfigId-STOP

-- ASN1STOP

#### –	BM-DataMeasResource

The IE BM-DataMeasResource specifies the measurement resources for UE to perform the NW side data collection in terms of AI/ML based beam management,

BM-DataMeasResource information element

-- ASN1START

-- TAG-BM-DataMeasResource-START

DataMeasResource-19 ::=              SEQUENCE {

bM-dataMeasResourceId-r19                                       BM-DataMeasResourceId-r19

servingcellId										ServCellIndex                                          OPTIONAL, -- Need R

measResourceSetAForBM-r19                                     CSI-ResourceConfigId                              OPTIONAL, -- Need R

measResourceSetBForBM-r19                               CSI-ResourceConfigId						      OPTIONAL, -- Need R

...,

}

-- TAG-BM-DataMeasResource-STOP

-- ASN1STOP

DataMeasResourceConfig field descriptions

measResourceSetAForBM, measResourceSetBforBM To indicate the CSI measurement resources for data collection of AI/ML based beam management

DataMeasResourceForBM field descriptions

servingCellId To indicate the serving cell where the measResourceSetAForBM and measResourceSetBForBM is from.

#### –	BM-DataMeasResourceId

The IE BM-DataMeasResourceId is used to identify one DataMeasResource.

BM-DataMeasResourceId information element

-- ASN1START

-- TAG-BM-DataMeasResourceId-START

DataMeasResourceId-r19 ::=        INTEGER (0..maxNrofDataMeasResource-1-r19)

-- TAG-BM-DataMeasResourceId-STOP

-- ASN1STOP

#### –	LoggedDataCollectionConfig

The IE loggedDatacCollection specifies the data collection for AI/ML functions to be performed by the UE, and covers measurement resource part, data logging part, and the linkage between measurement resource and data logging configuration.

LoggedDataCollectionConfig information element

-- ASN1START

-- TAG-LoggedDataCollectionConfig-START

LoggedDataCollectionConfig-19 ::=              SEQUENCE {

bm-dataMeasResourceToAddModList-r19             SEQUENCE (SIZE (1..maxNrofBM-DataMeasResources-r19)) OF BM-DataMeasResource-r19   OPTIONAL, -- Need N

bm-dataMeasResourceToReleaseList-r19            SEQUENCE (SIZE (1.. maxNrofBM-DataMeasResources-r19)) OF BM-DataMeasResourceId-r19 OPTIONAL, -- Need N

bm-loggingConfigToAddModList-r19					SEQUENCE (SIZE (1..maxNrofBM-LoggingConfig-r19)) OF BM-LoggingConfig-r19           OPTIONAL, -- Need N

bm-loggingConfigForBMToReleaseList-r19            SEQUENCE (SIZE (1.. maxNrofBM-LoggingConfig-r19)) OF BM-LoggingConfigId-r19           OPTIONAL, -- Need N

loggeDataColletionLinkageToAddModList-r19             SEQUENCE (SIZE (1..maxNrofLoggedDataCollectionLinkage-r19)) OF LoggedDataCollectionLinkage-r19   OPTIONAL, -- Need N

loggedDataCollectionLinkageToReleaseList-r19           SEQUENCE (SIZE (1..maxNrofLoggedDataCollectionLinkage-r19)) OF LoggedDataCollectionId-r19          OPTIONAL, -- Need N

...,

}

-- TAG-LoggedDataCollectionConfig-STOP

-- ASN1STOP

LoggedDataCollectionConifg field descriptions

bm-dataMeasResourceToAddModList Contains the measurement resource configurations for UE to perform the data collection.

bm-loggingConfigToAddModList Contains the data logging related configuration for UE to perform the data logging..

loggedDataCollectionlinkageToAddModList Contains the loggedDataCollectionId for UE to associate the bm-dataMeasResource with the bm-loggingConfig

#### –	LoggedDataCollectionLinkage

The IE LoggedDataCollecgionLinkage specifies the linkage between DataMeasResourceForBM and loggingConfigForBM

LoggedDataCleectionLinkage information element

-- ASN1START

-- TAG-LoggedDataCollectionLinkage-START

LoggedDataCollectionLinkage-r19 ::=                  SEQUENCE {

loggedDataCollectionId-r19                              LoggedDataCollectionId-r19,

bm-dataMeasResourceForBMId-r19                                  BM-DataMeasResourceId-r19,                  OPTIONAL, -- Need R

bm-loggingConfigForBMId-r19                      BM-LoggingConfigId-r19,                              OPTIONAL, -- Need R

...,

}

-- TAG-LoggedDataCollectionLinkage-STOP

-- ASN1STOP

#### –	LoggedDataCollectionId

The IE LoggedDataCollecgionId is used for identifying the LoggedDataCollectionLinkage

LoggedDataCleectionLinkage information element

-- ASN1START

-- TAG-LoggedDataCollecgionId-START

LoggedDataCollectionId-r19 ::=                  INTEGER (0..maxNrofLoggedDataCollectionLinkage-1-r19)

-- TAG-LoggedDataCollecgionId-STOP

-- ASN1STOP

Option 1b

### 6.3.2	Radio resource control information element

#### –	BM-DataLoggingConfig

The IE BM-DataLoggingConfig specifies the data logging related configuration for AI/ML functions.

BM-DataLoggingConfig information element

-- ASN1START

-- TAG-BM-DataLoggingConfig-START

BM-DataLoggingConfig-r19 ::=              SEQUENCE {

bm-DataLoggingConfigId-r19						BM-DataLoggingConfigId-r19,

bm-DataMeasResourceId-r19						BM-DataMeasResourceId-r19,

loggingType-r19                    CHOICE {

eventTriggerdLogging-r19				FFS,

Periodic-r19								BOOLEAN

},

loggingQuantity-r19                                FFS,

...,

}

-- TAG-BM-LoggedDataCollectionConfig-STOP

-- ASN1STOP

#### –	BM-DataLoggingConfigId

The IE BM-DataLoggingConfigId is used to identify the BM-DataLoggingConfig.

BM-DataLoggingConfig information element

-- ASN1START

-- TAG-BM-DataLoggingConfigId-START

BM-DataLoggingConfigId-r19 ::= INTEGER (0..maxNrofBM-DataLoggingConfig-1-r19)

-- TAG- BM-DataLoggingConfigId-STOP

-- ASN1STOP

#### –	BM-DataMeasResource

The IE BM-DataMeasResource specifies the measurement resources for UE to perform the NW side data collection in terms of AI/ML based beam management,

BM-DataMeasResource information element

-- ASN1START

-- TAG-BM-DataMeasResource-START

DataMeasResource-19 ::=              SEQUENCE {

bM-dataMeasResourceId-r19                                       BM-DataMeasResourceId-r19

measResourceSetAForBM-r19                                     CSI-ResourceConfigId                              OPTIONAL, -- Need R

measResourceSetBForBM-r19                               CSI-ResourceConfigId						      OPTIONAL, -- Need R

...,

}

-- TAG-BM-DataMeasResource-STOP

-- ASN1STOP

DataMeasResourceConfig field descriptions

measResourceSetAForBM, measResourceSetBforBM To indicate the CSI measurement resources for data collection of AI/ML based beam management

#### –	BM-DataMeasResourceId

The IE BM-DataMeasResourceId is used to identify one DataMeasResource.

BM-DataMeasResourceId information element

-- ASN1START

-- TAG-BM-DataMeasResourceId-START

DataMeasResourceId-r19 ::=        INTEGER (0..maxNrofDataMeasResource-1-r19)

-- TAG-BM-DataMeasResourceId-STOP

-- ASN1STOP

#### –	LoggedDataCollectionConfig

The IE loggedDatacCollection specifies the data collection for AI/ML functions to be performed by the UE, and covers measurement part, logging part.

LoggedDataCollectionConfig information element

-- ASN1START

-- TAG-LoggedDataCollectionConfig-START

LoggedDataCollectionConfig-19 ::=              SEQUENCE {

bm-dataMeasResourceToAddModList-r19             SEQUENCE (SIZE (1..maxNrofBM-DataMeasResources-r19)) OF BM-DataMeasResource-r19   OPTIONAL, -- Need N

bm-dataMeasResourceToReleaseList-r19            SEQUENCE (SIZE (1.. maxNrofBM-DataMeasResources-r19)) OF BM-DataMeasResourceId-r19 OPTIONAL, -- Need N

bm-loggingConfigToAddModList-r19					SEQUENCE (SIZE (1..maxNrofBM-LoggingConfig-r19)) OF BM-LoggingConfig-r19           OPTIONAL, -- Need N

bm-loggingConfigForBMToReleaseList-r19            SEQUENCE (SIZE (1.. maxNrofBM-LoggingConfig-r19)) OF BM-LoggingConfigId-r19           OPTIONAL, -- Need N

...,

}

-- TAG-LoggedDataCollectionConfig-STOP

-- ASN1STOP

LoggedDataCollectionConifg field descriptions

bm-dataMeasResourceToAddModList Contains the measurement resource configurations for UE to perform the data collection for AI/ML based beam management.

bm-loggingConfigToAddModList Contains the data logging related configuration for UE to perform the data logging for AI/ML based beam management.

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

ServingCellConfig information element

-- ASN1START

-- TAG-SERVINGCELLCONFIG-START

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink                         OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                                   OPTIONAL,   -- Cond SyncAndCellAdd

bwp-InactivityTimer                 ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30,

ms40,ms50, ms60, ms80,ms100, ms200,ms300, ms500,

ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1 }    OPTIONAL,   --Need R

defaultDownlinkBWP-Id               BWP-Id                                                                  OPTIONAL,   -- Need S

uplinkConfig                        UplinkConfig                                                            OPTIONAL,   -- Need M

supplementaryUplink                 UplinkConfig                                                            OPTIONAL,   -- Need M

pdcch-ServingCellConfig             SetupRelease { PDCCH-ServingCellConfig }                                OPTIONAL,   -- Need M

pdsch-ServingCellConfig             SetupRelease { PDSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

csi-MeasConfig                      SetupRelease { CSI-MeasConfig }                                         OPTIONAL,   -- Need M

sCellDeactivationTimer              ENUMERATED {ms20, ms40, ms80, ms160, ms200, ms240,

ms320, ms400, ms480, ms520, ms640, ms720,

ms840, ms1280, spare2,spare1}       OPTIONAL,   -- Cond ServingCellWithoutPUCCH

crossCarrierSchedulingConfig        CrossCarrierSchedulingConfig                                            OPTIONAL,   -- Need M

tag-Id                              TAG-Id,

dummy1                              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

pathlossReferenceLinking            ENUMERATED {spCell, sCell}                                              OPTIONAL,   -- Cond SCellOnly

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

...,

[[

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                                OPTIONAL,   -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern       OPTIONAL,   -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId     OPTIONAL,   -- Need N

downlinkChannelBW-PerSCS-List       SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

supplementaryUplinkRelease-r16      ENUMERATED {true}                                                       OPTIONAL,   -- Need N

tdd-UL-DL-ConfigurationDedicated-IAB-MT-r16    TDD-UL-DL-ConfigDedicated-IAB-MT-r16                         OPTIONAL,   -- Cond TDD_IAB

dormantBWP-Config-r16               SetupRelease { DormantBWP-Config-r16 }                                  OPTIONAL,   -- Need M

ca-SlotOffset-r16                   CHOICE {

refSCS15kHz                         INTEGER (-2..2),

refSCS30KHz                         INTEGER (-5..5),

refSCS60KHz                         INTEGER (-10..10),

refSCS120KHz                        INTEGER (-20..20)

}                                                                                                           OPTIONAL,   -- Cond AsyncCA

dummy2                              SetupRelease { DummyJ }                                                 OPTIONAL,   -- Need M

intraCellGuardBandsDL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

intraCellGuardBandsUL-List-r16      SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16           OPTIONAL,   -- Need S

csi-RS-ValidationWithDCI-r16        ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

lte-CRS-PatternList1-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList2-r16            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

crs-RateMatch-PerCORESETPoolIndex-r16  ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

enableTwoDefaultTCI-States-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultTCI-StatePerCoresetPoolIndex-r16 ENUMERATED {enabled}                                          OPTIONAL,   -- Need R

enableBeamSwitchTiming-r16          ENUMERATED {true}                                                       OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType1-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

cbg-TxDiffTBsProcessingType2-r16    ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

directionalCollisionHandling-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

channelAccessConfig-r16             SetupRelease { ChannelAccessConfig-r16 }                                OPTIONAL    -- Need M

]],

[[

nr-dl-PRS-PDC-Info-r17                 SetupRelease {NR-DL-PRS-PDC-Info-r17}                                OPTIONAL,   -- Need M

semiStaticChannelAccessConfigUE-r17    SetupRelease {SemiStaticChannelAccessConfigUE-r17}                   OPTIONAL,   -- Need M

mimoParam-r17                       SetupRelease {MIMOParam-r17}                                            OPTIONAL,   -- Need M

channelAccessMode2-r17              ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

timeDomainHARQ-BundlingType1-r17    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

nrofHARQ-BundlingGroups-r17         ENUMERATED {n1, n2, n4}                                                 OPTIONAL,   -- Need R

fdmed-ReceptionMulticast-r17        ENUMERATED {true}                                                       OPTIONAL,   -- Need R

moreThanOneNackOnlyMode-r17         ENUMERATED {mode2}                                                      OPTIONAL,   -- Need S

tci-ActivatedConfig-r17             TCI-ActivatedConfig-r17                                                 OPTIONAL,   -- Cond TCI_ActivatedConfig

directionalCollisionHandling-DC-r17 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

lte-NeighCellsCRS-AssistInfoList-r17  SetupRelease { LTE-NeighCellsCRS-AssistInfoList-r17 }                 OPTIONAL    -- Need M

]],

[[

lte-NeighCellsCRS-Assumptions-r17   ENUMERATED {false}                                                      OPTIONAL    -- Need R

]],

[[

crossCarrierSchedulingConfigRelease-r17 ENUMERATED {true}                                                   OPTIONAL    -- Need N

]],

[[

multiPDSCH-PerSlotType1-CB-r17      ENUMERATED {enabled, disabled}                                          OPTIONAL    -- Need R

]],

[[

lte-CRS-PatternList3-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

lte-CRS-PatternList4-r18            SetupRelease { LTE-CRS-PatternList-r16 }                                OPTIONAL,   -- Need M

pdcch-CandidateReceptionWithCRS-Overlap-r18  ENUMERATED {enabled}                                           OPTIONAL,   -- Need R

cjt-Scheme-PDSCH-r18                ENUMERATED {cjtSchemeA, cjtSchemeB}                                     OPTIONAL,   -- Need R

tag2-r18                            Tag2-r18                                                                OPTIONAL,   -- Need R

cellDTX-DRX-Config-r18              SetupRelease { CellDTX-DRX-Config-r18 }                                 OPTIONAL,   -- Need M

positionInDCI-cellDTRX-r18          INTEGER (0..maxDCI-2-9-Size-1-r18)                                      OPTIONAL,   -- Need R

cellDTX-DRX-L1activation-r18        ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

mc-DCI-SetOfCellsToAddModList-r18   SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18    OPTIONAL,   -- Need N

mc-DCI-SetOfCellsToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF SetOfCellsId-r18         OPTIONAL    -- Need N

]]

[[

loggedDataCollectionConfig-r19     SetupRelease { LoggedDataCollectionConfig-r19 }                                OPTIONAL,   -- Need M

]]

}

Tag2-r18 ::=                        SEQUENCE {

tag2-Id-r18                         TAG-Id,

tag2-flag-r18                       BOOLEAN,

n-TimingAdvanceOffset2-r18          ENUMERATED { n0, n25600, n39936, spare1 }                           OPTIONAL    -- Need S

}

UplinkConfig ::=                    SEQUENCE {

initialUplinkBWP                    BWP-UplinkDedicated                                                     OPTIONAL,   -- Need M

uplinkBWP-ToReleaseList             SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                              OPTIONAL,   -- Need N

uplinkBWP-ToAddModList              SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink                          OPTIONAL,   -- Need N

firstActiveUplinkBWP-Id             BWP-Id                                                                  OPTIONAL,   -- Cond SyncAndCellAdd

pusch-ServingCellConfig             SetupRelease { PUSCH-ServingCellConfig }                                OPTIONAL,   -- Need M

carrierSwitching                    SetupRelease { SRS-CarrierSwitching }                                   OPTIONAL,   -- Need M

...,

[[

powerBoostPi2BPSK                   BOOLEAN                                                                 OPTIONAL,   -- Need M

uplinkChannelBW-PerSCS-List         SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier                     OPTIONAL    -- Need S

]],

[[

enablePL-RS-UpdateForPUSCH-SRS-r16  ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUSCH0-0-r16 ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForPUCCH-r16    ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

enableDefaultBeamPL-ForSRS-r16      ENUMERATED {enabled}                                                    OPTIONAL,   -- Need R

uplinkTxSwitching-r16               SetupRelease { UplinkTxSwitching-r16 }                                  OPTIONAL,   -- Need M

mpr-PowerBoost-FR2-r16              ENUMERATED {true}                                                       OPTIONAL    -- Need R

]],

[[

srs-PosTx-Hopping-r18               SetupRelease { SRS-PosTx-Hopping-r18 }                                  OPTIONAL,   -- Need M

enablePL-RS-UpdateForType1CG-PUSCH-r18  ENUMERATED {enabled}                                                OPTIONAL,   -- Need R

powerBoostPi2BPSK-r18               BOOLEAN                                                                 OPTIONAL,   -- Need R

powerBoostQPSK-r18                  BOOLEAN                                                                 OPTIONAL    -- Need R

]]

}

DummyJ ::=                          SEQUENCE {

maxEnergyDetectionThreshold-r16         INTEGER(-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-20..-13),

ul-toDL-COT-SharingED-Threshold-r16     INTEGER (-85..-52)                                                  OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16         ENUMERATED {true}                                                   OPTIONAL    -- Need R

}

ChannelAccessConfig-r16 ::=         SEQUENCE {

energyDetectionConfig-r16           CHOICE {

maxEnergyDetectionThreshold-r16         INTEGER (-85..-52),

energyDetectionThresholdOffset-r16      INTEGER (-13..20)

}                                                                                                           OPTIONAL,   -- Need R

ul-toDL-COT-SharingED-Threshold-r16         INTEGER (-85..-52)                                              OPTIONAL,   -- Need R

absenceOfAnyOtherTechnology-r16             ENUMERATED {true}                                               OPTIONAL    -- Need R

}

IntraCellGuardBandsPerSCS-r16 ::=      SEQUENCE {

guardBandSCS-r16                       SubcarrierSpacing,

intraCellGuardBands-r16                SEQUENCE (SIZE (1..4)) OF GuardBand-r16

}

GuardBand-r16 ::=                      SEQUENCE {

startCRB-r16                          INTEGER (0..274),

nrofCRBs-r16                          INTEGER (0..15)

}

DormancyGroupID-r16 ::=         INTEGER (0..4)

DormantBWP-Config-r16::=               SEQUENCE {

dormantBWP-Id-r16                      BWP-Id                                                           OPTIONAL,   -- Need M

withinActiveTimeConfig-r16             SetupRelease { WithinActiveTimeConfig-r16 }                      OPTIONAL,   -- Need M

outsideActiveTimeConfig-r16            SetupRelease { OutsideActiveTimeConfig-r16 }                     OPTIONAL    -- Need M

}

WithinActiveTimeConfig-r16 ::=         SEQUENCE {

firstWithinActiveTimeBWP-Id-r16         BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupWithinActiveTime-r16       DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

OutsideActiveTimeConfig-r16 ::=        SEQUENCE {

firstOutsideActiveTimeBWP-Id-r16        BWP-Id                                                           OPTIONAL,   -- Need M

dormancyGroupOutsideActiveTime-r16      DormancyGroupID-r16                                              OPTIONAL    -- Need R

}

UplinkTxSwitching-r16 ::=              SEQUENCE {

uplinkTxSwitchingPeriodLocation-r16    BOOLEAN,

uplinkTxSwitchingCarrier-r16           ENUMERATED {carrier1, carrier2}

}

MIMOParam-r17 ::= SEQUENCE {

additionalPCI-ToAddModList-r17     SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF SSB-MTC-AdditionalPCI-r17  OPTIONAL,   -- Need N

additionalPCI-ToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofAdditionalPCI-r17)) OF AdditionalPCIIndex-r17     OPTIONAL,   -- Need N

unifiedTCI-StateType-r17           ENUMERATED {separate, joint}                                         OPTIONAL,   -- Need R

uplink-PowerControlToAddModList-r17  SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControl-r17      OPTIONAL,   -- Need N

uplink-PowerControlToReleaseList-r17 SEQUENCE (SIZE (1..maxUL-TCI-r17)) OF Uplink-powerControlId-r17    OPTIONAL,   -- Need N

sfnSchemePDCCH-r17                 ENUMERATED {sfnSchemeA,sfnSchemeB}                                   OPTIONAL,   -- Need R

sfnSchemePDSCH-r17                 ENUMERATED {sfnSchemeA,sfnSchemeB}                                   OPTIONAL    -- Need R

}

MC-DCI-SetOfCells-r18 ::=          SEQUENCE {

setOfCellsId-r18                   SetOfCellsId-r18,

nCI-Value-r18                      INTEGER (0..7),

scheduledCellListDCI-1-3-r18       SEQUENCE (SIZE (2..maxNrofCellsInSet-r18)) OF ServCellIndex          OPTIONAL,   -- Need R

scheduledCellListDCI-0-3-r18       SEQUENCE (SIZE (2..maxNrofCellsInSet-r18)) OF ServCellIndex          OPTIONAL,   -- Need R

scheduledCellComboListDCI-1-3-r18  SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 OPTIONAL,   -- Need R

scheduledCellComboListDCI-0-3-r18  SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 OPTIONAL,   -- Need R

antennaPortsDCI1-3-r18             ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI1-3

antennaPortsDCI0-3-r18             ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

tpmi-DCI0-3-r18                    ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

sri-DCI0-3-r18                     ENUMERATED {type1a, type2}                                           OPTIONAL, -- Cond TypeDCI0-3

priorityIndicatorDCI-1-3-r18       ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

priorityIndicatorDCI-0-3-r18       ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

dormancyDCI-1-3-r18                ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

dormancyDCI-0-3-r18                ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pdcchMonAdaptDCI-1-3-r18           ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pdcchMonAdaptDCI-0-3-r18           ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

minimumSchedulingOffsetK0DCI-1-3-r18        ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

minimumSchedulingOffsetK0DCI-0-3-r18        ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-OneShotFeedbackDCI-1-3-r18   ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-enhType3DCI-1-3-r18          ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-enhType3DCIfieldDCI-1-3-r18  ENUMERATED {enabled}                                        OPTIONAL,   -- Need R

pdsch-HARQ-ACK-retxDCI-1-3-r18     ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

pucch-sSCellDynDCI-1-3-r18         ENUMERATED {enabled}                                                 OPTIONAL,   -- Need R

tdra-FieldIndexListDCI-1-3-r18     SEQUENCE (SIZE (1..32)) OF TDRA-FieldIndexDCI-1-3-r18                OPTIONAL,   -- Need R

tdra-FieldIndexListDCI-0-3-r18     SEQUENCE (SIZE (1..64)) OF TDRA-FieldIndexDCI-0-3-r18                OPTIONAL,   -- Need R

rateMatchListDCI-1-3-r18           SEQUENCE (SIZE (1..16)) OF RateMatchDCI-1-3-r18                      OPTIONAL,   -- Need R

zp-CSI-RSListDCI-1-3-r18           SEQUENCE (SIZE (1..8)) OF ZP-CSI-DCI-1-3-r18                         OPTIONAL,   -- Need R

tci-ListDCI-1-3-r18                SEQUENCE (SIZE (1..16)) OF TCI-DCI-1-3-r18                           OPTIONAL,   -- Need R

srs-RequestListDCI-1-3-r18         SEQUENCE (SIZE (1..16)) OF SRS-RequestCombo-r18                      OPTIONAL,   -- Need R

srs-OffsetListDCI-1-3-r18          SEQUENCE (SIZE (1..8)) OF SRS-OffsetCombo-r18                        OPTIONAL,   -- Need R

srs-RequestListDCI-0-3-r18         SEQUENCE (SIZE (1..16)) OF SRS-RequestCombo-r18                      OPTIONAL,   -- Need R

srs-OffsetListDCI-0-3-r18          SEQUENCE (SIZE (1..8)) OF SRS-OffsetCombo-r18                        OPTIONAL    -- Need R

}

SetOfCellsId-r18 ::=                   INTEGER (0..maxNrofSetsOfCells-1-r18)

ScheduledCellCombo-r18 ::=             SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER (0..maxNrofCellsInSet-1-r18)

RateMatchDCI-1-3-r18 ::=               SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (1..2))

ZP-CSI-DCI-1-3-r18 ::=                 SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (1..2))

TCI-DCI-1-3-r18 ::=                    SEQUENCE (SIZE (2.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (3))

SRS-RequestCombo-r18 ::=               SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF BIT STRING (SIZE (2..3))

SRS-OffsetCombo-r18 ::=                SEQUENCE (SIZE (1.. maxNrofCellsInSet-r18)) OF INTEGER (0..3)

TDRA-FieldIndexDCI-1-3-r18 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofDL-Allocations-1-r18)

TDRA-FieldIndexDCI-0-3-r18 ::=         SEQUENCE (SIZE (2.. maxNrofBWPsInSetOfCells-r18)) OF INTEGER (0..maxNrofUL-Allocations-1-r18)

-- TAG-SERVINGCELLCONFIG-STOP

-- ASN1STOP

Annex 2

Title:	LS on NW-side data collection for beam management

Response to:	-

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	RAN2

To:	RAN3

Cc:

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- 1	 Overall description

For NW-side data collection framework, RAN2 has made the following agreements:

- The RRC framework with a same level of MeasConfig is adopt for the NW side data collection.

- Introduce the following new RRC configurations for NW side data collection configuration for AI/ML based Beam management:

  - Embed loggedDataCollectionConfig in RRCReconfiguration

  - Include bm-DataMeasResourceConfig (measurement resources referring to legacy L1 resource CSI-ResourceConfigId), bm-DataLoggingConfig (logging parameters), and loggedDataCollectionLinkage (resource-logging mapping).

.

- 2 	Actions

To RAN3:

ACTION: 	RAN2 respectfully asks RAN3 to take these agreements into account for their work and notify RAN2 if there are any concerns regarding NW side data collection framework for beam management.

- 3 	Dates of next TSG RAN WG2 meetings

TSG RAN2 Meeting #131		25 - 29 Aug 2025 					Bangalore, India