3GPP TSG-RAN WG2 #130	R2-2503859

Malta, May 19th – 23rd, 2025

Agenda item:	8.6.3

Source: 	Ericsson

Title:  	Discussion on L1 event-based measurements

Document for:	Discussion and Decision

# Introduction

The discussion for Event based L1 measurements in RAN2 was in last meeting focused around beam types and what conditions belongs to each type. This contribution presents our view on some further details around how some concepts should be defined.

# Discussion

## Concepts

### Triggered beams

In the last RAN2 meeting, the following agreement has been reached about the type of beams that can be included in an event-triggered L1 measurement report:

RAN2 will consider only the following options: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE. Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE. Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before). Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

According to the agreement, the definition of type1 and type2 beam is not entirely correct as the triggered beam is indeed a bit which has fulfilled the entering or leaving condition for the entire TTT but is also a bit which has entered the beamsTriggeredList for the first time or leaving it. Thus, we propose to update the definition of the beams to be included in the event-triggered L1 measurement report according to the following:

The following beams can be included within a event-triggered L1 measurement report:

  - Type1 beam: a beam that has satisfied the entry condition of the event for the entire TTT, triggers the MR MAC CE, and enters the beamTriggeredList for the first time.

  - Type2 beam: a beam that has satisfied the leaving condition of the event for the entire TTT, triggers the MR MAC CE, and exit the beamTriggeredList.

  - Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

  - Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

A remaining aspect is how to report these four beam types, and having 2 bits seems the straightforward solution. Therefore, we propose to confirm the working assumption from last RAN2 meeting. Thus, we propose:

RAN2 to confirm the working assumption to introduce four codepoints to represent one of {type1, type2, type3, type4} beams.

## SP CSI-RS activation/deactivation

In RAN2#129bis the following discussion took place:

R2-2502799 Remaining Issues of L1 Event Triggered Measurement Report Samsung discussion Rel-19 NR_Mob_Ph4-Core Proposal 2: RAN2 decide How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options. • Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch • Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch • Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

There was no conclusion but instead it was decided to revisit this next meeting.

From a functional perspective towards the target cell, the use of the semipersistent resources in the candidate cell seems to be fulfilled after a cell switch and hence the reporting could be stopped. However, for other cells it might still be useful to measure resources. Despite this, a simpler solution seems to be to activate the resources again if the network needs it.

UE stop to measure the activated semi-persistent CSI-RS resources after cell switch (Option 1).

## Open MAC issues

From the latest email discussion there is an outstanding MAC issue as described by the rapporteur:

| MAC-5 | The content of L1 MR MAC CE triggered by LTM2 Editor’s NOTE: FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

It is our view that since this event does not include any candidate configurations, only the source cell, it might not be necessary to include candidates beam information. However, it is reasonable to believe that this event type will be configured for the purpose of initiating a mobility procedure, and if this is the case then the network needs to identify possible candidates. These candidates could very well be reported together with the event LTM2 if they are available to the UE.

The UE might have already measured other LTM candidates even if it is only configured with LTM2.

To settle on somewhat of a middle ground, and not make UE impact bigger than necessary, it would be enough to say that the UE is allowed to report other beams if the information is available.

It is up to UE implementation to include measured beams for other LTM candidate cells when reporting a L1 MR MAC CE triggered by a LTM2 event.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The UE might have already measured other LTM candidates even if it is only configured with LTM2.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The following beams can be included within a event-triggered L1 measurement report:

a.	Type1 beam: a beam that has satisfied the entry condition of the event for the entire TTT, triggers the MR MAC CE, and enters the beamTriggeredList for the first time.

b.	Type2 beam: a beam that has satisfied the leaving condition of the event for the entire TTT, triggers the MR MAC CE, and exit the beamTriggeredList.

c.	Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

d.	Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

Proposal 2	RAN2 to confirm the working assumption to introduce four codepoints to represent one of {type1, type2, type3, type4} beams.

Proposal 3	UE stop to measure the activated semi-persistent CSI-RS resources after cell switch (Option 1).

Proposal 4	It is up to UE implementation to include measured beams for other LTM candidate cells when reporting a L1 MR MAC CE triggered by a LTM2 event.

# References

R2-25xxxxx- Running RRC CR for L1 measurements, vivo, RAN2#130, May 2025.