3GPP TSG-RAN WG2 Meeting #130	R2-2503865

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.12.3

Source:	Nokia

Title:	RAN2 Aspects of the NR MIMO

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

MIMO is one of the key technologies in NR systems, and Rel-19, that started with RAN1#116, will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID RP-242394.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this document, we present Nokia’s views regarding requirements that RAN2 need to discuss in relation to the WID, in particular about Objective 1: UE-initiated beam management;

Based on the open issue summary of [Post129bis][214][ MIMO_Ph5] Running CR for 38.331 (Ericsson) and [Post129bis][215][ MIMO_Ph5] Running CR for 38.321 (Samsung)), the following  proposals have been agreed to discuss via contributions in RAN2#130 on asymmetric DL sTRP/UL mTRP:

- For Rel-15 UL skipping (skipUplinkTxDynamic is configured), discuss whether to generate MAC PDU for multiplexing UCI of mode-A DG-based UE-initiated report in PUSCH

- Discuss UE behavior when UE-initiated report is triggered but there is no valid PUCCH/PUSCH resource to transmit UE-initiated report (e.g., PUCCH resource is released by RRC and type-1 CG is cleared in MAC due to TAT expired).

- Discuss whether the same rule of CSI report transmission is applied to UEI report (e.g., for activated/deactivated SCell/SCG, handling of measurement gaps, handling of FR2 UL gaps, BWP operation, cell DRX operation).

# 2	UE-initiated beam management

The latest agreements from RAN1 on UEIBM related to RAN2 discussion are:

Agreement RAN1#117 Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. Agreement RAN1#118 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - - Above applies at least for the single CC case. Agreement RAN1 #118 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case. Agreement RAN1 #119 On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1. Agreement RAN1#119 On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement RAN1 #120 Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. Agreement RAN1 #120Bis On UE-initiated/event-driven beam reporting, the procedure of triggering event determination is captured in RAN1 spec. Agreement RAN1 #120Bis On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.tCandidate#3: No further enhancement.

## Prohibit timer to enable first PUCCH re-transmissions

RAN1 has been discussing about introducing a prohibit timer to enable first PUCCH re-transmissions. Few candidates are on the table as per RAN1 #120Bis agreement copied above. In our understanding, Candidate#1 and Candidate#2 have technically the same effect, i.e., Option-1A is equivalent to Option-2A and Option-1B is equivalent to Option-2B, with the only difference that Candidate#1 uses the term “prohibit timer”, while Candidate#2 uses the term “time interval”, so here we comment only Candidate#1 options.

Option-1B is a prohibit timer after the first PUCCH, and it is mainly for Mode A: in case the UE does not receive a DCI after the first PUCCH within a certain time, then it is allowed to re-transmit the first PUCCH.

Option-1A is a prohibit timer after the PUSCH, i.e., after the UE sends a UEIBM report. When the UE sends a UEIBM report to the gNB triggered by an event, e.g., Event-2, in case the gNB does not do any beam switching after the UEIBM report sent by the UE, it may happen that the conditions triggering the same UEIBM report may persist over a period of time (the same Event-2, i.e., for same new beam(s) and for same conditions, e.g., same RSRP value(s) of the new beam(s) and of the current beam, keep repeating over time). In the current framework (even considering the enhancement where multiple Event-2 instances within a time window need to take place to trigger a UEIBM report) this causes the transmission of multiple consecutive UEIBM reports with same content, i.e., same top-N beams with same RSRP values, which bring no additional information to the network and just cause additional overhead (see also Figure 1 below). Generally, we think in Option-1A it would be beneficial to allow the transmission of a first PUCCH within the timer in case for example different beams triggered the same event or another event has been triggered.

Figure 1. Multiple consecutive UEIBM reports with same contents transmitted by the UE under the current framework.

Although prohibit timers are handled in RAN2 spec, the exact procedures are still not clear for now and up to detailed discussion in RAN1 (with also the possibility that no prohibit timer will be introduced at all). Therefore, on this topic, we suggest to wait for further RAN1 progress.

Observation 1: RAN1 has not concluded on the details of the prohibit timer, or even whether to introduce a timer.

Proposal 1: RAN2 waits for further progress from RAN1 regarding prohibit timer for first PUCCH re-transmissions.

## Rel-15 UL skipping for mode-A DG-based event-triggered beam report.

In the current procedure of UL skipping, it is specified that Rel-15 UL skipping is not applied when aperiodic CSI is requested for the PUSCH transmission.

TS 38.321 clause 5.4.3 … The MAC entity shall: 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

In our view, Mode-A DG-based event-triggered beam report is similar to the aperiodic CSI reporting and the same principle can be adopted for mode-A DG-based event-triggered beam report multiplexed on PUSCH.

Proposal 2: For Rel-15 UL skipping (skipUplinkTxDynamic),  same principle as that of aperiodic CSI reporting can be adopted for mode-A DG-based event-triggered beam reporting.

## UE-IBR in UL synchronized Scenario

The network continuously monitors the timing of uplink signals from the UE and adjusts the UE’s uplink transmission timing by sending Timing Advance Commands. If the UE does not receive a Timing Advance Command before the timeAlignmentTime expires, it assumes that uplink synchronization has been lost. As a result, before initiating any PUSCH, PUCCH, or SRS transmission, the UE must perform an explicit timing re-alignment through the random access procedure to restore uplink time alignment.

Upon the expiry of the timeAlignmentTimer, the UE:

TS 38.321 clause 5.2 … 1> when a timeAlignmentTimer expires: 2> if the timeAlignmentTimer is associated with a PTAG and the SpCell is not configured with two PTAGs; or 2> if the timeAlignmentTimer is associated with a PTAG, the SpCell is configured with two PTAGs, and the timeAlignmentTimer associated with the other PTAG is not running: 3> flush all HARQ buffers for all Serving Cells; 3> notify RRC to release PUCCH for all Serving Cells, if configured; 3> notify RRC to release SRS for all Serving Cells, if configured; 3> clear any configured downlink assignments and configured uplink grants; 3> clear any PUSCH resource for semi-persistent CSI reporting; 3> consider all running timeAlignmentTimers as expired; 3> maintain NTA (defined in TS 38.211 [8]) of all TAGs. 2> else: 3> if the timeAlignmentTimer is associated with a TAG for an SCell configured with only this TAG; or 3> if the timeAlignmentTimer is associated with a TAG for an SCell, and if the SCell is configured with two TAGs and the timeAlignmentTimer associated with the other TAG is not running: 4> flush all HARQ buffers for all such SCells; 4> notify RRC to release PUCCH, if configured for all such SCells; 4> notify RRC to release SRS, if configured for all such SCells; 4> clear any configured downlink assignments and configured uplink grants for all such SCells; 4> clear any PUSCH resource for semi-persistent CSI reporting for all such SCells; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG. 3> else if the timeAlignmentTimer is associated with a TAG for a Serving Cell configured with two TAGs, and if the timeAlignmentTimer associated with the other TAG is running, for all such Serving Cells: 4> clear any configured downlink assignment, if the activated TCI state(s) for all PUCCH resources configured for the configured downlink assignment is associated with the TAG of the expired timeAlignmentTimer; 4> clear any configured uplink grant, if the activated TCI state(s) for the configured uplink grant is associated with the TAG of the expired timeAlignmentTimer; 4> clear any PUSCH resource for semi-persistent CSI reporting, if the activated TCI state(s) for the PUSCH resource is associated with the TAG of the expired timeAlignmentTimer; 4> maintain NTA (defined in TS 38.211 [8]) of this TAG.

As we can observe if UL is not synchronized, the UE shall release PUCCH resources of the SR and consider the SR as  pending SR. For the pending SR,

TS 38.321 clause 5.4.4 … As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR; and 1> if there is no ongoing RACH-less LTM cell switch; and 1> if rach-LessHO is not configured: 2> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

Hence we propose:

Proposal 3: Upon UE initiated beam reporting trigger for  Mode A and Mode B UE initiated beam reporting,  if the UL is not synchronized, the UE shall initiate RACH procedure and cancel the pending SR.

## UE-IBR for Activated and Deactivated SCell

The MAC specification for CSI reporting on deactivated Scell does not support CSI report and PUCCH transmission on the Scell.

TS 38.321 clause 5.9 … 1> if the SCell is deactivated: 2> not transmit SRS on the SCell; 2> not report CSI for the SCell; 2> not transmit on UL-SCH on the SCell; 2> not transmit on RACH on the SCell; 2> not monitor the PDCCH on the SCell; 2> not monitor the PDCCH for the SCell; 2> not transmit PUCCH on the SCell; 2> if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured: 3> not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

In our view Mode A UE-IBR is similar to aperiodc CSI reporting and Mode B UE-IBR is similar to semi persistent CSI-reporting. Hence same principle as aperiodic and semi persistent CSI reporting can be applied for Mode A and Mode B UE IBM for deactivated Scell.

Proposal 4: Not to report UE-IBR (Mode A and Mode B) for the deactivated SCell, and cancel any pending UE-IBR

## Handling of measurement gaps

The current MAC specification does not support the UE to  perform the transmission of HARQ feedback, SR, and CSI on the serving cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig. Hence the UE cannot report UE-IBR on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig.

TS 38.321 clause 5.14 During an activated measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: 1> not perform the transmission of HARQ feedback, SR, and CSI; 1> not report SRS; 1> not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2; 1> if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running, or if there is an ongoing RACH-less LTM cell switch, or if there is an ongoing RACH-less handover: 2> monitor the PDCCH as specified in clauses 5.1.4, 5.1.4a, 5.1.5, and 5.7. 1> else: 2> not monitor the PDCCH; 2> not receive on DL-SCH.

Proposal 5: UE cannot report UE-IBR on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig.  No MAC impact

# Conclusion

This document has made the following observations:

Observation 1: RAN1 has not concluded on the details of the prohibit timer, or even whether to introduce a timer.

And proposed the following:

Proposal 1: RAN2 waits for further progress from RAN1 regarding prohibit timer for first PUCCH re-transmissions.

Proposal 2: For Rel-15 UL skipping (skipUplinkTxDynamic),  same principle as that of aperiodic CSI reporting can be adopted for mode-A DG-based event-triggered beam reporting.

Proposal 3: Upon UE initiated beam reporting trigger for  Mode A and Mode B UE initiated beam reporting,  if the UL is not synchronized, the UE shall initiate RACH procedure and cancel the pending SR.

Proposal 4: Not to report UE-IBR (Mode A and Mode B) for the deactivated SCell, and cancel any pending UE-IBR

Proposal 5: UE cannot report UE-IBR on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig.  No MAC impact