3GPP TSG-RAN WG2 Meeting #130 	R2-2503870

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.4

Source:	Panasonic

Title:	Discussion on addressing issue for AS ID assignment and segment retransmission

Document for:	Discussion

# Introduction

In this paper, we discuss some remaining issues on data transmission for A-IoT, including addressing issues for AS ID assignment and upper layer command for segment retransmission.

# Discussion

## Addressing issue for AS ID assignment

In previous meeting, it has been agreed that for CBRA, Msg2 is used for AS ID assignment and for CFRA, command message is used for AS ID assignment. We discuss the addressing issue separately for CBRA and CFRA below.

Msg2 addressing in case of CBRA

Msg2 contains RN16 that reader has received from Msg1. However, in case that reader uses Msg2 to assign a new AS ID, it is not sufficient to only rely on RN16 for Msg2 addressing. This is because if Msg3 is not received by the reader, reader cannot distinguish the following two scenarios (see Figure 1 below):

-	Scenario 1: Msg 2 is missed by device, meaning that AS ID is not acquired by device;

-	Scenario 2: Device has successfully received Msg 2 but Msg 3 delivery fails, meaning that AS ID is acquired by device.

Figure 1. Failure of Msg 3 creates reader’s confusion on AS ID availability at device

In the previous meeting, it has been agreed that device only keeps one AS ID at a time. This means that the only AS ID kept in device could be RN16 (in the above scenario 1) or a new AS ID (in the above scenario 2), but reader cannot know which one. Therefore, at least for requesting retransmission of Msg3, the Msg2 should be addressing to both AS ID and RN16. To simplify the Msg2 design, the initial transmission and retransmission of Msg2 should have the same format (which was already agreed in previous meeting). Therefore, we propose that regardless of initial or retransmission of Msg2, Msg2 should be addressed to both RN16 and AS ID (if reader intends to assign AS ID). To have both RN16 and AS ID in Msg2 also remove the need of introducing additional identifier(s) when a Msg2 contains the responses to multiple of Msg1. RAN1#120 agreed to support such case as following.

Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

The device can check whether the transmitted RN16 in Msg1 is contained or not in the received Msg2. In case that the device has already replaced the RN16 with a new AS ID (in above scenario 2), device can check whether the Msg2 contains the new AS ID. Then the device is able to know whether the received Msg2 is the response of its transmitted Msg1.

Proposal 1. For CBRA, both RN16 and AS ID in Msg2 are used for addressing purpose.

Proposal 2. For CBRA, if Msg2 contains responses for multiple devices who have sent Msg1, RN16 or AS ID in the received Msg2 can be used for the identifier of the transmitted Msg1s.

The follow-up question would be whether AS ID field in Msg2 is always present. From our understanding, it is up to reader whether to assign a new AS ID for the device by Msg2. If not, the RN16 will continue to be used as AS ID. At least for inventory only service, the procedure ends after reader successfully receives Msg3. There is no need for reader to change RN16 to a new AS ID by Msg2. Therefore, we suggest to make AS ID field in Msg2 optional.

Proposal 3. AS ID field in Msg2 for CBRA is optional.

addressing for AS ID assignment in case of CFRA

The following has been agreed in previous meeting:

Agreements on Msg1: - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario

In particular, the sentence “AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader” can be interpreted that even for first R2D command message to assign AS ID, there is no other ID needed for addressing. This is not a robust solution due to the following two reasons:

Firstly, consider that device could be located in a multi-reader overlapping area. If neighboring reader sends some R2D command to a device with its AS ID but this R2D command is received by the unintended device in another reader, the unintended device would mistakenly treat this message as an AS ID assignment.

Secondly, even for a single reader scenario, if multiple devices are targeted by one CFRA procedure (related to FFS in the above agreement), careful resource mapping needs to be designed and informed to the devices in order for the device to differentiate which R2D message is for which device.

Due to the above concerns, we suggest the RAN2 to reconsider the above agreement. Similar to CBRA, RN16 is transmitted by device in the first D2R message, and then both RN16 and AS ID in the R2D command message of AS ID assignment are used for addressing.

Proposal 4. (revert RAN2 previous agreement) RN16 is included in the first D2R message in the CFRA procedure and both RN16 and AS ID are used for addressing the device in the R2D command message of AS ID assignment.

## Segmentation retransmission

In previous meeting, the following has been agreed for segmentation.

Agreements on segmentation 1 For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. 2 FFS whether the reader always includes the command for retransmission of segments. 3 1-bit indication is sufficient to indicate whether more D2R data will be sent 4 For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption.

Relate to the above two FFSs, there are two possible methods:

-	Method 1: upper layer command and offset are always present regardless of whether the R2D message is to trigger (re)transmission of first segment/unsegmented D2R or retransmission of segments. In case of first segment/unsegmented data, offset is indicated as zero.

-	Method 2: upper layer command (but no offset field) is included in R2D when triggering (re)transmission of first segment/unsegmented D2R; Offset (but no upper layer command) is included in R2D when triggering retransmission of (non-first) segments

Both methods can work. From perspective of aiming for unified message format, method 1 is better. In addition, by sending (the same) command for retransmission of segments can avoid device to store the command, which shares the same spirit of sending offset every time. Therefore, we suggest RAN2 to adopt method 1.

Proposal 5. Upper layer command and offset are always present regardless of whether the R2D message is to trigger (re)transmission of first segment/unsegmented D2R or retransmission of segments. In case of first segment/unsegmented data, offset is indicated as zero.

# Conclusion

In this paper, we have discussed addressing issues for AS ID assignment and upper layer command for segment retransmission . We have the following proposals.

Proposal 1. For CBRA, both RN16 and AS ID in Msg2 are used for addressing purpose.

Proposal 2. For CBRA, if Msg2 contains responses for multiple devices who have sent Msg1, RN16 or AS ID in the received Msg2 can be used for the identifier of the transmitted Msg1s.

Proposal 3. AS ID field in Msg2 for CBRA is optional.

Proposal 4. (revert RAN2 previous agreement) RN16 is included in the first D2R message in the CFRA procedure and both RN16 and AS ID are used for addressing the device in the R2D command message of AS ID assignment.

Proposal 5. Upper layer command and offset are always present regardless of whether the R2D message is to trigger (re)transmission of first segment/unsegmented D2R or retransmission of segments. In case of first segment/unsegmented data, offset is indicated as zero.