3GPP TSG-RAN WG2 #129bis	R2-2503884

St. Julians, Malta, May 19th – 23rd, 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on NR NTN downlink coverage enhancements

Document for:        	Discussion and decision

Agenda Item:         	8.8.2		Downlink coverage enhancements

- Introduction

Based on the chair notes for last 2 meetings RAN2#129[1] / #129bis[2], the following agreements are made for downlink coverage enhancement:

RAN2 #129:

Agreements:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

- The number of SMTC/gaps a UE needs to consider at any time will not be increased further

RAN2 #129bis:

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

This paper attempts to investigate about consideration on cell barring for downlink coverage enhancement based on the above agreements.

- Discussion

2.1 SMTC list enhancement

To support multiple SMTC periodicities, specification change would be needed since current smtc4list can configure only one periodicity referring to smtc1. As mentioned in open issue list [3], with considering backward compatibility, Option1a and Option1b are raised in the open issue list. Option1a is to extend smtc4list, and Option1b is to introduce a new field (e.g. smtc5). We slightly prefer Option1b to introduce a new field rather than extending smtc4list, because introducing the new field can clearly separate the configuration for legacy UE and Rel-19 DL-CE UE (in a case where multiple SMTC periodicities exist), and it seems to be easier to understand the specification.

Proposal 1: RAN2 to introduce a new SMTC field to support multiple SMTC periodicities

2.2 Maximum number of SMTCs

In the last meeting, it was agreed to support more than 4 SMTC per frequency, and the maximum number of SMTCs is mentioned in the open issue list. We think 6 SMTCs is enough to avoid configuring too many SMTCs. Additionally, association of SMTC with location/beam information could help to optimize SMTCs.

Proposal 2: The maximum number of SMTCs per frequency could be 6

2.3 Association of SMTC with location/beam information

As mentioned in the open issue list, the association of SMTC with location/beam information could be discussed. To cover the large cell of NTN, this association would be beneficial for optimizing the usage of SMTC. Based on the discussion in the last meeting, there are several options to achieve this association, such as using the SSB-index, reference location, or cell direction. We prefer the SSB-index based solution since it seems to be the simplest approach from the UE's point of view.  Furthermore, we think SSB-index based solution could work in both idle/inactive and connected state. Therefore, we would like to propose to associate SSB-index with each SMTC.

Proposal 3: SSB-index could be associated with each SMTC for both idle/inactive and connected state

- Summary and proposal

This paper discussed about consideration on cell barring for downlink coverage enhancement and proposed the following:

Proposal 1: RAN2 to introduce a new SMTC field to support multiple SMTC periodicities

Proposal 2: The maximum number of SMTCs per frequency could be 6

Proposal 3: SSB-index could be associated with each SMTC for both idle/inactive and connected state

- References

[1] R2-2501333, “Report from Break-out session on NR-NTN and IoT-NTN”

[2] R2-2502983, “Report from Break-out session on NR-NTN and IoT-NTN”

[3] R2-250xxxx, “Remaining RRC open issues for NR NTN Rel-19”