3GPP TSG-RAN WG2 #130 meeting	R2-2503889

St Julian's, Malta, 19th May – 23rd May, 2025

Source:		NEC

Agenda item:		8.7.6	XR rate control

Title:		Uplink rate control for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#129bis meeting, agreements [1] on XR rate control were made as below:

- Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In this contribution, we will further discuss the remaining issues of XR rate control.

# 2. Discussion

2.1 Recommended bit rate.

When determining the recommended bit rate, only QoS flows that support bit rate adaptation should be considered. Regarding the source of this information for RAN, the following agreements were reached in RAN2#129 meetings.

- Agreements @ RAN2#129 - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. - We may revisit UAI option based on SA2/RAN3 reply

And in RAN3#127bis meeting, RAN3 confirmed that CN will inform gNB which QoS flow(s) are subject to uplink rate control.

- Agreements @ RAN3#127bis - Introduce an indication in NGAP, to allow the SMF to inform the gNB which QoS flow(s) are subject to uplink rate control.

Accordingly, we propose RAN2 to confirm that the gNB receives from the CN the information regarding which QoS flows are subject to uplink rate control. In other words, UAI for this information is not needed.

Proposal 1: RAN2 confirms that the gNB receives from CN the information regarding which QoS flows are subject to uplink rate control.

When the indication of QoS flows supporting bit rate adaptation from CN is available, RAN node should also configure this information to UE. UE then considers only the indicated QoS flows when calculating or querying the desired recommended bit rate in the recommended bit rate query MAC CE.

Proposal 2: RAN node configures UE with information regarding which QoS flows support bit rate adaptation.

According to TS 38.300 [2], 5GC may establish one or more PDU Sessions for a UE, and a QoS flow is identified within a PDU session by a QFI. This means that the same QFI value may be assigned to different QoS flows belonging to different PDU Sessions. Additionally, at Access Stratum level, the NG-RAN maps packets belonging to different PDU sessions to different DRBs. Several QoS Flows belonging to the same PDU session can be multiplexed in the same DRB. Therefore, there are 2 options to uniquely identify a QoS flow:

- Option 1: QFI + PDU Session ID;

- Option 2: QFI + DRB ID.

Regarding the issue of how QFI is indicated in the new recommended bit rate MAC CE, these 2 options could be evaluated. Option 1 requires MAC layer to be awareness of PDU session information, which would impact F1AP, as the CU needs to indicate the PDU session information to the DU. Moreover, compared to 5-bit DRB ID in Option 2, 8-bit PDU Session ID would bring extra overhead over Uu interface. Therefore, we propose RAN2 to adopt Option 2 for the solution.

Proposal 3: [MAC-04] RAN2 agrees to indicate “QFI + DRB ID” in the new recommended bit rate MAC CE.

Different QoS flows may have different preferences for bit rate recommendation granularity. For example, one might favor bit rate recommendation on a per-LCH basis, while another might prefer bit rate recommendation on a per-QoS flow basis. Consequently, UE should inform the network of its preference of bit rate recommendation granularity for a given QoS flow. Based on this, the network can indicate the recommended bit rate in the UE's preferred granularity.

Proposal 4: For a QoS flow, UE informs the network of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

2.2 recommended bit rate during CA/DC.

The data of a QoS flow may be transmitted over different carriers (or LCHs) belonging to the same or different MAC entities when performing PDCP duplication or split transmission. Due to varying congestion situations in different cells, UE may receive different recommended bit rate values for the same QoS flow from different cells or cell groups (e.g., during CA/DC). In this case, RAN2 should discuss how to apply the indicated recommended bit rates.

Figure 1: recommended bit rates from different cells/CGs

For example, UE may be provided with recommended bit rate 1 of a QoS flow from cell 1 and recommended bit rate 2 of the same QoS flow from cell 2.

- For PDCP duplication, the UE may not be expected to go beyond the recommended bit rate of min (recommended bit rate 1, recommended bit rate 2) for the QoS flow.

- For split transmission, the recommended bit rate for a QoS flow may be applied to the cell or cell groups where the MAC CE is received. UE should guarantee that the bit rate of the QoS flow in each cell or cell group will not go beyond the recommended bit rate of the corresponding cell or cell groups.

Proposal 5: [MAC-09] RAN2 considers the following solutions to apply the indicated recommended bit rate for a QoS flow in DC:

- For PDCP duplication, the UE may not be expected to go beyond the recommended bit rate of min (recommended bit rate 1, recommended bit rate 2) for the QoS flow.

- For split transmission, the recommended bit rate for a QoS flow may be applied to the cells or cell groups where the MAC CE is received.

# Summary

This contribution provides our analysis on the uplink rate control for XR, and has the following proposals:

Proposal 1: RAN2 confirms that the gNB receives from CN the information regarding which QoS flows are subject to uplink rate control.

Proposal 2: RAN node configures UE with information regarding which QoS flows support bit rate adaptation.

Proposal 3: [MAC-04] RAN2 agrees to indicate “QFI + DRB ID” in the new recommended bit rate MAC CE.

Proposal 4: For a QoS flow, UE informs the network of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

Proposal 5: [MAC-09] RAN2 considers the following solutions to apply the indicated recommended bit rate for a QoS flow in DC:

- For PDCP duplication, the UE may not be expected to go beyond the recommended bit rate of min (recommended bit rate 1, recommended bit rate 2) for the QoS flow.

- For split transmission, the recommended bit rate for a QoS flow may be applied to the cells or cell groups where the MAC CE is received.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7th Apr – 11th Apr, 2025

[2] TS 38.300, NR; NR and NG-RAN Overall Description, v18.5.0