3GPP TSG-RAN WG2 Meeting #130	R2-2503890

St. Julian, Malta, May 19th – 23rd, 2025

Agenda Item:	8.2.3

Source:	Panasonic

Title: 	Discussion on A-IoT random access procedure

Document for:	Discussion

# Introduction

In RAN2#129bis meeting the following agreements were made for A-IoT random access.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on msg1

- In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

- RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3. May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

This document discusses some of remaining issues on random access procedure, related to configuration aspects of A-IoT paging message and some issues related to random selection of resources from the configuration information.

# Discussion

In the previous meeting, RAN2 has discussed and agreed to introduce a new message in R2D to indicate the start of the Msg1 resources to transmit paging responses in CBRA procedure.

Agreements on new R2D message - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number.

But the configuration aspects regarding what parameters might be required or included in A-IoT paging message is not clear from the current discussions. Alongside, in the last meeting it was also captured as an assumption that, the new R2D message need not indicate slot numbering or count down numbering of the triggered set of resources, then it is not clear how device can identify the (randomly) selected resource for its Msg1 transmission, among a set of resources as it is intended to be configured within A-IoT paging message.

How resources are configured in time domain and frequency domain for the ongoing paging round?

To perform random selection of Msg1 resource in CBRA procedure, since the resource are separated in time domain and frequency domain for devices to randomly select one resource in both time and frequency.

The resources in time domain are further divided into multiple resource sets where each resource set can consist of X domain resources. From RAN1 discussion in the previous meeting, it was decided to support X = 1 or 2 for Rel-19, agreements from RAN1#120bis is shown below.

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s).

But from the above agreements it is not clear whether each R2D message triggering a resource set indicates an X value or is it applicable to all the R2D messages triggered within the same paging round.

Observation 1: For R2D messages triggering resource sets within one paging round (associated with one A-IoT paging message), whether each R2D message can indicate different X values when triggered for each resource set in time domain must be further discussed.

Proposal 1: RAN2 to discuss if the 1 bit to indicate the value of X (X=1 or X=2) will be configured with one of the below configuration options.

Option 1: A-IoT paging message configures X value of each R2D message which triggers a resource set, to keep the size of all the resource sets constant within the same paging round.

Option 2: R2D message which triggers one resource set can indicate the X value, possibly to trigger different X values within the same paging round.

For the resources in frequency domain, the number of frequency resources in the same paging round is sufficient to be kept constant for every R2D message triggered for the resource sets. Hence the configuration of frequency resources of the resource set can be included within the A-IoT paging message.

Proposal 2:  A-IoT paging message configures the frequency domain resources for each resource set triggered within the same paging round. Which means all the R2D messages sent to trigger a resource set in a paging round carries same number of frequency domain resources.

Is the paging round duration or size can be pre-determined and configured?

Then another aspect of configuration in A-IoT paging message is the number of resource sets expected to be triggered in the same paging round, using which the device can randomly select a resource set in time domain to transmit Msg1 in that resource set. Based on the paging service type, the larger the number of devices expected to respond in one paging round the higher number of resource sets necessary to avoid collision or contention among the larger group of devices. RAN2 to discuss if the number of required resource sets can be estimated earlier upon sending A-IoT paging message in R2D, for example using the expected size of the group. In case the estimation of the number of resource sets cannot be pre-determined (for example when the estimated size of the group in close proximity of the reader is not known) then it could be beneficial to extend or shorten the paging round dynamically and not configure or pre-determine at the time of sending A-IOT paging message.

With the above concern, the following proposal should be discussed in RAN2 before other aspects of resource configuration and solutions related to random selection for Msg1 transmission could be addressed.

Proposal 3: RAN2 to discuss if A-IoT paging message configures a fixed number of resource sets in a paging round, or whether the number of resource sets can be varied during the ongoing paging round.

How devices selects and transmits among configured Msg1 resources?

Consider the case where the number of resource sets in a paging round is fixed which could be configured in A-IoT paging message, then the device can randomly select one time domain and frequency domain resource upon receiving the configuration information from the paging message. Consequently, the selected resource must be searched from the corresponding resource set, likewise identify the corresponding R2D messages which triggers a resource set to transmit Msg1 on the selected resource accordingly. This approach has some drawbacks as seen below, in case the countdown numbering of remaining resources or some sort of sequential number of the current resource set is not sent in the R2D message.

- Device needs to monitor the sequencing or track the number of resource set triggered in R2D to identify the selected Msg1 resource slot, which increases device side complexity to store and maintain additional information related to the ongoing paging round.

- Misalignment of resource set triggered in R2D and hence lost opportunity to transmit in the ongoing paging round if the required Msg1 resource is not found due to one or more trigger messages in R2D is failed or not received at the device. Example illustrated in figure 1.

Figure 1: Resource selection misalignment

Observation 2: Resource set sequencing monitored and maintained by the device to search for the selected resource, increases device side complexity and results in missed opportunities to transmit Msg1 in the same paging round, in case of failure of one or more R2D messages.

To avoid the complexity as described above, device can perform random selection upon receiving R2D messages for each set of resources. Here a random number can be generated to make decision if the device can transmit Msg1 in the currently triggered set of resources or shall wait for the next triggered set of resources. Also, for this approach the device need not maintain any additional information related to the ongoing sequence of R2D messages triggering the set of resources. In addition, this approach would not require separate random selection for Msg1 resource selection and congestion control, as to reduce the probability to select currently triggered set of resources is equivalent to congestion control.

Observation 3: More discussion may be necessary to understand the assumption related to not including slot/ count down numbering in R2D message to trigger a set of resources. Or whether the device is expected to maintain sequencing of the received R2D messages to perform random selection and identifying the selected resource among the one or more set of resources.

Proposal 4: RAN2 to discuss and adopt a random selection approach (for Msg1 resource selection) where the device shall not maintain any sequencing of the transmitted messages in R2D (i.e. R2D messages which indicates the start of the set of resources).

# Conclusion

Time domain and frequency domain resource configuration

Observation 1: For R2D messages triggering resource sets within one paging round (associated with one A-IoT paging message), whether each R2D message can indicate different X values when triggered for each resource set in time domain must be further discussed.

Proposal 1: RAN2 to discuss if the 1 bit to indicate the value of X (X=1 or X=2) will be configured with one of the below configuration options.

Option 1: A-IoT paging message configures X value of each R2D message which triggers a resource set, to keep the size of all the resource sets constant within the same paging round.

Option 2: R2D message which triggers one resource set can indicate the X value, possibly to trigger different X values within the same paging round.

Proposal 2:  A-IoT paging message configures the frequency domain resources for each resource set triggered within the same paging round. Which means all the R2D messages sent to trigger a resource set in a paging round carries same number of frequency domain resources.

Number of resource sets in single paging round

Proposal 3: RAN2 to discuss if A-IoT paging message configures a fixed number of resource sets in a paging round, or whether the number of resource sets can be varied during the ongoing paging round.

Random selection approach for the device

Observation 2: Resource set sequencing monitored and maintained by the device to search for the selected resource, increases device side complexity and results in missed opportunities to transmit Msg1 in the same paging round, in case of failure of one or more R2D messages.

Observation 3: More discussion may be necessary to understand the assumption related to not including slot/ count down numbering in R2D message to trigger a set of resources. Or whether the device is expected to maintain sequencing of the received R2D messages to perform random selection and identifying the selected resource among the one or more set of resources.

Proposal 4: RAN2 to discuss and adopt a random selection approach (for Msg1 resource selection) where the device shall not maintain any sequencing of the transmitted messages in R2D (i.e. R2D messages which indicates the start of the set of resources).