3GPP TSG-RAN WG2 Meeting #130	  R2-2503891

St. Julians, Malta, 19th – 23th May, 2025

Agenda item:	8.7.4.1

Source:		Lenovo

Title:			Open issues on Intra-UE prioritization/LCP enhancements

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss remaining open issues on the enhanced Intra-UE prioritization/LCP procedure.

# Discussion

In RAN2#129 the following agreements were reached:

1. Only one additional priority is configured to an LCH for LCP enhancement. 2. We keep an existing agreement (remaining time threshold is configured per LCH) 3. There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. 4. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact 5. FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination 6. No additional PBR is needed for priority adjusted data 7. FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

RAN2#129bis agreements are listed below:

Agreements on LCP enhancements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - We will check the impact of using additional LCP priority for SR priority determination during intra-UE prioritization in specifications and make a final decision on whether to have it based on that, i.e. it should be simple enough. - It should be explicitly specified which priority, e.g. “default” or additional priority, to use for a LCH in order to determine the priority of the corresponding UL grant, i.e. highest priority used/to be used during LCP. - For shared spectrum case, UE shall apply same rules for the LCH priority being used for determining the priority of a HARQ process as for determining the priority of an UL grant. - Understanding in RAN2 is that we will not address any other NR-U specific issues to support XR. - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

## Remaining open issues for LCP procedure

### Impact of priority adjustment on SR priority determination

Similar to the considerations for the LCP procedure, also for the intra-UE prioritization mechanism which was introduced in Rel-16 the remaining time/delay of the data should be considered as a decisive parameter determining the priority of an UL grant. Corresponding agreements have been reached and already implemented for the UL grant prioritization and HARQ process prioritization case in TS38.321.

In RAN2#129bis further discussions on the SR collision case have taken place. It was agreed that whether UE should also consider an additional priority configured for a LCH for determining the priority of a SR in case of a collision with e.g. an UL grant is decided by RAN2 based on the expected specification impact. Basically, if the required changes in specifications are simple, RAN2 should agree to introduce such behaviour. A corresponding TP from Huawei can be seen here:

We agree that the proposed changes to implement the corresponding behaviour look simple. One issue, however, with the suggested TP is that the meaning/definition of the “time of SR transmission” is not that straightforward. One should keep in mind that, whether SR will be transmitted, e.g. on PUCCH, depends on the result of the priority comparison, e.g. whether SR is prioritized or deprioritized. Therefore, when determining whether to adjust the priority of a LCH in order to determine the priority of a SR, UE needs to perform already the comparison of the priority of SR with the colliding UL transmission in order to find out whether SR will be transmitted or not. Maybe it would be more accurate to use the terminology “SR transmission opportunity” rather than referring the actual SR transmission.

Proposal 1: RAN2 to further discuss the detailed UE behaviour/specification wording for considering an additional priority configured for a LCH when determining the priority of a SR in case of a collision.

### Impact of congestion (i.e. PSI-based SDU discard) on priority adjustment

The PDU set importance based PDCP discard has been introduced for Rel-18 XR. In case of UL congestion, PSI based discard can be activated by NW with the consequence that a lower PDCP discard timer value, e.g. discardTimerForLowImportance is applied to the low importance PDCP SDUs.  When PSI based discarding is activated, UE only supports DSR triggering/reporting for high importance data. Following the same principle, when LCP procedure considers the remaining time/delay of data prioritization, the enhancement may also only be applied to high importance data, e.g. delay-critical LCH that has high importance data in its buffers.

Proposal 2: RAN2 to discuss whether the LCP enhancements are only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

### Priority fallback with consideration of PDU Set integrated handling

One open issue raised during the CR review phase is the handling of Priority fallback when PDU Set integrated handling is enabled. More specifically, the question is, whether UE shall consider the remaining Time of a PDU set rather than of a PDU/SDU when adjusting the LCH priority during LCP. The issue was only brought up for the fallback behavior before the 2nd round of LCP, e.g. should the UE fallback to the default LCH priority, if there is no more PDCP SDU with PDCP discard timer remaining value smaller than the threshold, but there is still at least one PDCP SDU with PDU Set Remaining Time smaller than the threshold.

First of all, we think that the same issue could also happen when determining whether to adjust the LCH priority before the 1st round of LCP, i.e. not only for the fallback behaviour. In case of consecutive UL grants, it may for example happen that UE has still some PDUs of a PDU Set in its buffer, for which the remaining Time of the individual SDUs is larger than the Threshold, whereas the corresponding PDU Set remaining Time smaller than the Threshold.

Therefore, RAN2 should discuss the priority adjustment procedure in general in the context of PDU set integrated handling, e.g. should PDU set remaining delay be considered for the adjustment of a LCH priority when PDU Set integrated handling is enabled.

We think that the general agreed principle is that UE adjusts the priority of a LCH if there is delay-critical data pending in the buffer for transmission during LCP. Given that delay-critical data – as defined in TS38.323 – also considers PDU sets, if pdu-SetDiscard is configured, it would make sense in our view that the priority adjustment procedure also considers the PDU Set remaining time rather than the remaining Time of an individual SDU/PDU if pdu-SetDiscard is configured.

On the other hand, the consideration of a PDU Set remaining Time or in general the treatment of PDU Sets in MAC layer requires some additional inter-layer communication and hence implementation complexity, e.g. PDCP needs to inform MAC of a PDU Set remaining Time and further needs to indicate which MAC SDUs belong to a PDU Set, since MAC layer is not aware of PDU Sets.

We don’t have a strong opinion here, but we would prefer that the LCH priority adjustment procedure considers the PDU Set remaining time if pdu-SetDiscard is configured, if this finds a majority support.

Proposal 3: RAN2 to discuss, whether LCH priority adjustment procedure considers PDU Sets/PDU Set remaining time if pdu-SetDiscard is configured during the complete LCP procedure, e.g. not only for priority fallback behaviour before 2nd round of LCP.

# Conclusion

In this contribution, we discuss enhancements to the uplink scheduling. We have the following proposals:

Proposal 1: RAN2 to further discuss the detailed UE behaviour/specification wording for considering an additional priority configured for a LCH when determining the priority of a SR in case of a collision.

Proposal 2: RAN2 to discuss whether the LCP enhancements are only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

Proposal 3: RAN2 to discuss, whether LCH priority adjustment procedure considers PDU Sets/PDU Set remaining time if pdu-SetDiscard is configured during the complete LCP procedure, e.g. not only for priority fallback behaviour before 2nd round of LCP.

# References

### TS 38.321 v18.0.0