3GPP TSG-RAN WG2 Meeting #130	R2-2503901

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda Item	: 8.13.2

Source	: LG Electronics Inc.

Title	: Discussion on discovery and relay reslection

Document for	: Discussion and Decision

1.	Introduction

The work item on NR sidelink multi-hop relay was approved for Rel-19 and the WID is shown in RP-241609 including the following objectives.

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this document, we propose the discovery response message sending condition at the intermediate Relay UE. Also, we investigate how to send the notification message when relay reselection has happened and whether to modify the cause value of the receiving notification before sending it to the child UE.

2.	Discussion

2.1 Whether the intermediate Relay UE can send response message without forwarding solicitation message

According to the SA2 spec (23.304), an intermediate Relay UE can send the response message without forwarding the received solicitation message if the intermediate Relay UE already has a PC5 link. The relevant SA2 spec is as follows:

6.3.2.5.3 Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B <omitted> 3a. If an indication that multi-hop relay is supported is contained in the received Solicitation message, the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1a, of a 5G ProSe Intermediate UE-to-Network Relay and the Target Info matches the User Info ID of the 5G ProSe Intermediate UE-to-Network Relay (if any), the 5G ProSe Intermediate UE-to-Network Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe Intermediate UE-to-Network Relay shall drop the received Solicitation message if the hop count (corresponding to the number of Relays included in the message) has reached the Hop-Limit of the received Solicitation message or the (pre)configured maximum number of hops associated with the RSC. The 5G ProSe Intermediate UE-to-Network Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), without sending Solicitation message. i.e. steps 4a-7a are skipped and step 8a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate UE-to-Network Relay(s). If the same information on User Info IDs of Remote UE and UE-to-Network Relay is received from different ProSe UEs, the 5G ProSe Intermediate UE-to-Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.). <omitted>

RAN2 can follow the same procedure as the SA2 spec for the discovery response message delivery. So, RAN2 can confirm whether the discovery response message can be generated at the intermediate Relay UE, not the last Relay UE, without forwarding the solicitation message toward the last Relay UE, if the intermediate Relay UE already has a PC5 link with its parent UE. When the intermediate Relay UE sends the response message without forwarding the solicitation message, the intermediate Relay UE has to follow the PC5 AS layer condition in the same way as the last Relay UE.

Proposal 1: For discovery model B, if there is an established PC5 link between the intermediate Relay UE and its parent UE, the intermediate Relay UE can send a response message toward the Remote UE without forwarding the solicitation message to its parent UE.

Proposal 2: For discovery model B, if the intermediate Relay UE can generate the response message without forwarding the solicitation message, the intermediate Relay UE needs to check the PC5 AS condition before sending a discovery response message towards the Remote UE.

2.2 Notification message transmission and reception

In the Rel-17 U2N Relay, the Remote UE in RRC_CONNECTED performs RRC reestablishment when receiving the notification message, regardless of the cause value. If the Remote UE is in RRC_IDLE/INACTIVE, it’s up to the implementation of the Remote UE whether to release or keep the PC5 unicast link. The relevant spec description in 38.331 is as follows:

5.8.9.10.4 Actions related to reception of NotificationMessageSidelink message Upon receiving the NotificationMessageSidelink, the Remote UE shall: 1> if the UE is acting as U2N Remote UE: 2> if the indicationType is included: 3> if the UE is L2 U2N Remote UE in RRC_CONNECTED: 4> if MP is configured and MCG transmission (i.e. direct path) is not suspended; 5> if the indicationType is relayUE-HO; 6> suspend indirect path transmission; 5> else: 6> initiate the indirect path failure information procedure as specified in 5.7.3c to report indirect path failure; 4> else if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7; 3> else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE): 4> if the PC5-RRC connection with the U2N Relay UE is determined to be released: 5> indicate upper layers to trigger PC5 unicast link release; 4> else (i.e., maintain the PC5 RRC connection): 5> if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection: 6> consider cell re-selection occurs; NOTE 1: For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link. NOTE 2: The L2 U2N Remote UE may ignore the NotificationMessageSidelink if it does not release the PC5 unicast link in source side yet during an indirect-to-direct path switch, i.e. T304 is running. <omitted>

In the Rel-19 multi-hop relay, when the intermediate Relay UE in RRC_CONNECTED receives a notification message from its parent UE, the intermediate Relay UE will perform the RRC reestablishment procedure regardless of the cause value. If the child UE of the intermediate Relay UE is in RRC_CONNECTED, the child UE will perform the RRC reestablishment procedure. In this case, the cause value doesn’t have any influence on the child UE. Regardless of the cause value, the child UE in RRC_CONNECTED will perform the RRC reestablishment procedure.

If the child UE is in RRC_IDLE/INACTIVE, it’s up to the implementation of the child UE whether to release or keep the PC5 link. The decision is not related to the cause value. Even though the cause value is PC5/Uu RLF, the child UE can decide to keep the link. As long as the intermediate Relay UE tries to make a new PC5 link, it may be worth keeping the link. Anyway, the child UE does not know whether the intermediate Relay UE is performing to make a new PC5 link. In the legacy U2N operation, if the Remote UE in RRC_IDLE/INACTIVE decides to keep the link after receiving the notification message and the cause value is ‘relayUE-HO’ or relayUE-CellReselection’, the Remote UE considers cell re-selection occurs. However, if the Remote UE in RRC_IDLE/INACTIVE decides to keep the link, the Remote UE can check whether the cell re-selection occurs via the SIB message from the Relay UE.

So, we think the notification message can be forwarded without changing the cause value when the intermediate Relay UE receives the notification message. If the child UE in RRC_CONNECTED receives the notification message, the child UE performs RRC reestablishment. If the child UE in RRC_IDLE/INACTIVE receives the notification message, it’s up to the implementation of the child UE whether to keep or release the PC5 link. If the child UE decides to keep the PC5 link, the child UE can check whether to be changed the cell ID via the SIB from the parent UE.

Proposal 3: When the intermediate Relay UE receives a notification message, the intermediate Relay UE can deliver the notification message as it is without changing the cause value.

Proposal 4: When a child UE in RRC_CONNECTED receives a notification message, the child UE performs the RRC reestablishment procedure regardless of the cause value.

Proposal 5: When a child UE in RRC_IDLE/INACTIVE receives a notification message, it’s up to the implementation of the child UE whether to keep or release the PC5 link. The child UE can check whether to change its cell ID via SIB from the parent UE.

2.3 The condition not to send the notification message

If the intermediate Relay UE is in RRC_CONNECTED, the intermediate Relay UE receives an indirect-to-indirect path switching command. If the intermediate Relay UE is in RRC_IDLE/INACTIVE, the intermediate Relay UE performs relay reselection. In these cases, the intermediate Relay UE may have a target Relay UE belonging to the same cell/gNB of the source Relay UE. If the target Relay UE is in the same cell/gNB of the source Relay UE, the intermediate Relay UE may not send a notification message to the child Relay UE(s). The gNB of the target relay UE already has the context of the child UE if the child UE is in RRC_CONNECTED. If the child UE is in RRC_IDLE/INACTIVE, there is no change in terms of cell/gNB ID.

In the discovery model A, the intermediate Relay UE selects a parent UE. The announcement discovery message is forwarded after the intermediate Relay UE establishes the PC5 link with the parent UE. In the discovery model B, the Remote UE selects the whole path to the last Relay UE by using a solicitation and response message.

Proposal 6: If the intermediate Relay UE in RRC_CONNECTED receives an indirect-to-indirect path switching command to the target first Relay UE in the same cell/gNB belonging to the source first Relay UE, the intermediate Relay UE doesn’t need to send a notification message to the child UE.

Proposal 7: If the intermediate Relay UE in RRC_IDLE/INACTIVE performs relay reselection and the target first Relay UE is in the same cell/gNB belonging to the source first Relay UE, the intermediate Relay UE doesn’t need to send a notification message to the child UE.

If the intermediate Relay UE in RRC_CONNECTED receives indirect-to-indirect path switching and the intermediate Relay UE uses discovery model A, the intermediate Relay UE sends the announcement discovery message after establishing the PC5 link with the target first Relay UE. The intermediate Relay UE forwards the discovery announcement message received from the target first Relay UE to the child UE. Since the discovery announcement message includes hop count, the child UEs may update the hop count if the hop count is changed due to the path switching command. We think the discovery announcement message can handle the case of the hop count change. The hop count handling via the discovery announcement message can be applied to the case of relay re-selection occurrence at the intermediate Relay UE.

Proposal 8: In the discovery model A, the announcement discovery message may handle the hop count change if the hop count is changed due to the indirect-to-indirect path switching command or relay re-selection occurrence.

If the multi-hop path is composed by discovery model B, the intermediate relay UE acting as Remote UE selects the whole path from the intermediate Relay UE and the last Relay UE. If the intermediate Relay UE performs path switching or relay reselection, the path information will be changed. The changed path information should be delivered to the child UE. It’s an upper layer realm of how to inform the changed path information. If the child UE receives the message including the changed path information and the changed path is available to the child UE, the child UE may maintain the current link. Otherwise, the child UE releases the PC5 link. Whether to keep or release the link is decided by the upper layer.

Proposal 9: In the discovery model B, RAN 2 can ask to the SA2 how to handle the path information when the path information is changed (including the case hop count is changed).

In the control plane, two approaches are discussed, i.e., the basic approach and the fast/parallel approach. The SRAP header for SRB0 delivery is different for each approach. So, if two approaches are acceptable, we need to consider that some UEs only support the basic approach, but others support both. The Remote UE trying to establish a fast connection needs to know whether the candidate Relay UE can support the fast/parallel approach. That information may be included in the discovery message.

Proposal 10: RAN2 may need to discuss whether the discovery message should include which approaches (basic approach (approach 1), fast/parallel approach (approach 2), or both) are supported.

3.	Conclusion

Proposal 1: For discovery model B, if there is an established PC5 link between the intermediate Relay UE and its parent UE, the intermediate Relay UE can send a response message toward the Remote UE without forwarding the solicitation message to its parent UE.

Proposal 2: For discovery model B, if the intermediate Relay UE can generate the response message without forwarding the solicitation message, the intermediate Relay UE needs to check the PC5 AS condition before sending a discovery response message towards the Remote UE.

Proposal 3: When the intermediate Relay UE receives a notification message, the intermediate Relay UE can deliver the notification message as it is without changing the cause value.

Proposal 4: When a child UE in RRC_CONNECTED receives a notification message, the child UE performs the RRC reestablishment procedure regardless of the cause value.

Proposal 5: When a child UE in RRC_IDLE/INACTIVE receives a notification message, it’s up to the implementation of the child UE whether to keep or release the PC5 link. The child UE can check whether to change its cell ID via SIB from the parent UE.

Proposal 6: If the intermediate Relay UE in RRC_CONNECTED receives an indirect-to-indirect path switching command to the target first Relay UE in the same cell/gNB belonging to the source first Relay UE, the intermediate Relay UE doesn’t need to send a notification message to the child UE.

Proposal 7: If the intermediate Relay UE in RRC_IDLE/INACTIVE performs relay reselection and the target first Relay UE is in the same cell/gNB belonging to the source first Relay UE, the intermediate Relay UE doesn’t need to send a notification message to the child UE.

Proposal 8: In the discovery model A, the announcement discovery message may handle the hop count change if the hop count is changed due to the indirect-to-indirect path switching command or relay re-selection occurrence.

Proposal 9: In the discovery model B, RAN 2 can ask to the SA2 how to handle the path information when the path information is changed (including the case hop count is changed).

Proposal 10: RAN2 may need to discuss whether the discovery message should include which approaches (basic approach (approach 1), fast/parallel approach (approach 2), or both) are supported.

4.	Reference

- RP-241609, Approved WID: NR sidelink multi-hop relay, LG Electronics Inc., InterDigital, FirstNe..