3GPP TSG-RAN WG2 #130	R2-2503919

St Julian, Malta, May 19th – 23rd, 2025

Agenda item:	   8.1.2.3

Source:		Lenovo

Title:	LCM for AIML based positioning with UE-sided model

Document for:		Discussion and Decision

# Introduction

In this paper we discuss the 1st priority use case for LCM for AIML based positioning with UE-sided model, i.e., case 1.

- (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

# Discussion

## Separate positioning methods for case1 and case 3a/3b

- RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

In the previous RAN2 meeting, it was agreed that AI positioning will be a new method. In the meanwhile, the discussion at that time focused on case 1 which is downlink based, while case 3a/3b are uplink based positioning using AI technique. Case 1 and Case3a/3b overall procedures are very different. For the specification implementation, RAN2 is suggested to distinguish Case 1 and Case3a/3b as two different AIML based positioning methods, namely DL-AIML Positioning and UL-AIML Positioning methods.

RAN2 is suggested to distinguish Case 1 and Case3b (FFS 3a) as two different AIML based positioning methods, namely DL-AIML Positioning and UL-AIML Positioning methods.

## Applicability reporting

In the applicability reporting discussion in AIML based beam management use case, both Option A and Option B are supported wherein:

- Option A: NW configures multiple sets of inference configurations to UE, and UE replies to NW if any of them are applicable or inapplicable.

- Option B: NW configures multiple sets of inference related parameters to UE, and UE replies to NW if any of them are applicable or inapplicable. Then NW further provide the inference configuration to UE which is expected to be applicable.

For AIML based positioning case 1, RAN2 has made the following relevant agreements, which are more or less follow the similar principle as for AIML based beam management use case.

The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control.

On the other hand, it needs to be clarified whether the Option A-like or Option B-like design will be also supported in AIML based positioning. To be more specific:

- Option A-like: LMF will configure UE with one or multiple sets of configurations (e.g., different sets of NR-DL-PRS-AssistanceData configurations) for the same DL-AIML positioning method. Then, in the LPP provide location information message, UE will report the UE location estimation using the one that is applicable and reply to LMF the relevant applicability information.

In our view, the Option A-like approach is supported by the LPP request location information and LPP provide location information messages with only one set of configuration (e.g., NR-DL-PRS-AssistanceData), since the outcome of the AIML based positioning is only the UE location. Comparatively, in AIML based beam management, UE can be configured to perform different types of predictions (time domain, spatial domain).

RAN2 is suggested to confirm that only one set of inference configuration is configured to UE for AIML based positioning.

- Option B-like: LMF and UE exchanges the applicable parameters related to the inference before LMF requests the UE location estimation via LPP request location information message.

This is supported by two means:

- Option B-1:

  - LMF sends LPP provide assistance information containing one set of NR-DL-PRS-AssistanceData

  - UE replies in LPP provide capabilities message, if the provided NR-DL-PRS-AssistanceData is applicable or not.

- Option B-2:

  - Via the existing on demand PRS transmission procedure, the LMF can configure UE with a list of PRS configurations (nr-On-Demand-DL-PRS-Configurations) via LPP provide assistance data message, and UE can request a particular PRS configuration (NR-On-Demand-DL-PRS-Request) via LPP request assistance data message.

  - On the other hand, the PRS configuration is considered as part of the assistance data (NR-DL-PRS-AssistanceData) that is needed for UE to determine the applicability.

One set of NR-DL-PRS-AssistanceData is provided in LPP provide assistance information message in legacy.

Legacy on-demand PRS transmission procedure supports transfer of applicable PRS configuration from multiple sets, however the PRS configuration parameter is only part of the parameters (e.g., element within NR-DL-PRS-AssistanceData) for UE to determine the applicability.

RAN2 is suggested to clarify if multiple sets of NR-DL-PRS-AssistanceData can be provided in the LPP provide assistance information message, such that UE can report the (in)applicability information in the LPP provide capabilities message or request the applicable ones via the LPP request assistance data message.

## Error cause values

Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-LocationServerErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.) Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-TargetDeviceErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

The error cause values generally provide an indication that certain actions/behaviour could not be fulfilled or certain information may be missing to perform a certain requested action, which reported to the peer entity/node, e.g., from LMF to target UE or from UE to LMF.

#### NR-DL-AI-ML-LocationServerErrorCauses

The IE NR-DL-AI-ML-LocationServerErrorCauses is used by the location server to provide NR AI/ML positioning error reasons to the target device.

-- ASN1START

NR-DL-AI-ML-LocationServerErrorCauses-r19 ::= SEQUENCE {

cause-r19		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

notProvidedAssistanceDataNotSupportedByServer,

on-demand-dl-prs-NotSupportedByServer,

on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer,

...

},

...

}

-- ASN1STOP

Editor's Note: The above list of error causes is FFS.

Table 1 shows the possible error causes required from a Location Server (LMF) perspective for AI/ML positioning with an associated motivation behind the error cause:

Table 1: List of Location Server (e.g., LMF) Error Causes

| Error Cause | Motivation |
| --- | --- |
| undefined | May be re-used to cover undefined error causes |
| assistanceDataNotSupportedByServer | May be re-used in similar fashion to UE-based legacy techniques to indicate if certain AD is not supported by LMF |
| assistanceDataSupportedButCurrentlyNotAvailableByServer | May be re-used in similar fashion to UE-based legacy techniques to indicate that AD is supported but not readily available at LMF |
| notProvidedAssistanceDataNotSupportedByServer | May be re-used in similar fashion to UE-based legacy techniques to indicate not providing AD is not supported by LMF |
| on-demand-dl-prs-NotSupportedByServer | May be re-used in similar fashion to UE-based legacy techniques since support of on-demand PRS is also extended to AI/ML Positioning. |
| on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer | May be re-used in similar fashion to UE-based legacy techniques since support of on-demand PRS is also extended to AI/ML Positioning. |
| labelNotAvailable | An error cause may also be provided to the UE in the event that there is an unavailability of ground truth or label information for various reasons. |

For location server error (NR-DL-AI-ML-LocationServerErrorCauses), a new cause value “labelNotAvailable” can be added for the scenario that UE requests ground truth from LMF for training while LMF cannot provide.

#### NR-DL-AI-ML-TargetDeviceErrorCauses

The IE NR-DL-AI-ML-TargetDeviceErrorCauses is used by the target device to provide NR AI/ML positioning error reasons to the location server.

-- ASN1START

NR-DL-AI-ML-TargetDeviceErrorCauses-r19 ::= SEQUENCE {

cause-r19						ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceived,

locationCalculationAssistanceDataMissing,

...

},

remoteUE-Indication-r18			ENUMERATED {true}						OPTIONAL,	-- Cond NR

...

}

-- ASN1STOP

Table 2 shows the possible error causes required from a UE perspective for AI/ML positioning with an associated motivation behind the error cause

Table 2: List of UE/Target Device Error Causes

| Error Cause | Motivation |
| --- | --- |
| undefined | May be re-used in a similar fashion to legacy to cover undefined error causes |
| assistance-data-missing | May be re-used in a similar fashion to legacy to indicate if certain AD missing at UE |
| unableToMeasureAnyTRP | May be re-used in a similar fashion to legacy to indicate that the UE was unable to perform measurements on any TRP, e.g., serving TRP |
| attemptedButUnableToMeasureSomeNeighbourTRPs | May be re-used in a similar fashion to legacy to indicate that UE attempted to measure but could not perform a measurement of neighbouring TRP(s) |
| thereWereNotEnoughSignalsReceived | May be re-used in a similar fashion to legacy to indicate that not sufficient PRS was received to perform a AI-ML positioning measurement for Case 1. Suggest wording change to “thereWereNotEnoughSignalsReceivedForUeBasedAI-ML“ to emphasize that it is relevant for AI/ML UE-based pos- case. |
| locationCalculationAssistanceDataMissing | May be re-used in a similar fashion to legacy to indicate that the one or more location calculation AD is missing. |
| aIMLModelNotAvailable | May be used to indicate that training is not yet complete/valid. May also provide an indication to Location Server that assistance data needs to be provided again as a subsequent Location Server behaviour upon receiving this error cause. |
| methodTemporarilyNotApplicable | An indication the AI/ML based positioning Case 1 method becomes non-applicable (e.g., due to UE side condition such as capacity/battery issues) when LMF requests UE location estimation. The AIML method may become applicable later. |
| unabletoPerformAIMLPositioning | An indication that the AI/ML positioning cannot be performed. Location Server is expected to de-configure the AIML positioning method. To be checked if this is covered by the “requestedMethodNotSupported” in the CommonIEsProvideLocationInformation. |
| aIMLPerformanceMonitoringOutcomeNotAvailable | An indication that UE AI-ML Performance Monitoring result in unavailable if requested by the Location Server. |

For target device error (NR-DL-AI-ML-TargetDeviceErrorCauses), the following new cause values can be added:

  - aIMLModelNotAvailable

  - methodTemporarilyNotApplicable

  - unabletoPerformAIMLPositioning

  - aIMLPerformanceMonitoringOutcomeNotAvailable

## 2.4	Positioning accuracy monitoring

RAN1 Agreement Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded. Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

RAN1 also discussed the monitoring of AI-based positioning for case 1, and at least (Option A) target UE can perform the monitoring metric calculation and may transmit the outcome to the LMF. In our understanding, there are two possible scenarios:

- Option A initiated by UE: based on UE’s own demand, UE may perform monitoring by itself. UE may need LMF to provide related configuration or assistance information. UE will not send the monitoring outcome to LMF, instead UE may indicate to LMF if the AI-based positioning become non-applicable (e.g. via an Error cause)

  - This can be supported by LPP Request Assistance Data and Provide Assistance Data messages.

- Option A initiated by LMF: LMF configures UE to perform monitoring and send the monitoring outcome back to LMF.

  - This can be supported by LPP Request Location Information and Provide Location Information messages.

(Monitoring Option A initiated by UE) LPP RequestAssistanceData and ProvideAssistanceData messages are used for UE to request LMF to provide relevant configuration/data for monitoring metric calculation at UE without sending the outcome to LMF.

(Monitoring Option A initiated by LMF) LPP RequestLocationInformation and ProvideLocationInformation messages are used for LMF to configure the UE to calculate monitoring metric and send the outcome to LMF.

## 2.5	UE requested training data collection

RAN1 Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB

In the previous RAN1 discussion, RAN1 has considered using the following two options to provide necessary DL PRS configuration. And in our view, both can be supported based on legacy LPP Assistance Data Transfer procedure (RequestAssistanceData/ProvideAssistanceData) or LPP Assistance Data Delivery Procedure (ProvideAssistanceData)

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option A, UE can send a RequestAssistanceData message to LMF requesting on-demand DL PRS configuration, including relevant information, e.g., time duration, PRS characteristics. Then LMF will provide the demanded DL PRS configuration to UE via ProvideAssistanceData message.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option B, LMF could provide the DL PRS configuration to UE via ProvideAssistanceData message directly.

The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request NR-DL-PRS-AssistanceData from LMF for training data collection.

The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide NR-DL-PRS-AssistanceData to UE for training data collection.

RAN1#120 Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

Also, RAN1 supports the scenario where the label data of location is generated by LMF and transferred from LMF to UE in the DL. However, there is no existing LPP procedure can be directly reused for such purpose. Therefore, RAN2 is suggested to discuss if LMF can transfer the label data of location to UE via existing LPP message (e.g., ProvideAssistanceData) or a new LPP message proactively/reactive.

RAN2 discusses if LMF can transfer the label data of location to UE via existing LPP message (e.g., ProvideAssistanceData) or a new LPP message proactively/reactively.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	One set of NR-DL-PRS-AssistanceData is provided in LPP provide assistance information message in legacy.

Observation 2	Legacy on-demand PRS transmission procedure supports transfer of applicable PRS configuration from multiple sets, however the PRS configuration parameter is only part of the parameters (e.g., element within NR-DL-PRS-AssistanceData) for UE to determine the applicability.

Based on the discussion above, we propose:

Proposal 1	RAN2 is suggested to distinguish Case 1 and Case3b (FFS 3a) as two different AIML based positioning methods, namely DL-AIML Positioning and UL-AIML Positioning methods.

Proposal 2	RAN2 is suggested to confirm that only one set of inference configuration is configured to UE for AIML based positioning.

Proposal 3	RAN2 is suggested to clarify if multiple sets of NR-DL-PRS-AssistanceData can be provided in the LPP provide assistance information message, such that UE can report the (in)applicability information in the LPP provide capabilities message or request the applicable ones via the LPP request assistance data message.

Proposal 4	For location server error (NR-DL-AI-ML-LocationServerErrorCauses), a new cause value “labelNotAvailable” can be added for the scenario that UE requests ground truth from LMF for training while LMF cannot provide.

Proposal 5	For target device error (NR-DL-AI-ML-TargetDeviceErrorCauses), the following new cause values can be added:

a.	aIMLModelNotAvailable

b.	methodTemporarilyNotApplicable

c.	unabletoPerformAIMLPositioning

d.	aIMLPerformanceMonitoringOutcomeNotAvailable

Proposal 6	(Monitoring Option A initiated by UE) LPP RequestAssistanceData and ProvideAssistanceData messages are used for UE to request LMF to provide relevant configuration/data for monitoring metric calculation at UE without sending the outcome to LMF.

Proposal 7	(Monitoring Option A initiated by LMF) LPP RequestLocationInformation and ProvideLocationInformation messages are used for LMF to configure the UE to calculate monitoring metric and send the outcome to LMF.

Proposal 8	The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request NR-DL-PRS-AssistanceData from LMF for training data collection.

Proposal 9	The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide NR-DL-PRS-AssistanceData to UE for training data collection.

Proposal 10	RAN2 discusses if LMF can transfer the label data of location to UE via existing LPP message (e.g., ProvideAssistanceData) or a new LPP message proactively/reactively.

# Reference