3GPP TSG-RAN WG2 #130	R2-2503932

St. Julian's, Malta, 19th – 23rd May 2025

Source:		NEC

Agenda item:		8.10.2	MRO enhancements for Rel-18 mobility features

Title:	MRO enhancement for LTM

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, for MRO of LTM, the following agreements have been made [1]:

Agreements - We will not add support for including the unnecessary LTM configuration, e.g. LTM candidate cell(s) unused, in SON reports, for both failure and near failure cases. - Do not specify support for reporting difference between the TA value received from the LTM cell switch command and TA value measured by the UE - Do not specify support for reporting of elapsed time between UE obtains the target cell TA and UE receives the cell switch command, or an indication to indicate whether the UE obtains TA before UE receives LTM cell switch command - For LTM-related RA reports, the UE sets the cellId to the CGI if available, otherwise sets it based on ltm-CandidatePCI and ssb-Frequency in corresponding LTM-Candidate.

In this paper, we provide our consideration on how to optimize the case of LTM failure due to invalid/outdated TA.

- Discussion

A failure of a LTM cell switch procedure may be caused by several reasons, for example due to wrong beam or due to invalid/outdated TA, and MRO enhancement is being discussed for these two cases in both RAN2 and RAN3. In this paper, we mainly discuss on the case of LTM failure cause due to invalid/outdated TA.

For LTM failure due to invalid TA/outdated TA, the case is that the UE gets an invalid/outdated TA in the LTM cell switch command, and therefore fails to access to the target cell. And the UE later performs LTM recovery to the same target cell successfully after the failure, and the TA provided by the target network in RAR during the LTM recovery procedure is different from the TA received in the LTM cell switch command. For this case, maybe the target DU is possible to detect the TA difference between the TA triggered by early sync and TA during the successful LTM recovery, but target DU is not aware of whether the source DU sent TA in the LTM cell switch command or not. Additional signalling from CU to DU regarding this for each LTM cell switch is not a good solution. Therefore, the network is not aware of the occurrence of invalid/outdated TA without additional information from the UE side, and information collected at the UE side is perfered.

There are two options of the additional information from the UE to the network in RLF report

- Option 1: TA value obtained during the LTM recovery procedure

- Option 2: The difference between the TA value in the cell switch command and the TA value obtained during the LTM recovery procedure

In our understanding, both options can work. Considering that reporting the TA difference can be useful even for the case that the TA in the LTM cell switch command is not stored at the network side, we think Option 2 is better than Option 1.

Proposal 1: In case of LTM cell switch failure, UE includes in RLF report the difference between the TA value in the cell switch command and the TA value obtained during the successful LTM recovery procedure.

# 3. Conclusion

In this paper, we discuss on how to optimize the case of LTM failure due to invalid/outdated TA, and have the following proposal:

Proposal 1: In case of LTM cell switch failure, UE includes in RLF report the difference between the TA value in the cell switch command and the TA value obtained during the successful LTM recovery procedure.

# Reference

[1] Meeting report of RAN2 #129bis