3GPP TSG-RAN WG2 Meeting #130	R2-2504010

St.Julians, Malta,  May 19th – 23rd , 2025

Agenda item:		8.8.2 Downlink coverage enhancements

Title:		Remaining issues on DL CE in NR NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 103th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

1. Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for downlink coverage enhancements in NTN.

Agreements: 1. With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work. 2. We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements. 2. RAN2 assumes that both EFC and EMC are supported

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) 3. (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

During the 128th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

During the 129th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer 2. RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame 3. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) 4. The number of SMTC/gaps a UE needs to consider at any time will not be increased further.

During the 129bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits 2. We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring 3. RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). 4. We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) 5. Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In this paper, we would like to provide our views on the highlighted aspects as well as remaining issues on downlink coverage enhancements in NTN.

# 2. Discussion

2.1	SMTC impacts due to beam hopping

During the 129th RAN WG2 meeting, it was agreed that the network may configure more than four SMTCs per frequency (e.g., up to six) for UEs in RRC_IDLE and RRC_INACTIVE states. Furthermore, it was also agreed that for Rel-19 UEs, the network may provide assistance information (not limited to UEs supporting DL coverage enhancement) to help UEs perform SMTC selection in these modes. This contribution further addresses the following remaining issues to enable accurate and efficient SMTC selection in NR-NTN idle/inactive modes:

- Maximum number of SMTCs that needs to be configured.

- Details of location information

- SMTC window adjustment

The maximum number of SMTCs for idle/inactive mode

Figure 1 illustrates two deployment scenarios—clustered and scattered cell illuminations—where more than four SMTCs are required to ensure proper SSB monitoring without overlap. In the clustered-cell scenario, where active cells form spatial clusters and beams within each cluster are synchronized, up to six SMTC windows may still be required when multiple clusters have differing activation times. In the scattered-cell scenario, where active cells transmit SSBs independently, the number of required SMTCs may also need six SMTCs.

Figure 1. Clustered and Scattered-cells illumination scenarios

Proposal 1: RAN2 to agreed that the maximum number of SMTCs that can be configured for UEs in RRC_IDLE and RRC_INACTIVE modes should be increased to six.

Details of location information for SMTC selection in idle/inactive mode

Given that UEs are typically allowed to monitor only up to four SMTCs at a time, a selection mechanism is needed when more than four SMTCs are configured. While a simple approach would be to use RSRP-based SMTC selection, this method might not be unreliable in NTN environments due to the following factors:

- Minimal RSRP variation within wide satellite beams,

- Minimal spatial power distribution caused by free-space propagation,

- Ambiguous beam boundaries and overlapping coverage areas.

Consequently, RSRP does not serve as a reliable indicator for SMTC selection.

Observation 1: RSRP may be not a reliable metric for SMTC selection in NR-NTN.

Proposal 2: RAN2 to introduce a location information-based SMTC selection mechanism to assist or complement RSRP-based selection for UEs in idle/inactive mode.

Two location information-based approaches have been discussed during the last RAN WG2 meeting:

- Option 1: Reference location-based SMTC selection

- Option 2: Serving cell’s SSB index-based SMTC selection

In Option 1, each SMTC is associated with a reference location (e.g., geographic coordinate or area ID) and an optional distance threshold. The UE acquires its location (e.g., via GNSS), and when initiating neighbour cell measurements, selects SMTCs whose reference location falls within the configured distance threshold. This approach enables:

- Accurate filtering of geographically relevant SMTCs,

- Reduced power consumption by avoiding unnecessary measurements,

In Option 2, the UE selects SMTCs based on the SSB index (or beam ID) of the serving cell, assuming that the SSB index implicitly reflects UE location. However, this method presents several challenges in NTN:

- Many NTN cells may have only one SSB index per PCI, limiting spatial granularity.

- Overlapping beam coverage may result in the same SSB index being visible to UEs in different locations.

- SSB index selection is implementation-specific and does not always reflect UE location accurately, especially in RSRP environments with minimal variation.

Observation 2: SSB index-based SMTC selection lacks robustness in NTN due to limited granularity, ambiguous beam overlaps, and minimal signal variation.

Proposal 3: RAN2 to introduce a reference location-based SMTC selection as the preferred mechanism for idle/inactive mode SMTC selection.

SMTC window adjustments

In legacy NR-NTN, SMTC window adjustment mechanism was introduced to account for propagation delay differences caused by multiple satellites. To support this, the UE estimates its location (e.g., via GNSS).

Before evaluating the need for such SMTC window adjustment in beam hopping operations, the following neighbouring cell scenarios should be considered:

- Case 1: All neighbouring cells are formed by multiple beams from a single satellite.

- Case 2: Some neighbouring cells are formed by beams from different satellites.

In Case 1, multiple SMTCs for neighbouring beams from a single satellite have fixed window timing offsets from the UE’s perspective. Therefore, adjustment may not be needed.

Observation 3: SMTC window adjustment, as introduced in legacy NR-NTN, may not be necessary for neighbouring beams from a single satellite.

In Case 2, as illustrated in Figure 2, a serving cell may have neighbouring cells from multiple satellites. SSBs from neighbouring cells of the same satellite share the same propagation delay, while those from different satellites may differ. Without location-based SMTC window adjustment, the UE may not receive SSBs from these neighbouring cells.

Figure 2. Example of SMTC configurations with no SMTC window adjustment

Observation 4: Without location-based SMTC window adjustment, the UE may fail to receive SSBs from neighbouring beam-hopping cells originating from other satellites.

Proposal 4: RAN2 to consider omitting SMTC window adjustment for neighbouring beam-hopping cells from the serving satellite, while applying SMTC window adjustment—similar to legacy NR-NTN—for neighbouring beam-hopping cells from different satellites.

# 3. Conclusions

Proposal 1: RAN2 to agreed that the maximum number of SMTCs that can be configured for UEs in RRC_IDLE and RRC_INACTIVE modes should be increased to six.

Observation 1: RSRP is not a reliable metric for SMTC selection in NR-NTN.

Proposal 2: RAN2 to introduce a location information-based SMTC selection mechanism to assist or complement RSRP-based selection for UEs in idle/inactive mode.

Observation 2: SSB index-based SMTC selection lacks robustness in NTN due to limited granularity, ambiguous beam overlaps, and minimal signal variation.

Proposal 3: RAN2 to introduce a reference location-based SMTC selection as the preferred mechanism for idle/inactive mode SMTC selection.

Observation 3: SMTC window adjustment, as introduced in legacy NR-NTN, may not be necessary for neighbouring beams from a single satellite.

Observation 4: Without location-based SMTC window adjustment, the UE may fail to receive SSBs from neighbouring beam-hopping cells originating from other satellites.

Proposal 4: RAN2 to consider omitting SMTC window adjustment for neighbouring beam-hopping cells from the serving satellite, while applying SMTC window adjustment—similar to legacy NR-NTN—for neighbouring beam-hopping cells from different satellites.