3GPP TSG-RAN WG2#130 											      R2-2504033

Malta, MT, May. 19th – 23rd, 2025

Agenda Item:	8.8.5

Source: 	NEC

Title:	        Stage 2 Updates for Regenerative Payload

Document for:	 Discussion

# Discussion

Mobility between regenerative mode and transparent mode satellites

UE barely can distinguish regenerative mode and transparent mode, and we also concluded that UE would support regenerative mode without any additional capability. Straightforward, UE shall support mobility between regenerative mode and transparent mode, we propose to make it clear as same as for the mobility between different orbits.

#### 16.14.3.2 Mobility in RRC_CONNECTED ##### 16.14.3.2.1 Handover The same principle as described in 9.2.3.2 applies unless hereunder specified: During mobility between NTN and Terrestrial Network (TN), a UE is not required to connect to both NTN and TN at the same time. NOTE: NTN TN handover refers to mobility in both directions, i.e. from NTN to TN (hand-in) and from TN to NTN (hand-out). DAPS handover is not supported for NTN in this release of the specification. UE may support mobility between gNBs operating with NTN payloads in different orbits (e.g., GSO, NGSO at different altitudes). RACH-less handover as specified in 9.2.3.6, in TS 38.321 [6] and in TS 38.331 [12] is supported in NTNs.

Proposal 1: clarify UE supports mobility between gNBs operating with transparent and regenerative NTN payloads in section 16.14.3.2

Satellite switch with re-synchronization

RAN2 has discussed several times on supporting satellite switch with re-synchronization in regenerative mode, but could not agree, because satellite switch means gNB changes in regenerative mode, and much more work must be done especially in RAN3. At this point, we can confirm that it will not be supported in Rel19, and hence it should be clarified the section below is only for transparent mode:

##### 16.14.3.2.3 Satellite switch with re-synchronization Upon both hard and soft satellite switch over in the quasi-Earth fixed scenario with the same SSB frequency and the same gNB, the satellite switch with re-synchronization procedure is supported. The satellite switch with re-sync avoids a L3 mobility for UEs in the cell by maintaining the same PCI on the geographical area covered by quasi-Earth fixed beam. For soft satellite switch over, the UE can start synchronizing with the target satellite before the source satellite ends to serve the cell. It is not required for the UE to be connected to source satellite when the UE switches to target satellite. For hard satellite switch over, the UE can only start synchronizing with the target satellite after the switch to the target satellite is initiated. When both CHO and Satellite switch with re-synchronization are configured, it is up to UE implementation which procedure to initiate, if both of them are triggered simultaneously. For the re-synchronization to the target satellite, random access can be triggered by a PDCCH order via the target

Proposal 2: clarify in section 16.14.3.2.3 that satellite switch with re-synchronization is only for transparent mode