3GPP TSG-RAN WG2#129bis											   R2-2504035

Malta, MT, May. 19th – 23rd, 2025

Agenda Item:	8.8.2

Source: 	NEC

Title:	        Details on SMTC enhancement

Document for:	 Discussion

# Introduction

This document addresses remaining details regarding agreed SMTC enhancements.

# Discussion

As follows, smtc4list-r17 is used for SMTC configuration in a NTN cell, where maximum 4 can be configured. All configured STMCs share the same periodicity (which is provided by SSB-MTC). Each SMTC can be configured with different offsets and linked to a PCI list.

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

smtc4list-r17                          SSB-MTC4List-r17                OPTIONAL,   -- Need R

SSB-MTC4List-r17::=                    SEQUENCE (SIZE(1..3)) OF SSB-MTC4-r17

SSB-MTC4-r17 ::=                         SEQUENCE {

pci-List-r17       EQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL,  -- Need M

offset-r17          INTEGER (0..159)}

Two SMTC enhancements were tentatively agreed

- SMTC selection out of Maximum 6 configured:

 We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking);  Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

UE will select not more than 4 out of 6 SMTCs based on assistance information provided by network. There are two popular options regarding the type of assistance information:

- Option 1: reference locations. UE selects SMTC based on distance between UE and the reference locations.

Reference location

With option1, network provides every neighboring cell’s reference location as above.

In case more than one neighboring cells’ SSB periodicity and offset are aligned/closed to each other, measurements on them can be covered by one SMTC. It should be allowed to associate one SMTC with multiple neighboring cells’ reference locations. Because one SMTC is also associated with a list of PCIs, we can either configure one reference location per PCI or a list of reference locations per SMTC.

Considering overlapped TN network measurement, it should also be allowed that one SMTC is not associated with any reference location. In this case, UE shall not down select the SMTC

- Option 2: serving cell's SSB index. UE choose which SMTC to measure based on the selected SSB of serving cell (i.e. best SSB).

With option 2, the network provides SMTC configuration per SSB (i.e. per serving beam) rather than per cell.

It should be allowed to associate the same SMTC with two different SSB, e.g., in case one neighboring cell is close to both SSB beams or two different neighboring cells’ measurement can be covered by the same SMTC.

Both options increase SMTC configuration granularity in either neighboring cells perspective or serving cell perspective:  Option 1 have the SMTC configuration per neighboring cell rather than per neighboring frequency. Option 2 have the SMTC configuration per serving beam rather than per serving cell.

We believe both have pros and cons, e.g. option1 could potentially reduce the number of selected SMTC in both NTN scenarios including one beam in one cell, and multiple beams in one cell. This option can work well for homogenous NTN cell deployment scenario but may not for other scenarios; option 2 simply extend per serving cell to per serving beam configuration, having better generalization to other types of beams forming deployments and having better future compatibility. However, in one beam-one cell NTN scenarios, this option does not provide gain:

RAN2 adopt one of following two options:

- Option1: introduce reference locations to be associated to the configured SMTCs.

  - Option1a: a list of reference locations (0, 1 or multiple) is associated to a SMTC.

  - Option 1b: 0 or 1 reference location is associated to each PCI in the PCI list linked to a SMTC.

- UE shall always select the SMTCs without reference location.

- Option2: introduce per SSB SMTC configuration, maximum 4 SMTC per SSB.

We believe that network still need to adopt the solution “Configure one SMTC with small periodicity, in each period (SMTC occasion) up to UE implementation to measure a subset PCIs)” to support Pre Rel-19 UE and Rel19 UE does not supporting SMTC selection.

RAN2 confirm that we need the implementation option as follows for UE not supporting SMTC selection, legacy smtc4list-r17 should be kept for this purpose, and independent smtc6withRefLocationlist-r19 or SSBsmtc4list-rel19 should be introduced.

- “Configure Maximum 4 SMTC with small periodicity to cover all beam-hopping neighboring cells, in each period (SMTC occasion) up to UE implementation to measure a subset PCIs)”

We agreed that SMTC selection out of maximum 6 SMTCs is for idle and inactive mode only.

RAN2 confirm SMTC selection out of maximum 6 SMTCs is only for idle and inactive mode UEs, and no need of capability bit.

- Two Different SMTC periodicity:

 RAN2 considers supporting configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode:

Two different SMTC periodicities were agreed for both idle/inactive and connected modes.

For RRC connected mode, because maximum 6 SMTC is not supported, simply we could introduce an additional SMTC periodicity (different from periodicity provided by SSB-MTC) and associated with the existing SMTCs in smtc4list.

For RRC connected mode, introduce an Additional SMTC periodicity, and associate to a subset of all configured SMTCs (form smtc4list-r17), other SMTCs is then associated to the legacy periodicity.

Introduce one capability bit to indicate support of additional SMTC periodicity.

For RRC idle and inactive mode UE, considering network and UE respectively may or may not support maximum 6 SMTCs and may or may not support extra SMTC periodicity. It starts to be complicated for network to configure the extra SMTC periodicity properly, define the association with both smtc4list-r17 and rel-19 SMTC configurations, and make both UEs supporting or not supporting SMTC selection works. On the other hand, both SMTC section (out of maximum 6 SMTC) and extra SMTC periodicity solutions are both motivated due to beam hopping/SSB extension, one solution is enough for RRC idle and inactive mode.

For RRC idle and inactive mode, RAN2 discuss not to support additional SMTC periodicity.

# Conclusion

In summary, we would like to share our view on downlink coverage enhancement with following proposals:

- RAN2 adopt one of following two options:

- Option1: introduce reference locations to be associated to the configured SMTCs.

  - Option1a: a list of reference locations (0, 1 or multiple) is associated to a SMTC.

  - Option 1b: 0 or 1 reference location is associated to each PCI in the PCI list linked to a SMTC.

- UE shall always select the SMTCs without reference location.

- Option2: introduce per SSB SMTC configuration, maximum 4 SMTC per SSB.

RAN2 confirm that we need the implementation option as follows for UE not supporting SMTC selection, legacy smtc4list-r17 should be kept for this purpose, and independent smtc6withRefLocationlist-r19 or SSBsmtc4list-rel19 should be introduced.

•	“Configure Maximum 4 SMTC with small periodicity to cover all beam-hopping neighboring cells, in each period (SMTC occasion) up to UE implementation to measure a subset PCIs)”

RAN2 confirm SMTC selection out of maximum 6 SMTCs is only for idle and inactive mode UEs, and no need of capability bit.

For RRC connected mode, introduce an Additional SMTC periodicity, and associate to a subset of all configured SMTCs (form smtc4list-r17), other SMTCs is then associated to the legacy periodicity.

Introduce one capability bit to indicate support of additional SMTC periodicity.

For RRC idle and inactive mode, RAN2 discuss not to support additional SMTC periodicity.