3GPP TSG-RAN WG2 Meeting #130	R2-2504038

St.Julian, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.4

Source:	vivo

Title:	Discussion on adaptation of common signal transmissions

Document for:	Discussion and Decision

Introduction

In the RAN2#129bis meeting, RAN2 has made the following agreements.

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

In this contribution, we will discuss the remaining open issues on paging adaptation and RACH adaptation in time domain.

Discussion

## 2.1 Paging adaptation in the time domain

In this section, we will discuss the following open issues:

- Signaling overhead optimization of configuring Rel-19 firstPDCCH-MonitoringOccasionOfPO;

- Whether to introduce a separate pei-ConfigBWP for paging adaptation.

Signaling overhead optimization of configuring Rel-19 firstPDCCH-MonitoringOccasionOfPO:

In the legacy firstPDCCH-MonitoringOccasionOfPO configuration, each PO (first PDCCH MO of the PO) can be configured on any symbol between two adjacent PFs. Thus, the maximum signaling overhead of legacy firstPDCCH-MonitoringOccasionOfPO is 4*17 = 68 bits, given by the configuration shown below.

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

In Rel-19 NES, N = T/32, Ns = 8 are supported. Follow the legacy logical, the following table 1 analyzes the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO, assuming N = T/32, Ns = 8.

Table 1: signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO

| SCS | Total OFDM symbols per radio frame | Total OFDM symbols across T (T = 32) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (Ns = 8) |
| --- | --- | --- | --- |
| 15KHz | 140 | 4480 (13 bits) | 104 bits |
| 30KHz | 280 | 8960 (14 bits) | 112 bits |
| 60KHz | 560 | 17920 (15 bits) | 120 bits |
| 120KHz | 1120 | 35840 (16 bits) | 128 bits |
| 480KHz | 4480 | 143360 (18 bits) | 144 bits |

It can be observed that the maximum signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO configuration exceeds 2 times that of the legacy. Additionally, it should be noted that both the legacy configuration and the Rel-19 configuration can be configured at the same time, meaning only firstPDCCH-MonitoringOccasionOfPO configuration would occupy 68 + 144 =  212 bits, and the maximum SIB1 size is only 2976 bits. We think such signaling overhead is unacceptable and should be optimized.

TS 38.331 NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

In fact, the value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO corresponding to N = 1/32T does not need to span the whole 32 radio frames, considering the target scenario is for NES. Otherwise, the network can simply configure N = 1/16T or 1/8T etc. Therefore, one solution is to restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO. Table 2 summarized the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO if different maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO is used.

Table 2: signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO for different maximum value range

| SCS | Total OFDM symbols per radio frame | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 32 frames) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 16 frames) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 2 frames) |
| --- | --- | --- | --- | --- |
| 15KHz | 140 | 104 bits | 96 bits | 72 bits |
| 30KHz | 280 | 112 bits | 104 bits | 80 bits |
| 60KHz | 560 | 120 bits | 112 bits | 88 bits |
| 120KHz | 1120 | 128 bits | 120 bits | 96 bits |
| 480KHz | 4480 | 144 bits | 136 bits | 112 bits |

Another approach is to sacrifice some unnecessary flexibility in case of NES by having the network configure evenly spaced POs. For example, the network only needs to configure the starting PO position among 8 POs and the interval between them. Both the starting PO position and the interval require up to 18 bits from the overhead listed in Table 1 for 480KHz, thus the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO can be limited to 36 bits. Given the above, we have the following proposal:

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, RAN2 to discuss the following options:

- Option 1: restrict the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N);

- Option 2: support configuring evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them.

Whether to introduce a separate pei-ConfigBWP for paging adaptation?

In the current RRC Running CR, a separate PEI-Config-r17 is introduced.

There is an Editor’s note in the CR:

FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

In the existing PEI scheme, the UE determines its PEI-O based on both PEI-Config-r17 and pei-ConfigBWP-r17 configurations, where PEI-Config-r17 provides a reference point that is a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, and pei-ConfigBWP-r17 provides a symbol-level offset from the reference point to the start of the first PDCCH MO of PEI-O. In case of Rel-19 NES, if Ns = 8 is configured, all 8 POs belongs to a single PF and will be linked to a single reference time (pei-FrameOffset-r17) and 1~4 PEI-O(s) which is based on the configured po-NumPerPEI, please note that po-NumPerPEI cannot be set to 1, since the maximum of PEI-O per PF (maxPEI-perPF-r17) is 4. And the corresponding PDCCH MO of 1~4 PEI-O(s) is according to the pei-ConfigBWP-r17. Therefore, to determine the PEI-O of Rel-19 PO, both PEI-Config-r17 and pei-ConfigBWP-r17 configurations are required.

Proposal 2: Introduce a separate pei-ConfigBWP for paging adaptation. FFS whether to introduce a new maxPEI-perPF-r19.

Moreover, similar signaling overhead issue exists also for firstPDCCH-MonitoringOccasionOfPEI-O-r17, thus the similar handling of Rel-19 firstPDCCH-MonitoringOccasionOfPO can be considered for firstPDCCH-MonitoringOccasionOfPEI-O-r17.

Proposal 3: If P1 is agreed, similar handling of Rel-19 firstPDCCH-MonitoringOccasionOfPO is also applied to Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O-r17.

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}

pei-FrameOffset Offset, in number of frames from the start of a reference frame for PEI-O to the start of a first paging frame of the paging frames associated with the PEI-O, see TS 38.213 [13], clause 10.4A.

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

## 2.2 RACH adaptation in the time domain

RAN1 has made the following agreements related to DCI-based PRACH adaptation and sent an LS R2-2503312 to RAN2 in the last RAN1 meeting. As requested, RAN2 needs to confirm whether the use of one bit in short message is feasible.

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Agreement

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

Agreement

- For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI

- The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

- (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})

It seems no other WI requires to use of these bits in the short message. In addition, using one bit in short message to indicate the additional PRACH resources are available has no impact for the RRC_IDLE /INACTIVE/CONNECTED UE monitoring SI change notification and ETWS/CMAS notification, thus, we think it is feasible.

Proposal 4：It is feasible to use one bit from the Bits 5-8 within the Short Message field for DCI-based adaptation for additional PRACH resources for RRC_IDLE/INACTIVE/CONNECTED UEs.

In the current specification, for RRC_CONNECTED UEs configured with common search space on the active BWP to monitor paging, if the UE is capable of ETWS/CMAS, the UE shall monitor PWS notification at least once every defaultPagingCycle; otherwise, shall monitor SI change indication at least once per modification period.

UEs in RRC_IDLE or in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging.

Based on RAN1 agreements above, the additional RACH resources can be available at any frame that contains PO in the current modification period. As the use of additional RACH resources by UEs in RRC_IDLE/INACTIVE state has significantly mitigated RACH congestion, we think it is sufficient to require the RRC_CONNECTED UEs to monitor additional RACH resources availability indication at least once per modification period. If the UEs in RRC_CONNECTED are interested in the additional RACH resources, it’s up to UE whether to monitor the indication more frequently.

Proposal 5：UEs in RRC_CONNECTED monitor for additional RACH resources availability indication in any paging occasion at least once per modification period if the UE is provided with common search space on the active BWP to monitor paging.

Conclusion

Based on the discussion, we have the following proposals:

Adaptation of paging occasions in the time domain

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, RAN2 to discuss the following options:

- Option 1: restrict the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N);

- Option 2: support configuring evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them.

Proposal 2: Introduce a separate pei-ConfigBWP for paging adaptation. FFS whether to introduce a new maxPEI-perPF-r19.

Proposal 3: If P1 is agreed, similar handling of Rel-19 firstPDCCH-MonitoringOccasionOfPO is also applied to Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O-r17.

Adaptation of RACH adaptation in the time domain

Proposal 4：It is feasible to use one bit from the Bits 5-8 within the Short Message field for DCI-based adaptation for additional PRACH resources for RRC_IDLE/INACTIVE/CONNECTED UEs.

Proposal 5：UEs in RRC_CONNECTED monitor for additional RACH resources availability indication in any paging occasion at least once per modification period if the UE is provided with common search space on the active BWP to monitor paging.