3GPP TSG-RAN WG2 Meeting #130                              R2-2504046

St.Julians, Malta, May 19th–23rd, 2025

Agenda Item:	8.9.2

Source: 	Transsion Holdings

Title:  	Discussion on support of Store&Forward

Document for:	Discussion and Decision

Introduction

In the last RAN2#129b meeting, the support of Store&Forward for IoT-NTN was discussed, and the following agreements were made[1]:

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current "S&F operation mode" to "normal mode" (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

3.	We don’t introduce any new release cause related to S&F operation

In this contribution, we provide our views on these issues to proceed the work item.

Discussion

Transition time information between satellite S&F mode and normal mode

- In last meeting it has been agreed that an indication in system information for normal mode to S&F mode transition will be introduced, FFS on the details (e.g. whether we can link this to other existing information. From our point of view, there is no need to introduce new IE to indicate this, the time information provided in SIB31 as agreed in last meeting can achieve this. When the S&F operation indication is not broadcast, it means the satellite is work in normal mode, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from normal mode to S&F operation, when the S&F operation indication is broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from S&F operation to normal mode.

- Proposal 1: When the S&F operation indication is not broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from normal mode to S&F operation, while when the S&F operation indication is broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from S&F operation to normal mode.

- Proposal 2: If the remaining time from normal mode to S&F mode is not enough to finish the service, the UEs not support S&F mode may not triggered to access the network.

Paging enhancement in satellite Store&Forward mode

- As the satellite has already stored the downlink data for UE in advance, it means the network knows which UE needed to be paged and the paging demand is not random anymore and as the large coverage of the satellite, the number of UEs needed to be paged and the amount of downlink data needed to be send by the satellite maybe huge, which may cause paging congestion.

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 3: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- The satellite in S&F mode can distribute the paging message in time to avoid congestion from network’s perspective, however the UE has to monitor the PF/POs in every DRX cycle no matter whether there is a paging for the UE or not and there are no more paging message from the core network as there is no feeder link between the satellite and the ground, so the paging send by satellite and paging monitoring by UE can be optimized for power saving. For example there can be two DRX parameters which is different for S&F mode and normal mode, the DRX cycle or nB for S&F mode can be set larger than those in normal mode. Moreover if the UE keeps monitoring paging for some cycle or some times or some duration and if no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

- Proposal 4: Two different Paging parameters can be configured for S&F mode and normal mode.

- Proposal 5: UE keeps monitoring paging for some cycle or some times or some duration and if no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

UP CIoT EPS optimisation support in satellite Store&Forward mode

For UP mode, when UE resume the RRC connection with a eNB, if the eNB is not the old eNB which suspend the connection, the eNB has to fetch the context from the old eNB. However for satellite in S&F mode, as the fast moving satellite and the no ISL between the satellite, the eNB the UE trys to resume may not the old eNB and can’t get the context from the old eNB, the UE has to fallback to the RRC connection setup procedure. To avoid this, it has to make sure that the eNB the UE resume has the UE context. To achieve this, the context can be transfer to specific satellites and UE only resume the connection to this satellites. As to how to decide these satellites, there can be two methods: one is the UE itself determines the satellites based on the satellite assistance information and the UE traffic pattern and send the satellite list to the network, then the netwrok confirms some or all of them and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered. The other method is the UE provide some assistant information such as UE location and traffic pattern to the network and the network determines the satellite list and then transfer/distribute the context to these satellites via the network on ground when the feederlink recovered.

- Proposal 6: When UP mode is used, the UE context can be transfered/distributed to specific satellites via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

When UE resume the RRC connection to a new eNB, the security key used by the new eNB is generated by the old eNB using the old key or NH、the target PCI and target EARFCN-DL. For S&F mode, if the satellite list the UE will resume are determined, the security key used by the new eNB in the satellite list in the future can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list. The generated security key are associated with each of the satellite in the satllite list and will be transfered to these satellites in advance via the ground network when the feeder link recovered.

- Proposal 7: The security key used by the new eNB can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list and transfered to these satellties via the ground network.

Conclusion

In this contribution we discussed issues related to S&F mode in IoT-NTN and made the following proposals:

- Observation 1  The paging demand maybe not random anymore and the paging and downlink data needed to be send by network maybe huge in S&F mode which may cause paging congestion.

- Proposal 1: When the S&F operation indication is not broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from normal mode to S&F operation, while when the S&F operation indication is broadcast, the time information in SIB31 can be interpreted as the time when the current satellite operation will transit from S&F operation to normal mode.

- Proposal 2: If the remaining time from normal mode to S&F mode is not enough to finish the service, the UEs not support S&F mode may not triggered to access the network.

- Proposal 3: RAN2 to consider the paging congestion issue caused by the non-randomized and huge paging demand.

- Proposal 4: Two different Paging parameters can be configured for S&F mode and normal mode.

- Proposal 5: UE keeps monitoring paging for some cycle or some times or some duration and if no paging for the UE is received or the UE has monitored the paging for the UE, the UE stop monitoring paging in the rest time when the current satellite is in S&F mode.

- Proposal 6: When UP mode is used, the UE context can be transfered/distributed to specific satellies via the network on ground when the feederlink recovered and the UE only resumes connecton to these satellites. How the satelllites are determined is FFS.

- Proposal 7: The security key used by the new eNB can be generated advance by the old eNB using the parameters(e.g. PCI、EARFCN-DL) associated with the determined satellite list and transfered to these satellties via the ground network.

Reference

- “Draft_RAN2_129bis_Meeting_Report_v3”