3GPP TSG-RAN WG2 Meeting #130	R2-2504048

St. Julien, Malta, 19th – 23rd May 2025

| Agenda Item: | 8.6.2 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Remaining issues on inter-CU LTM |
| Document for: | Discussion and Decision |

1.	Introduction

For inter-CU LTM, this paper discusses the remaining issues on inter-CU LTM from three aspects, i.e., 1) cell switch completion procedure, 2) secondary security update in inter-CU MCG LTM, and 3) LTM based recovery. Specifically the following issues are discussed:

- FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message

- FFS whether the list of sk-Counters can be used only for the case of inter-MN LTM

- FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration.

- FFS if fast failure recovery with different Rel-19 IDs is allowed

- FFS whether or not to keep the LTM configuration as result of the LTM recovery based on inter-CU candidate cell

2.	Discussion

2.1.	Cell switch completion in inter-CU LTM

In the post-129bis email discussion [115], the following open issue is noted:

2.2 RRC-2 Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message. Rapporteur comment: Whether this is an issue need to be clarified, but probably good to discuss this based on companies’ contributions (if companies think that current text is not okay).

In the latest RRC running CR, the setting the content of the RRCReconfigurationComplete message for LTM cell switch completion is implemented as follows:

5.3.5.3 Reception of an RRCReconfiguration by the UE The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch: […unrelated texts are omitted…] 1> set the content of the RRCReconfigurationComplete message as follows: […unrelated texts are omitted…] 2> if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6: 3> if this RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup or via SRB3: 4> include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration; 3> else if this RRCReconfiguration message was received via SRB1 but not within the nr-SCG within mrdc-SecondaryCellGroup: 4> if a new sk-Counter value has been selected due to the LTM cell switch execution procedure: 5> include selectedSK-Counter and set its value to the selected sk-Counter value; Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message. […unrelated texts are omitted…]

In Rel-18 LTM procedure, the appliedLTM-CandidateId is included in the RRCReconfigurationComplete message regardless of whether the LTM cell switch is triggered on MCG or SCG. In the latest RRC running CR for Rel-19, on the other hand, the appliedLTM-CandidateId is included in the RRCReconfigurationComplete message only when the LTM cell switch is triggered on SCG based on the green-highlited part. In our view, RAN2 have never discussed this. By the way, we don’t see any motivation to include the appliedLTM-CandidateId into the RRCReconfigurationComplete message only for the SCG LTM case. Further, it is simple to include the appliedLTM-CandidateId into the RRCReconfigurationComplete message for any LTM case

Proposal 1. For both MCG and SCG RRCReconfigurationComplete message, the appliedLTM-CandidateId is included.

In the procedural text in the running CR, the selectedSK-Counter is included in the RRCReconfigurationComplete message only when the LTM cell switch is triggered on MCG based on the cyan-highlighted part. In our view, this is weird implementation because the selectedSK-Counter needs to be included in to the RRCReconfigurationComplete message for the case of SCG LTM with secondary key update. In addition, it is not necessaty to distinguish whether the LTM cell switch is triggered on MCG or SCG in order to include the selectedSK-Counter into the RRCReconfigurationComplete message. Hence, the selectedSK-Counter is included into the RRCReconfigurationComplete message if a new sk-Counter value has been selected due to the LTM cell switch execution procedure regardless of whether the LTM cell switch is triggered on MCG or SCG.

Proposal 2. The selectedSK-Counter is included into the RRCReconfigurationComplete message if a new sk-Counter value has been selected due to the LTM cell switch execution procedure regardless of whether the LTM cell switch is triggered on MCG or SCG.

Based on Proposal 1 and Proposal 2, TP1 is provided in Annex.

Proposal 3. Adopt TP1 if Proposal 1 and Proposal 2 are agreed.

2.1.	Security update for inter-CU MCG LTM with DC

Regarding the security update for inter-CU MCG LTM with DC, the following open issue is noted:

2.7 RRC-7 Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM. Rapporteur comment: This is something that we never discussed. Current Running CR re-use the list of sk-counters also for the case of inter-MN LTM (to support subsequent LTM) so our suggestion is to keep current text.

To be specific, the provision of sk-Counter for SN security update during inter-CU MCG LTM with DC (i.e., SN is not changed) is implemented as follows in the latest RRC running CR:

5.3.5.18.6 LTM cell switch execution Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall: […unrelated texts are omitted…] 1> if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 1> if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID: 2> if the LTM cell switch is triggered on the MCG: 3> update the master security key by performing the AS security key update procedure as specified in 5.3.5.7; 3> if the LTM-Candidate IE indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release: 4> consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChangeID according to the field ltm-NoSecurityChangeID as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7; 4> remove the selected sk-Counter value from the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChangeID; Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM. […unrelated texts are omitted…]

Based on the latest RRC running CR given above, the UE uses the first value of sk-Counter in the list sk-Counter associated with the corresponding ltm-NoSecurityChangeID for SN security update during inter-CU MCG LTM with DC (i.e., SN is not changed). However, the current procedural text in the RRC running CR is not aligned with the agreement RAN2 made in the meeting RAN2#127:

For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

The agreement given above (i.e., RAN2#127 agreement) means that the legacy (i.e., Rel-15) SN security update procedure is re-used for the case where CU is acting as MN and SN is unchanged. Note that the Rel-15 SN security update procedure is always performed if sk-Counter is included in RRCReconfiguration-v1560-IEs of the RRCReconfiguration message.

Specifically in legacy, the network sends to the UE the MN RRCReconfiguration message including the new MN security configuration and the single counter value (i.e., sk-Counter) for updating the SN security if the MN security update is needed due to, e.g, MN handover while the SN connection is maintained before/after MN handover. To this end, the SN RRCReconfiguration message including sk-Counter is embedded in the MN RRCReconfiguration message, i.e., sk-Counter is provided within nrSCG within MRDC-SecondaryCellGroupConfig included into MN RRCReconfiguration message. Upon receiving the RRCReconfiguration message, the UE updates the MN security based on the new MN security configuration (i.e., new KgNB). Subsequently, the UE updates the SN security based on the new KgNB and sk-Counter (i.e. new S- KgNB). To sum up, the SN security key update is based only on the provision of sk-Counter in the case of MN handover with keeping the existing SN connection.

For LTM cell switch, one or more candidate cell configurations are provided to the UE beforehand. Upon receiving an LTM cell switch command (CSC) MAC CE from the network, the UE performs the LTM cell switch toward the candidate cell indicated by the LTM CSC MAC CE, i.e., the UE first applies the stored candidate cell configuration and subsequently the UE accesses the candidate cell based on the stored candidate cell configuration and the LTM CSC MAC CE. In order to update the SN security with legacy Rel-15 RRC reconfiguration with sync procedure in the case of inter-CU MCG LTM while the SN connection is maintained, the pre-configured candidate cell configuration of MCG LTM target cell should include the single counter value, i.e., sk-Counter, for updating SN security.

In post-129bis email discussion [115], it is observed that some companies also think the current text in the running CR is not aligned with the agreement. Since it is not desirable to revert the previous agreement unless it causes confusion or conflict, we argue the following proposal for clarification.

Proposal 4. RAN2 confirm that a single value of sk-Counter is provided within RRCReconfiguration-v1560-IEs of the RRCReconfiguration message to the UE for the SN key update in inter-CU MN LTM with unchanged SN (i.e., follow the legacy R15 RRC reconfiguration with sync procedure).

In RAN2#125bis meeting, RAN2 agreed the following:

Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

With the above agreement, subsequent intra-CU MCG LTM cell switches after inter-CU MCG LTM cell switch can happen as decribed in [1]. Based on the legacy Rel-15 RRC reconfiguration with sync procedure, whether to perform the SN security update during MN RRC reconfiguration (e.g., MN handover) does not depend on whether the MN security update is performed but rather on whether sk-Counter is provided as shown below.

5.3.5.3 Reception of an RRCReconfiguration by the UE […unrelated texts are omitted…] 1> if the RRCReconfiguration includes the sk-Counter: 2> perform security key update procedure as specified in 5.3.5.7; […unrelated texts are omitted…]

This means that the legacy Rel-15 RRC reconfiguration with sync procedure cannot prevent redundant SN security update during subsequent intra-CU MCG LTM cell switches.

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured. Network doesn't include this field for DRB if the bearer is configured as DAPS bearer, if the RadioBearerConfig IE is part of an RRCReconfiguration message within the LTM-Config IE, or if the RadioBearerConfig IE is part of an RRCReconfiguration message associated with subsequent CPAC within the ConditionalReconfiguration IE

Based on the field description of reestablishPDCP given above, PDCP reestablishment is followed by security key update. This implies that redundant SN security update during subsequent intra-CU MCG LTM cell switches results in redundant PDCP reestablishment at the PDCP entities associated with SN. Consequently, the UE undergoes data interruption due to redundant PDCP reestablishment.

Observation 1. Redundant SN security update during subsequent intra-CU MCG LTM cell switches can result in redundant PDCP reestablishment at the PDCP entities associated with SN and consequently data interruption can occur.

To avoid the data interruption due to redundant SN security update during subsequent LTM cell switches, SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) is performed only if MN security update is performed during an MCG LTM cell switch.

Proposal 5. SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) is performed only if MN security update is performed during an MCG LTM cell switch.

In order to implement Proposal 5, the TP2 or TP3 in Annex can be considered.

Proposal 6. RAN2 consider TP2 or TP3 as an implementation of Proposal 5.

2.3.	LTM based recovery for inter-CU LTM cell switch failure

About LTM based recovery for inter-CU LTM cell switch failure, the following open issues are noted:

2.6 RRC-6 Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration. Rapporteur comment: It is still not crystal clear how the fast recovery procedure would work, and especially when the UE should revert back to the previous PCell configuration. We suggest to discuss this based on companies’ contribution. 2.8 RRC-8 FFS if fast failure recovery with different Rel-19 IDs is allowed. Rapporteur comment: This is an open issue from last RAN2 meeting and, even if we don’t support the case mentioned in the FFS, things should work fine anyway. However, our suggestion is to discuss this based on companies’ contribution.

First, we discuss the open issue RRC-6. In the meeting RAN2#129bis, the following is agreed for fast recovery in inter-CU LTM scenario:

At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Based on the agreement, the UE is allowed to perform fast recovery only when the selected cell’s R19 ID is equal to the Rel-19 ID with the target cell, thereby avoiding new security update. However, upon expiry of timer T304 after the inter-CU LTM failure, it is unclear whether the UE should revert to the source PCell configuration or retain the target cell configuration to support the fast recovery.

Even though retaining the target cell configuration helps avoid unnecessary security updates, if the revert-back step is skipped and the fast recovery fails, the UE may fail to perform RRC Re-establishment. In this case, the UE must perform the Re-establishment procedure on the source cell configuration that the network expects. Otherwise, the network may reject the RRC re-establishment request due to mismatched UE identity.

Thus, determining when and how to revert back is essential for the recovery. We identify two potential methods for performing the revert-back step:

Method 1: Skipping revert back upon T304 expiry and revert back during RRC Re-establishment

The Method 1 means the UE retains the target cell configuration until RRC Re-establishment is initiated after the fast recovery failure. If RRC Re-establishment is initiated after the fast recovery failure or the selected cell is not associated with the R19 ID of the target cell, the UE then performs the revert back to the previous PCell configuration which was successfully used (i.e. source cell configuration before the fast recovery trial).

The Method 1 has pros that it enables fast recovery without switching back to the old configuration unnecessarily.

The Method 1 has cons that, if the selected cell does not belong to the same R19 ID as the target cell or the fast recovery fails, the UE must revert back while initiating RRC Re-establishment and it makes specification change complicated.

Method 2: Immediate revert back upon T304 expiry

The Method 2 means the UE always reverts back to the source PCell configuration upon T304 expiry as like the current principle.

The Method 2 has pros that it is simpler and less complicated behavior than the Method 1.

The Method 2 has cons that the UE need to remember the target cell’s R19 ID due to the revert back step. Also, the UE may need to apply the cell configurations twice (to source first, then again for fast recovery), which seems inefficient and potentially contradicts the purpose of fast recovery without security update.

However, the first issue may be mitigated because the UE can easily remember the R19 ID by referring association with the candidate cell ID from the LTM cell switch command. As for the second issue, it is arguably not observable from the network side, meaning it may not introduce actual problem.

Based the above statements, because both Methods are feasible, we propose to discuss the revert back behaviour based on the following two options.

Proposal 7. RAN2 to conclude when the UE shall revert back after inter-CU LTM cell switch failure, based on the following options:

- Option 1: The UE doesn’t revert back to the source PCell configuration upon T304 expiry if the LTM cell switch procedure required security key refresh (i.e. inter-CU LTM). The UE reverts back to the previously connected PCell configuration if the selected cell is not associated with the R19 ID of the target cell.

- Option 2: The UE reverts back to the source PCell configuration except for the state variables upon T304 expiry.

Based on the RRC running CR [2], the procedural text for Option 1 and Option 2 can be implemented, where Option 1 and Option 2 correspond to Method 1 and Method 2 respectively.

Regarding Method 1 (i.e. Option 1), the current RRC running CR already implies that the UE does not perform the revert back upon T304 expiry of the inter-CU LTM. This is illustrated by the behavior highlighted below:

2> else: 3> if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution as described in 5.3.5.18.6 and 5.3.5.18.x: 4> if the LTM cell switch execution procedure does not require a security key refresh: 5> revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG; // if the LTM cell switch execution procedure requires a security key refresh, the UE doesn’t revert back to the UE configuration used in the source PCell.

Accordingly, during the RRC Re-establishment procedure, if the selected cell while T311 is running is not one of the candidate cells belonging to the same R19 ID as the target cell, the UE shall perform reverting back to the prevously connected PCell configuration.

Regarding Method 2 (i.e., Option 2), the UE always reverts back to the source PCell configuration upon T304 expiry, regardless of whether the failed LTM cell switch is for either intra-CU LTM or inter-CU LTM. Later, during RRC Re-establishment, the UE checks whether the newly selected cell’s R19 ID is equal to the R19 ID of the target cell. The current RRC running CR already implies that the UE refers to the candidate cell previously provided in the LTM cell switch command to check target cell’s R19 ID. This behavior is described as the following highlighted part:

2> if the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6 and the seleted cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected: 3> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

Based on above, we provide the TPs for both options in Annex, i.e., TP4 for Option 1 and TP5 for Option 2.

Proposal 8. RAN2 to consider TP4 (Option 1) and TP5 (Option 2) as an implementation of Proposal 7.

Now, we discuss the open issue RRC-8, i.e., FFS if fast failure recovery with different Rel-19 IDs than the target cell is allowed. In order to allow fast failure recovery with different Rel-19 IDs than the target cell, the following requirements may arise: 1) provision of an NCC value for fast recovery in addition to the NCC value provided by the LTM CSC MAC CE for inter-CU LTM cell switch; and 2) early key distribution from the target CU to other involving CUs before LTM cell switch completion. In other words, in order to perform fast recovery with different Rel-19 IDs than the target cell, the UE needs to be provided in advance an additional NCC value for additional security update and the target CU should distribute the new key to involving CUs.

For the first requirement, an additional NCC value can be signaled by the new LTM CSC MAC CE. However, it is not convinced that these kinds of enhancements are needed for fast failure recovery. Furthermore, LTM based fast recovery for all Rel-19 IDs can be achieved through the early new key distribution on network side (i.e., the second requirement). This means that whether or not to support LTM based fast recovery for all Rel-19 IDs depends more on RAN3's opinion than RAN2's. That is, it is up to RAN3 discussion whether to support the second requirement. In our view, RAN3 basically assumes that new key distribution is performed by the path switch procedure after successful LTM cell switch completion. The early key distribution from the target CU to other involving CUs before LTM cell switch completion (e.g., upon Cell Switch Notification) may increase the opportunity of fast recovery but it seems a non-essential optimization. In Rel-19, RAN3 should focus on the basic functionality of network signaling for inter-CU LTM support. Hence, we propose not to allow the fast failure recovery to a candidate cell with different Rel-19 IDs than the target cell.

Proposal 9. The fast failure recovery to a candidate cell with different Rel-19 IDs than the target cell is NOT allowed.

There is an open issue that RAN2 have not been discussed before, which is FFS whether or not to keep the LTM configuration as result of the LTM recovery based on inter-CU candidate cell. As shown below in Rel-18, the UE does not remove the LTM configuration after successful recovery because LTM can still work after LTM-based recovery.

5.3.7.3 Actions following cell selection while T311 is running […] 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6; NOTE 2: In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute. […]

Similar to Rel-18 LTM-based recovery, the UE keeps the LTM configuration after successful LTM-based recovery.

Proposal 10. UE keeps the LTM configuration as result of the LTM recovery based on inter-CU candidate cell.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Inter-CU LTM cell switch completion

Proposal 1. For both MCG and SCG RRCReconfigurationComplete message, the appliedLTM-CandidateId is included.

Proposal 2. The selectedSK-Counter is included into the RRCReconfigurationComplete message if a new sk-Counter value has been selected due to the LTM cell switch execution procedure regardless of whether the LTM cell switch is triggered on MCG or SCG.

Proposal 3. Adopt TP1 if Proposal 1 and Proposal 2 are agreed.

Security update for inter-CU MCG LTM with DC

Proposal 4. RAN2 confirm that a single value of sk-Counter is provided within RRCReconfiguration-v1560-IEs of the RRCReconfiguration message to the UE for the SN key update in inter-CU MN LTM with unchanged SN (i.e., follow the legacy R15 RRC reconfiguration with sync procedure).

Observation 1. Redundant SN security update during subsequent intra-CU MCG LTM cell switches can result in redundant PDCP reestablishment at the PDCP entities associated with SN and consequently data interruption can occur.

Proposal 5. SN security update during subsequent MCG LTM cell switches with DC (i.e., SN is not changed) is performed only if MN security update is performed during an MCG LTM cell switch.

Proposal 6. RAN2 consider TP2 or TP3 as an implementation of Proposal 5.

LTM based recovery for inter-CU LTM cell switch failure

Proposal 7. RAN2 to conclude when the UE shall revert back after inter-CU LTM cell switch failure, based on the following options:

- Option 1: The UE doesn’t revert back to the source PCell configuration upon T304 expiry if the LTM cell switch procedure required security key refresh (i.e. inter-CU LTM). The UE reverts back to the previously connected PCell configuration if the selected cell is not associated with the R19 ID of the target cell.

- Option 2: The UE reverts back to the source PCell configuration except for the state variables upon T304 expiry.

Proposal 8. RAN2 to consider TP4 (Option 1) and TP5 (Option 2) as an implementation of Proposal 7.

Proposal 9. The fast failure recovery to a candidate cell with different Rel-19 IDs than the target cell is NOT allowed.

Proposal 10. UE keeps the LTM configuration as result of the LTM recovery based on inter-CU candidate cell.

4.	References

[1] R2-2500136 Discussion on remaining issues of inter-CU LTM, LG Electronics, RAN2#129

[2] [POST129bis][115][MOB] RRC inter-CU and CLTM (Ericsson)

5. Annex

5.1.	TP1

5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

[…unrelated texts are omitted…]

1>	set the content of the RRCReconfigurationComplete message as follows:

[…unrelated texts are omitted…]

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

3>	if a new sk-Counter value has been selected due to the LTM cell switch execution procedure:

4>	include selectedSK-Counter and set its value to the selected sk-Counter value;

Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

[…unrelated texts are omitted…]

5.2.	TP2

5.3.5.3	Reception of an RRCReconfiguration by the UE

[…unrelated texts are omitted…]

- if the RRCReconfiguration includes the sk-Counter:

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE in ltm-Config indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

3>	if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID:

4>	if the LTM cell switch is triggered on the MCG:

5>	if the LTM-Candidate IE indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release:

6>	perform security key update procedure as specified in 5.3.5.7;

2>	else:

3>	perform security key update procedure as specified in 5.3.5.7;

[…unrelated texts are omitted…]

5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

[…unrelated texts are omitted…]

1>	if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

1>	if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID:

2>	if the LTM cell switch is triggered on the MCG:

3>	update the master security key by performing the AS security key update procedure as specified in 5.3.5.7;

Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

[…unrelated texts are omitted…]

5.3.	TP3

5.3.5.3	Reception of an RRCReconfiguration by the UE

[…unrelated texts are omitted…]

1>	if the RRCReconfiguration includes the sk-Counter:

2> if this RRCReconfiguration message is applied due to other than an LTM cell switch execution procedure:

3>	perform security key update procedure as specified in 5.3.5.7;

[…unrelated texts are omitted…]

5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

[…unrelated texts are omitted…]

1>	if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

1>	if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID:

2>	if the LTM cell switch is triggered on the MCG:

3>	update the master security key by performing the AS security key update procedure as specified in 5.3.5.7;

3>	if the LTM-Candidate IE indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release:

4>	derive or update the secondary key (S-KgNB) by performing security key update procedure as specified in 5.3.5.7;

Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

[…unrelated texts are omitted…]

5.3.	TP4 (Option 1 of Proposal 7)

5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

1>	if T304 of the MCG expires; or

1> if T420 expires; or,

1> if the target L2 U2N Relay UE (i.e., the UE indicated by targetRelayUE-Identity in the received RRCReconfiguration message containing reconfigurationWithSync indicating path switch as specified in 5.3.5.5.2) changes its serving PCell before path switch:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source Pcell, according to clause 5.3.10.3:

[…unrelated texts are omitted…]

2>	else:

3>	if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution as described in 5.3.5.18.6 and 5.3.5.18.x:

4>	if the LTM cell switch execution procedure does not require a security key refresh:

5>	revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG;

3>	else if the T304 of the MCG expiry is caused by an LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3:

4>	revert back to the UE configuration used in the source PCell (i.e., the source Pcell prior to the starting of timer T311) except for the PDCP state variables for SRB(s) associated to the MCG;

5.3.7.3	Actions following cell selection while T311 is running

[…unrelated texts are omitted…]

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers or fulfilment of LTM cell switch conditions as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	if the selected cell does not have the field ltm-NoSecurityChangeID configured and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

2>	if the cell selection is triggered by detecting radio link failure of the MCG and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

2>	if the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6 and the seleted cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected:

3>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

1>	else:

2> if the cell selection is triggered by T304 expiry for an LTM cell switch procedure and the UE didn’t revert back to the UE configuration used in the source Pcell at the T304 expiry:

3> revert back to the UE configuration used in the source Pcell;

2>	if UE is configured with attemptCondReconfig; or

2>	if UE is configured with attemptLTM-Switch:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

5.3.	TP5 (Option 2 of Proposal 7)

5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

1>	if T304 of the MCG expires; or

1> if T420 expires; or,

1> if the target L2 U2N Relay UE (i.e., the UE indicated by targetRelayUE-Identity in the received RRCReconfiguration message containing reconfigurationWithSync indicating path switch as specified in 5.3.5.5.2) changes its serving PCell before path switch:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source Pcell, according to clause 5.3.10.3:

[…unrelated texts are omitted…]

2>	else:

3>	if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution as described in 5.3.5.18.6 and 5.3.5.18.x:

4>	revert back to the UE configuration used in the source Pcell except for the PDCP state variables for SRB(s) associated to the MCG;

3>	else if the T304 of the MCG expiry is caused by an LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3:

4>	revert back to the UE configuration used in the source PCell (i.e., the source Pcell prior to the starting of timer T311) except for the PDCP state variables for SRB(s) associated to the MCG;

5.3.7.3	Actions following cell selection while T311 is running

[…unrelated texts are omitted…]

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers or fulfilment of LTM cell switch conditions as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	if the selected cell does not have the field ltm-NoSecurityChangeID configured and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

2>	if the cell selection is triggered by detecting radio link failure of the MCG and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or

2>	if the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6 and the seleted cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected:

3>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.