3GPP TSG RAN WG2 Meeting #130			R2-2504053

St. Julians, Malta, May 19th – 23rd    2025

Agenda Item: 	8.2.4

Source:		Sony

Title:			Considerations on segmentation and remaining bits

Document for:	Discussion & Decision

# Introduction

At RAN#106 meeting, a new WID for Ambient IoT in NR was approved including the following RAN2 objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

At RAN2#129 [3], the following agreements were made related to Data transmissions.

Agreements - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Agreements

From RAN2 perspective only the following types of procedures will be considered in the normative phase: “Inventory only” and “Inventory and command”.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

At RAN2#129bis meeting [4], the following agreements were made related to Data transmission,

Agreements

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

Agreements on segmentation

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

- FFS whether the reader always includes the command for retransmission of segments.

- 1-bit indication is sufficient to indicate whether more D2R data will be sent

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.  Notify SA2 about this assumption.

Agreements on MAC PDU format

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte.  The actual TBS value depends on RAN1.   FFS for R2D trigger message

- RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1.

- The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message

- The MAC padding is supported at least for D2R from RAN2 perspective.   The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

In this contribution, we discuss our view on segmentation and how to handle and indicate remaining bits, offset and energy level.

# Discussions on segmentation and remaining bits

Segmentation and remaining bits:

Segmentation can be needed in order to fit the data or packages into suitable Transport Blocks (TB) or units for transmission over the radio interface.

For Ambient IoT, e.g. transport block sizes up to 1000 bits has been discussed during study phase [2], but segmentation on data level is understood as too complex and only simplified segmentation may be supported in the A-IoT MAC layer.

From the TR it clarified that how large TB that can be transported at a given time depends on target coverage/data rate, energy consumption/device availability, etc.

And additionally, the following candidate solutions or restrictions have been listed in [2]:

-	The sequence number, the segment number and the number of segments are not supported;

-	An indication is used to indicate to the reader on whether the data is segmented and whether the MAC PDU is the last segment. It needs to be further discussed on the size of this indication (one or two bits) and the corresponding further details;

-	It is beneficial for the reader to be able to trigger a re-transmission of a segment;

-	It is assumed that the A-IoT device will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all segment(s) are stored in upper layer(s).

At the first meeting for the work item phase the following agreement was made for segmentation [3]:

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

Based on these agreements we like to raise some open questions and potential issues.

For the first agreement using only 1-bit indicating whether there is more data or not, this can work if there are only 2 segments, and they are delivered in correct order. However, when there is a need for a retransmission, we need to ensure the Reader understands in which order they are supposed to be assembled. This may not be possible with only 1-bit indication without allowing any other numbering approaches.

At last meeting, RAN2#129bis [4] it was agreed that 1-bit indication is sufficient to indicate whether more D2R data will be sent, and yes that is a relevant indication, but still there may be a need to discuss how much more data will be transmitted in future segments, which may depend on use case or situation as mentioned above.

Proposal 1: Further discuss whether additional bits would be needed for different use cases.

The last agreement about MAC PDU format

- in case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.  related to MAC PDU,

expresses that the device indicates how many bits the actual data part contains. This would imply that the device has the possibility to indicate also the number of remaining bits. In case no padding is used, the Reader would implicitly understand that this is the total amount of remaining bits if the MAC PDU is NOT the last segment.

Proposal 2: The device indicating how many bits are present in the SDU, can be used for indicating total remaining bits if not being the last segment.

Offset:

At the last [RAN2#129bis] meeting, the following agreement was made:

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

Though we have not really made any agreements related to the meaning of offset. Is the offset always an indication from the Reader on how many bits received in the previous segment/MAC SDU, or can it be total number of data bits received including all segments so far.

Proposal 3: Clarify the definition of offset.

Energy level;

This relates to the open question when there is not energy enough to transmit the whole data packet received from upper layers and which layer is aware of the amount of energy. In the following we discuss potential alternatives to address this.

Option 1: Upper layer knows energy level.

If the upper layer knows the amount of available energy, then it can decide to only respond to the reader if there is enough energy. This may be good for AS layer, since the AS layer does not have to be involved in energy status aspects.

The drawback is that upper layers may not know if packet for different reasons needs to be segmented or re-transmitted, which may lead to an energy shortage.

Option 2: MAC layer has the information.

Then MAC layer can do segmentation based on how large segment can be transmitted, before harvesting is needed.

This is good since MAC can know max Transport Block Sizes etc.

The drawback is that MAC may not have instantaneous information on the remaining level of stored energy during an ongoing transmission.

Option 3: PHY has the information.

For this case MAC can create e.g. max TRB of 1000 bits and push it down to PHY.

Then PHY has to do segmentation based on either energy level, or based on number of re-transmissions etc., if not handled in the MAC layer.

Proposal 4: RAN2 to discuss which layer in the device handles information about energy level. Opt1. Upper layer, Opt2. MAC layer, Opt3. PHY layer.

# Summary

In this contribution, we have discussed our views on aspects related to segmentation and remaining bits, offset and energy level.

Proposal 1: Further discuss whether additional bits would be needed for different use cases.

Proposal 2: The device indicating how many bits are present in the SDU, can be used for indicating total remaining bits if not being the last segment.

Proposal 3: Clarify the definition of offset, and if there is a need to clearly relate the offset to a specific MAC PDU.

Proposal 4: RAN2 to discuss which layer in the device handles information about energy level. Opt1. Upper layer, Opt2. MAC layer, Opt3. PHY layer.

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- TR.38.769

- Minutes RAN2#129, Athens

- Minutes RAN2#129bis, Wuhan