3GPP TSG-RAN WG2 Meeting #130  	                            R2-2504054

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.5.3

Source: 	Sony

Title:	Remaining details for on-demand SIB1

Document for:	Discussion

# Introduction

In RAN2#129bis meeting, the agreements on OD-SIB1 were agreed as follows:

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

In this contribution, we discuss remaining aspects on OD-SIB1.

# Discussion

In RAN2#127bis meeting, it was agreed that if UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for the cell before requesting SIB1 of that cell.

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

And further in RAN1#120bis meeting, the agreement regarding UE monitoring SIB1 transmissions is agreed as follows:

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=12 for FR2, select the following: - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission

With Alt. 3, it implies the UE’s behaviour to monitor the SIB1 transmission doesn’t need to be specified. RAN2 may need to revisit previous agreement regarding whether to monitor the OD-SIB1 is being broadcasted before sending the request.

Observation 1: RAN2 may need to revisit the previous agreement regarding UE’s behaviour on monitoring OD-SIB1 is being broadcasted before sending the request, after RAN1 makes decision on this issue.

It is better for RAN2 to inform RAN1 of their decision so that RAN1 can discuss a solution that is not against RAN2 decision.

Proposal 1: Send LS to RAN1 informing RAN2 decision on “if UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.”.

# Conclusion

We have observation as follows.

Observation 1: RAN2 may need to revisit the previous agreement regarding UE’s behaviour on monitoring OD-SIB1 is being broadcasted before sending the request, after RAN1 makes decision on this issue.

We propose RAN2 to discuss following proposals:

Proposal 1: Send LS to RAN1 informing RAN2 decision on “if UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.”.