3GPP TSG-RAN WG2 Meeting #130  	                            R2-2504060

St.Julians, Malta, May 19th – 23rd, 2025							          (Resubmission of R2-2502497)

Agenda Item:	8.13.2

Source: 	Sony

Title:	Multi-hop relay selection/re-selection

Document for:	Discussion

# Introduction

In RAN2#129 meeting, multi-hop relay discovery/selection/re-selection was discussed with agreements on relay selection/re-selection as following:

Agreements:

For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold.

-	When U2N Remote UE receives a PC5-S link release message from current first relay UE.

-	When U2N Remote UE detects PC5 RLF with the current first relay UE.

-	Indicated by upper layer.

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

-	release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

-	FFS: link quality degradation of the upstream links resulting in a notification.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS applicability to L3.

In this contribution, we discuss the issues of sidelink relay selection/re-selection in multi-hop relay.

# Discussion

In Rel-17 sidelink L2 U2N relay, if there are multiple suitable U2N Relay UEs, it is up to U2N Remote UE implementation to choose one of the U2N Relay UEs among them. With the introduction of multi-hop L2 U2N relay, both L2 single hop U2N relay and L2 multi-hop U2N relays could be deployed in the network for remote UE to choose from. On the other hand, from reliability and latency point of view, the reliability will reduce as the number of hops will increase and the latency will increase as the number of hops increases. If it is fully up to remote UE implementation to select/re-select the relay among the relay candidates and if remote UE is not aware of the number of hops to the network, the remote UE will tend to only consider the PC5 radio link quality (and/or any existing criteria) on the first hop e.g., the radio link quality between remote UE and first relay UE candidate. But the QoS performance e.g., reliability, latency will fluctuate among the choice of different relay paths with different number of hops on the path, the QoS requirement may not be met for certain relay paths if only the first hop PC5 radio link quality is considered.

Observation: The QoS requirement may not be guaranteed for certain relay paths with different number of hops on the path, if only existing one hop L2 U2N relay criteria e.g., PC5 radio link quality of the first hop is considered for multi-hop relay selection/re-selection.

So, the criteria on how to select the relay among different types e.g., number of hops of relay candidates should be specified, in addition to existing one hop L2 U2N relay criteria, e,g., the radio link quality of the first hop, if the above problem is recognised.

Proposal 1: RAN2 to discuss the criteria to select/re-select L2 U2N relay, in addition to the existing one hop L2 U2N relay criteria, with the introduction of multi-hop relay.

In Rel-17 U2N relay, the remote UE triggers U2N relay selection when the direct Uu signal strength of current serving cell of the remote UE is below a configured signal strength threshold. Similarly, the same principle should be applied when remote UE performs the selection of U2N multi-hop relay. Network may configure a Uu link quality threshold e.g., RSRP threshold, indicating that only when the Uu link quality is below this threshold, the remote UE will be allowed to select multi-hop L2 U2N relay. This threshold can be included in the broadcast signalling or pre-configured.

And it was agreed that in RAN2#129, the direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold will trigger the multi-hop relay selection. It is FFS whether the same Uu RSRP threshold as single hop U2N relay will be configured for multi-hop U2N relay or not. It is preferrable that a separate Uu RSRP threshold is configured for multi-hop U2N relay if network would like to restrict the condition for remote UE to perform multi-hop U2N relay selection.

Proposal 2: Network configures a separate Uu signal strength threshold for remote UE to perform the multi-hop U2N relay selection.

# Conclusion

We have observation as follows.

Observation: The QoS requirement may not be guaranteed for certain relay paths with different number of hops on the path, if only existing one hop L2 U2N relay criteria e.g., PC5 radio link quality of the first hop is considered for multi-hop relay selection/re-selection.

We propose RAN2 to discuss following proposals:

Proposal 1: RAN2 to discuss the criteria to select/re-select L2 U2N relay, in addition to the existing one hop L2 U2N relay criteria, with the introduction of multi-hop relay.

Proposal 2: Network configures a separate Uu signal strength threshold for remote UE to perform the multi-hop U2N relay selection.