3GPP TSG-RAN WG2 Meeting #130	R2-2504066

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.4.3

Source:	Huawei, HiSilicon

Title:	Further discussion on the criteria for RRM measurement relaxation and offloading

Document for:	Discussion and Decision

- Introduction

The following agreements are reached on RRM measurement relaxation and offloading in previous RAN2 meetings.

RAN2#127 - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation RAN2#129 - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds. RAN2#129bis - RAN2 assumes for the entry/ exit conditions of serving cell measurement offloading and serving cell RRM measurement relaxation: separate MR thresholds (according to RAN1 agreement)/LR thresholds can be configured for different types of LP WUR if a cell supports both types of LRs (can revisit based on RAN1 and RAN 4 progress, if any). - RAN2 assumes the entry/exit thresholds for RRM relaxation/offloading for OFDM-based WUR measuring LP-SS only are the same as that for OOK-based WUR measuring LP-SS. It can be revisited based on RAN1/RAN4 process, if any. Network is allowed to provide either OOK based threshold or OFDM based WUR mesasuring SSB threhold or both. - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell. - The metrics for RRM measurement offloading/relaxation criteria include (LP-)RSRP and optional (LP-)RSRQ. - How to define LP-RSRP and LP-RSRQ is up to RAN1. - The duplication between RAN2 and RAN4 specification on RRM relaxation and offloading should be avoided, details up to running CR rapporteur and companies’ review. - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

The following agreements are reached in RAN4 for RRM measurement relaxation and offloading.

The following cases were discussed in RAN4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off; ON. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On with relaxation measurement; ON. #5 Exit case; On; On; Off. RAN4#111 Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4#112 Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. RAN4#114 Agreement: - MR RRM relaxation for serving cell/neighbour cell: scaling factor is 16. RAN4#114bis Agreement: - Support relaxed higher priority frequency layer search at case 1

In this contribution, we further discuss the criteria for RRM measurement relaxation and offloading for IDLE/INACTIVE mode.

- Discussion

Reducing the number of LP-WUS related thresholds

In the RAN2#129bis meeting, a concern was raised regarding the tremendous number of LP-WUS related thresholds including entry/exit condition for LP-WUS monitoring, serving cell measurement relaxation/offloading, and neighboring cell relaxation. This complexity may pose implementation challenges for LP-WUS. Consequently, RAN2 is deliberating on potential optimizations, such as merging the thresholds across different UE behaviors for LP-WUS operation.

Currently RAN2 made the agreement that the entry/exit condition between serving cell measurement relaxation and neighbor cell relaxation can be merged [1].

Agreement in RAN2-129b - Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion). - It is up to NW to configure either serving cell relaxation or serving cell offloading or both in one cell.

Another possible way to reduce the number of thresholds for LP-WUS is to merge the entry/exit condition for LP-WUS monitoring and RRM measurement relaxation (i.e., RAN4 CASE#3) or RRM measurement fully offloading (i.e., RAN4 CASE#1). We first discuss the UE behaviors for different cases of RRM measurement relaxation and LP-WUS monitoring as below.

Table 1. UE behaviors for different cases of RRM measurement relaxation and LP-WUS monitoring

| Cases |  |  | UE behaviors |
| --- | --- | --- | --- |
| No. | Criteria for RRM measurement relaxation | Criteria for LP-WUS monitoring |  |
| ① | Fulfilled | Fulfilled | UE can monitor LP-WUS, perform LR measurement and relaxed MR serving/neighbour cell measurement |
| ② | Not fulfilled | Not fulfilled | Legacy behaviors for paging reception and RRM measurements |
| ③ | Fulfilled | Not fulfilled | UE only performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS If UE can perform LR measurement, why cannot UE monitor LP-WUS? |
| ④ | Not fulfilled | Fulfilled | UE only monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (note that fully offloading criteria is not fulfilled either) There is no power saving gain for this case |

For case ④, it is not reasonable that UE enters LP-WUS coverage but still performs normal RRM measurement in which case there is no power saving gain, so case④ should not be supported. Then for case③, if the LR measurement can be used, then LP-WUS can also be used for further reducing power consumption for paging reception, otherwise, the thresholds for RRM measurement relaxation criteria and LP-WUS monitoring criteria are not properly set. Hence, it makes sense to only support case① and case②.

Observation 1: The following two cases should not be supported:

- UE performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS;

- UE monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (i.e., no RRM measurement relaxation).

RAN2 agreed that it is up to NW to configure either RRM measurement relaxation or RRM measurement fully offloading or both in one cell, we discuss based on the following scenarios:

Scenario 1: NW only configures RRM measurement relaxation (CASE#3)

If the network only configures the entry/exit condition for RRM measurement relaxation (CASE#3), then the it can be merged with the entry/exit condition for LP-WUS monitoring, i.e., if the entry condition of RRM measurement relaxation (CASE#3) is fulfilled, the UE starts LP-WUS monitoring, and performs RRM serving/neighbor cell measurement relaxation. If the exit condition of RRM measurement relaxation (CASE#3) is fulfilled, the UE starts legacy procedure by MR and performs legacy RRM measurement by MR.

Scenario 2: NW only configures RRM measurement fully offloading (CASE#1)

Similar to the above discussion on RRM measurement relaxation (CASE#3), if the network only configures the entry/exit condition for RRM measurement fully offloading (CASE#1), then it is not reasonable for UE to monitor LP-WUS but still perform legacy measurement, or to perform measurement only on LR without LP-WUS monitoring.

In this case, the entry/exit condition for RRM measurement fully offloading (CASE#1) can be reused for LP-WUS monitoring, which means if the entry condition of RRM measurement fully offloading (CASE#1) is fulfilled, the UE starts LP-WUS monitoring, and performs RRM serving cell offloading. If the exit condition of RRM measurement fully offloading (CASE#1) is fulfilled, the UE starts legacy procedure by MR and performs legacy RRM measurement by MR.

Scenario 3: NW configures both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1)

If the network configures both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1), for CASE#1, since neighbor cell of equal or lower priority (including intra-frequency, NR inter-frequency and inter-RAT frequency) is off, it means UE is not likely to perform cell re-selection. For example, UE is near the center of the cell. On the other hand, for CASE#3, UE may be near the cell edge so that neighboring cell measurement is on. So obviously the RSRP threshold for CASE#1 should be higher than for CASE#3.

Observation 2: RAN4 CASE#1 is the RRM measurement behaviors for UEs in center of the cell and RAN4 CASE#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage.

Proposal 1: The threshold(s) for RRM measurement fully offloading (CASE#1) should be higher than threshold(s) for RRM measurement relaxation (CASE#3).

Based on above, how LP-WUR and MR perform RRM measurement is illustrated in the figure below.

Fig. 1. UE behaviors for RRM measurement fully offloading and relaxation cases

It is obvious that once both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1) are configured, then UE shall start LP-WUS monitoring if the entry condition of RRM measurement relaxation (CASE#3) is fulfilled, and the UE starts legacy procedure by MR and performs legacy RRM measurement by MR if the exit condition of RRM measurement relaxation (CASE#3) is fulfilled.

From the discussion on the above 3 scenarios, we can conclude that:

Proposal 2: (38304-4/RRC-8) RAN2 to agree on the following cases for merging the thresholds of RRM measurement relaxation (CASE#3) or RRM measurement fully offloading (CASE#1) with LP-WUS monitoring:

- If the network configures the RRM measurement relaxation (CASE#3), the UE can regard the entry/exit condition of RRM measurement relaxation (CASE#3) as the entry/exit condition of LP-WUS monitoring, regardless of whether the RRM measurement fully offloading (CASE#1) is configured.

- If the network ONLY configures the RRM measurement fully offloading (CASE#1), the UE can regard the entry/exit condition of RRM measurement fully offloading (CASE#1) as the entry/exit condition of LP-WUS monitoring.

The exit condition for RRM measurement relaxation

Regarding the exit condition of RRM measurement relaxation (CASE#3), a separate exit condition based on LR measurement is preferred. On the one hand, since the MR RRM measurement is relaxed and RAN4 agreed that the scaling factor for MR RRM relaxation is 16, MR measurement results cannot be obtained in a timely manner, which delays the exit of LP-WUS monitoring and may result in paging loss. On the other hand, with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism. Furthermore, it is also aligned with Proposal 8 above, since “exit condition based on LR measurement” was agreed for LP-WUS entry/exit condition.

Observation 3: Defining a separate exit condition based on LR measurement has the following advantages:

- the LR measurement results is more timely compared with MR measurement results, since RAN4 agreed that the scaling factor for MR RRM relaxation is 16;

- with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism.

Proposal 3: (38304-3/RRC-7) The exit condition for RRM measurement relaxation is based on the LR measurement, i.e., when the LR measurement is below a threshold.

The entry condition for RRM measurement fully offloading

In RAN2#127 meeting, RAN2 discussed the entry/exit condition for RRM measurement fully offloading (CASE#1) without taking CASE#3 (which was agreed in parallel by RAN4) and scaling factor for MR RRM relaxation (which was agreed by RAN4) into account. Currently, with both CASE#1 and CASE#3 configured by the network, RAN2 needs to further think about the entry/exit condition considering: switch between CASE#1 and CASE#3.

If UE is in the RRM measurement relaxation case, RAN4 agreed that the scaling factor for MR RRM relaxation is 16. In this case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading case. Thus, the serving cell quality measured by LR is more appropriate to be used for the entry condition of fully offloading.

Observation 4: In the RRM measurement relaxation case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading.

Proposal 4: If UE is currently in the RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

Measurement relaxation for higher priority frequency

In the RAN4’s latest meeting, it was agreed as followed that the UE shall perform relaxed measurement for the neighbor cell with higher priority when implementing RRM measurement fully offloading (CASE#1).

Agreement in RAN4-114b Issue 1-1-10: Considerations on higher priority frequency layer Agreement: - Support relaxed higher priority frequency layer search at case 1

In Rel-16, the IE highPriorityMeasRelax was introduced in the RRM relaxation configuration to indicate whether the measurements can be relaxed on high priority frequencies. The UE is not allowed to perform high-priority measurement relaxation with the absence of this indication, to ensure UE can reselect to a high-priority frequency with good channel condition as soon as possible. The same indication can be reused by the network to flexibly control the UE measurement behavior of high priority frequencies when UE operates under RRM measurement relaxation or RRM measurement fully offloading.

Proposal 5: (38304-7/RRC-9) RAN2 to consider reusing the highPriorityMeasRelax to indicate whether the measurements can be relaxed on high priority frequencies during RRM measurement fully offloading (CASE#1). FFS whether it also applies to RRM measurement relaxation (CASE#3).

RRM measurement with low mobility criterion

For the existing Rel-16 relaxed measurement rules, in addition to the criteria for not being at cell edge (based on the absolute serving cell quality), the criteria for low mobility (based on serving cell quality variation over time) was also defined. However, introducing mobility-based criteria would significantly increase the complexity of the current decision rules for entering/exiting both RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1), which are merely based on the serving cell quality.

Furthermore, implementing mobility criteria could potentially impact the availability of the LP-WUS function for high-mobility UEs, such as smartphones or in-vehicle devices. Rather than imposing network-defined mobility restrictions, we recommend allowing UE implementation to determine when to exit LP-WUS monitoring during high mobility state.

Based on these considerations, we propose maintaining the current approach without introducing similar low-mobility criteria for CASE#1 and CASE#3.

Proposal 6: (38304-6/RRC-10) The low mobility criteria is not supported for the entry/exit condition of RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1).

Proposal 7: It is up to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state.

Relationship between the thresholds for Rel-19 RRM measurement relaxation/offloading and legacy neighbor cell measurement condition

The neighbour cell measurement rules for cell reselection is specified in TS 38.304 as followed [2].

Following rules are used by the UE to limit needed measurements: - If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements. - Otherwise, the UE shall perform intra-frequency measurements. - The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1: - For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8]. - For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency: - If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - Otherwise, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8]. - If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

Considering that RRM measurement fully offloading (CASE#1) and neighboring cell measurement criteria are both configured by the network, there might occur the scenario that the thresholds of the legacy neighbor cell measurement criteria are greater than the thresholds of CASE#1 as shown in the figure below.

Fig. 2. Thresholds of the neighboring cell measurement criteria are greater than the thresholds of CASE#1

In such case, if UE meets the entry condition of CASE#1 (i.e., within the blue circle of the diagram), then UE shall not do the serving cell measurement on MR (accoring to the the UE behavior defined for CASE#1), while the LP-WUR shall be responsible for serving cell measurement.

However, this behavior is inherently problematic since the UE cannot obtain serving cell measurement results from the MR to evaluate whether the legacy neighbor cell measurement criteria are fulfilled. This creates ambiguity in the RRM measurement behavior, as the UE lacks the necessary measurement results to make critical decision on the cell reselection, which may affect the UE mobility.

Observation 5: There exists unreasonable UE measurement behavior when UE meets the entry condition of CASE#1 under the scenario that the thresholds of the legacy neighbor cell measurement criteria are greater than those of CASE#1.

One potential way to solve the case is to specify in the spec that the network shall not configure the thresholds of CASE#1 lower than those of the legacy neighbour cell measurement criteria. However, according to the current agreements made by RAN2, the entry condition of CASE#1 can optionally include threshold based on LP-WUR, and the exit condition relies entirely on LP-WUR-based serving cell measurement results, while threholds of the legacy neighbor cell measurement criteria are always based on MR. Due to the different accuracy and performance between MR and LP-WUR, the network cannot always guarantee the relationship between the thresholds for CASE#1 and legacy neighbor cell measurement criteria. This may somehow introduce potential conflicts in measurement behavior, making it difficult to maintain mobility management.

Another solution we slightly prefer is to constrain that the UE should ignore the neighbor cell measurement criterion when it is in the condition of CASE#1. Compared with the preceding solution, this solution completely avoids the improper UE behavior in the above scenario without introduce any restrictions on network configuration.

Proposal 8: (RRC-8) If UE fulfills the entry condition of RRM measurement fully offloading, the UE ignores the legacy measurement rules for cell reselection (i.e., s-IntraSearchP/s-NonIntraSearchP) as described in TS 38.304, to avoid the ambiguity of UE measurement behavior.

- Conclusion

In this contribution, we discuss the remaining issues on the criteria for RRM measurement relaxation and offloading for IDLE/INACTIVE mode, and have the following proposals:

Reducing the number of LP-WUS related thresholds

Observation 1: The following two cases should not be supported:

- UE performs LR measurement and relaxed MR serving/neighbour cell measurement, but does not monitor LP-WUS;

- UE monitors LP-WUS, but performs normal MR serving/neighbour cell measurement (i.e., no RRM measurement relaxation).

Observation 2: RAN4 CASE#1 is the RRM measurement behaviors for UEs in center of the cell and RAN4 CASE#3 is the RRM measurement behaviors for UEs near the edge of the LP-WUS coverage

Proposal 1: The threshold(s) for RRM measurement fully offloading (CASE#1) should be higher than threshold(s) for RRM measurement relaxation (CASE#3).

Proposal 2: (38304-4/RRC-8) RAN2 to agree on the following cases for merging the thresholds of RRM measurement relaxation (CASE#3) or RRM measurement fully offloading (CASE#1) with LP-WUS monitoring:

- If the network configures the RRM measurement relaxation (CASE#3), the UE can regard the entry/exit condition of RRM measurement relaxation (CASE#3) as the entry/exit condition of LP-WUS monitoring, regardless of whether the RRM measurement fully offloading (CASE#1) is configured.

- If the network ONLY configures the RRM measurement fully offloading (CASE#1), the UE can regard the entry/exit condition of RRM measurement fully offloading (CASE#1) as the entry/exit condition of LP-WUS monitoring.

The exit condition for RRM measurement relaxation

Observation 3: Defining a separate exit condition based on LR measurement has the following advantages:

- the LR measurement results is more timely compared with MR measurement results, since RAN4 agreed that the scaling factor for MR RRM relaxation is 16;

- with different conditions for exit and entry, the ping-pong issue can be avoided by NW implementation when configuring the conditions and there is no need to define additional hysteresis mechanism.

Proposal 3: (38304-3/RRC-7) The exit condition for RRM measurement relaxation is based on the LR measurement, i.e., when the LR measurement is below a threshold.

The entry condition for RRM measurement fully offloading

Observation 4: In the RRM measurement relaxation case, serving cell measurement by MR is not timely enough to be used for the evaluation of the entry condition of fully offloading.

Proposal 4: If UE is currently in the RRM measurement relaxation case, serving cell measurement by LR is used for the entry condition of fully offloading.

Measurement relaxation for higher priority frequency

Proposal 5: (38304-7/RRC-9) RAN2 to consider reusing the highPriorityMeasRelax to indicate whether the measurements can be relaxed on high priority frequencies during RRM measurement fully offloading (CASE#1). FFS whether it also applies to RRM measurement relaxation (CASE#3).

RRM measurement with low mobility criterion

Proposal 6: (38304-6/RRC-10) The low mobility criteria is not supported for the entry/exit condition of RRM measurement relaxation (CASE#3) and RRM measurement fully offloading (CASE#1).

Proposal 7: It is up to UE implementation to determine whether to exit LP-WUS monitoring when in high mobility state.

Relationship between the thresholds for Rel-19 RRM measurement relaxation/offloading and legacy neighbor cell measurement condition

Observation 5: There exists unreasonable UE measurement behavior when UE meets the entry condition of CASE#1 under the scenario that the thresholds of the legacy neighbor cell measurement criteria are greater than those of CASE#1.

Proposal 8: (RRC-8) If UE fulfills the entry condition of RRM measurement fully offloading, the UE ignores the legacy measurement rules for cell reselection (i.e., s-IntraSearchP/s-NonIntraSearchP) as described in TS 38.304, to avoid the ambiguity of UE measurement behavior.

- References

RAN2#129bis chairman notes.

3GPP TS 38.304 V16.11.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state