3GPP TSG-RAN WG2 Meeting #130	                              R2-2504071

St. Julians, Malta, 19 - 23 May 2025

Agenda item:	8.19

Source:	Huawei, HiSilicon, China Southern Power Grid, Turkcell

Title:	ETWS geo-fencing for eMTC NTN and for TN

Document for:	Discussion and Decision

- Discussion

In Rel-19 NR NTN, RAN2 has agreed to support geo-fencing for ETWS message by introducing warning area coordinates to the NR ETWS SIBs. Meanwhile, since RAN is discussing the support of PWS for NB-IoT in Rel-19 IoT NTN, it was also agreed to introducing warning area coordinates to the NB-IoT ETWS SIBs. The corresponding agreements are listed below [1]:

- Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented) - Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB - Send a LS to CT1 indicating that we will provide warning area coordinates for ETWS for both NR NTN and NB-IoT NTN without segmentation for primary notification

However, it remains a question whether the geo-fencing should be extended to eMTC NTN, and even further to eMTC TN/LTE TN or NR TN. During the discussion in IoT NTN topic, the main concern from companies was that this is not in the scope of Rel-19 IoT NTN since the objective of PWS is only for NB-IoT NTN. Also considering that we need to take eMTC TN/LTE TN and NR TN into account, we think TEI19 should be the right place for this discussion to take place. The following table summarizes all the scenarios and the highlighted scenarios need further discussion:

| Scenarios | Whether ETWS geo-fencing is applied | Impact if ETWS geo-fencing is applied |
| --- | --- | --- |
| NR NTN | Yes | Introducing coordinates to NR SIB6/SIB7 IN 38.331 (already agreed) |
| NB-IoT NTN | Yes | Introducing coordinates to NB-IoT SIB10-NB/SIB11-NB (already agreed) |
| NB-IoT TN | Yes (Agreement: The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.) | No extra spec impact |
| eMTC NTN | FFS (not in IoT NTN WID) | Introducing coordinates to LTE SIB10/SIB11 |
| eMTC TN | FFS | No extra spec impact (if ETWS geo-fencing is applied to eMTC NTN) |
| LTE TN | FFS | No extra spec impact (if ETWS geo-fencing is applied to eMTC NTN) |
| NR TN | FFS | No extra spec impact |

eMTC NTN

At least in NTN scope, the principle should be similar, i.e., eMTC NTN should follow NR NTN and NB-IoT NTN due to the same motivation (the coverage of one cell may be larger than the ETWS valid area). So we see no reason to leave eMTC NTN out of the ETWS geo-fencing supporting group.

Proposal 1: RAN2 to confirm that ETWS geo-fencing is supported for eMTC NTN and introduce“warning area coordinates” to SIB10 and SIB11.

eMTC TN and LTE TN

When discussing the PWS support for NB-IoT NTN, RAN2 agreed that this can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. Here once Proposal 1 is agreed, the newly introduced “warning area coordinates” in SIB10 and SIB11 may also be used TN without extra spec effort. In any case, if it is not useful in TN, NW can choose not to broadcast the coordinates by implementation. Meanwhile, if we want to limit the usage to only NTN, RAN2 can further discuss whether it requires additional restriction in the spec.

Proposal 2: RAN2 to discuss whether ETWS geo-fencing is applied to eMTC TN and LTE TN in case there is no extra spec effort.

NR TN

Similarly since the“warning area coordinates” has been introduce to SIB6 and SIB7 for NR NTN, they may be also used by NR TN without extra spec effort.

Proposal 3: RAN2 to discuss whether ETWS geo-fencing is applied to NR TN in case there is no extra spec effort.

- Conclusion

In this paper, we discuss whether to apply the ETWS geo-fencing to eMTC NTN and to TN and have the following proposals.

Proposal 1: RAN2 to confirm that ETWS geo-fencing is supported for eMTC NTN and introduce“warning area coordinates” to SIB10 and SIB11.

Proposal 2: RAN2 to discuss whether ETWS geo-fencing is applied to eMTC TN and LTE TN in case there is no extra spec effort.

Proposal 3: RAN2 to discuss whether ETWS geo-fencing is applied to NR TN in case there is no extra spec effort.

- References

R2-2501333, Report from Break-out session on NR-NTN and IoT-NTN