3GPP TSG-RAN WG2 Meeting #130	                              R2-2504075

St. Julians, Malta, 19 - 23 May 2025

Agenda Item:	8.18.2

Source:	Huawei, HiSilicon

Title:	Discussion on time-frequency interleavers for MBMS

Document for: Discussion and Decision

1	Introduction

In RAN#106, a work item on introduction of time and frequency interleaving for LTE- based 5G Terrestrial Broadcast was approved [1], and the objective in which RAN2 is involved is listed as following:

For MBMS-dedicated cells, specify time-frequency interleavers [RAN1] and corresponding signaling [RAN2, RAN3] - For the time-interleaving part of 5G Broadcast TFI, one transport block is mapped to multiple non-consecutive subframes, following the principles of NR TBoMS. The following steps are followed: - Scale the TBS by a factor of N, where N is the number of subframes over which the TB is mapped. - RAN1 to discuss the details of the scaling, the value(s) for N and the set of subframes over which the TB is mapped, including how to schedule the TBs. - Within one subframe, the transmitted rate matched bits are read consecutively from the circular buffer. - In a given subframe, the procedure in TS 36.212 Section 5.1.4.1.2 applies (except for the parameter k0k0 which needs to be modified according to point 3 below) - Before frequency interleaving (if applied), the mapping from coded bits to RE follows the current specification. - Across subframes belonging to the same TB, the starting point for reading from the circular buffer (k0(k0 in TS 36.212 Section 5.1.4.1.2) is to be decided by RAN1 - NOTE 1: The maximum TBS after scaling and soft buffer size should follow limitations imposed by current LTE categories. RAN1 to discuss how to capture this limitation. - NOTE 2: Time interleaving is not applied for PMCH carrying MSI and/or MCCH. - For the frequency-interleaving part of 5G Broadcast TFI, interleaving is performed via a row-column interleaving with the following steps: - Data symbols in an OFDM symbol are written column-wise. - RAN1 to decide what is the unit over which data symbols are written (e.g. a single RE, four REs, etc.) - The rows and elements in each row may be permuted, with details to be decided by RAN1. - Data symbols in an OFDM symbol are read row-wise. - RAN1 to decide the number of {rows, columns} of the interleaver. NOTE: Time-frequency interleaving is not applicable to 0.37kHz SCS.

In this contribution, we will analyse the RAN2 impacts to support time-frequency interleavers for MBMS.

2	Discussion

## 2.1 Signalling to support time-frequency interleaving

For time interleaving：

Currently, LTE-based 5G broadcast does not support time interleaving, which is mainly used to disperse transmission of a TB in the time domain to different MBSFN subframes. Lack of time interleaving can impair the performance in the presence of multi-path and fading channels. The principle of time interleaving is shown in the following figure where each TB is separated to 4 transmissions in different subframes.

RAN1 has achieved some agreements about time interleaving, as follows:

Agreement Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH. Agreement The basic transmission pattern of PMCH with time interleaving consists of a set of (M x N) consecutive MBSFN subframes, excluding MCCH and MSI, where two transmissions of the same TB are separated by (M-1) MBSFN subframes, excluding MCCH and MSI. Agreement Support at least the following values of time interleaver depth: - Values M={4,8,16,32} and N={1,2,4,8,16} are supported. - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI. FFS: larger values, including M=64, N={32,64}.

According to RAN1 agreement, the time interleaving is only applied to MTCH, not to MCCH and MSI MAC CE. A time interleaving pattern M x N should be indicated to UE, where parameter N indicates the TBS scaling factor N, and parameter M indicates the interval between two transmissions of the same TB.

Observation 1: The time interleaving is only applied to MTCH, not for MCCH and MSI MAC CE.

Observation 2: The eNB should indicate time interleaving parameters M and N to UE, where parameter N indicates the TBS scaling factor N, and parameter M indicates the interval between two transmissions of the same TB.

RAN1 is discussing how to indicate time interleaving parameters M and N:

Agreement The agreement in the previous meeting is updated as follows: Regarding configuration for time interleaving transmission, the following options can be considered: - Option 1: Per PMCH configuration, e.g., via pmch-Config - Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

As can be seen, the granularity of time interleaving configuration is discussed by RAN1, i.e. per PMCH or per MBMS session. But RAN2 can discuss the location of time interleaving configuration. For the signalling design of time interleaving configuration, RAN2 should consider a backwards compatibility to avoid affecting the legacy MBMS UEs. If the parameters are indicated per PMCH configuration, the new Rel-19 field like PMCH-InfoListExt-r19 including the PMCH-InfoExt-r19 with parameter M and N should be introduced. For example:

PMCH-InfoList information element

-- ASN1START

PMCH-InfoList-r9 ::=				SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-Info-r9

PMCH-InfoListExt-r12 ::=			SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-InfoExt-r12

PMCH-InfoListExt-r19 ::=			SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-InfoExt-r19

PMCH-InfoExt-r19 ::=				SEQUENCE {

pmch-Config-r19						PMCH-Config-r19,

mbms-SessionInfoList-r12			MBMS-SessionInfoList-r9,

...

}

PMCH-Config-r19 ::=					SEQUENCE {

sf-AllocEnd-r19						INTEGER (0..1535),

dataMCS-r19							CHOICE {

normal-r12							INTEGER (0..28),

higerOrder-r12						INTEGER (0..27)

},

mch-SchedulingPeriod-r19		ENUMERATED {

rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, rf512, rf1024},

pmch-TimeInterleaving				SEQUENCE {

pmch-TimeInterleaving-N				ENUMERATED {n1, n2, n4, n8, n16},

pmch-TimeInterleaving-M				ENUMERATED {n4, n8, n16, n32}

},

...

}

If the parameters are indicated per MBMS session configuration via MBMS-SessionInfo, the new Rel-19 field like PMCH-InfoListExt-r19 including the PMCH-InfoExt-r19 with new field mbms-SessionInfoList-r19 should be introduced. For example:

PMCH-InfoList information element

-- ASN1START

PMCH-InfoList-r9 ::=				SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-Info-r9

PMCH-InfoListExt-r12 ::=			SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-InfoExt-r12

PMCH-InfoListExt-r19 ::=			SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-InfoExt-r19

PMCH-InfoExt-r19 ::=				SEQUENCE {

pmch-Config-r12						PMCH-Config-r12,

mbms-SessionInfoList-r19			MBMS-SessionInfoList-r19,

...

}

MBMS-SessionInfoList-r19 ::=		SEQUENCE (SIZE (0..maxSessionPerPMCH)) OF MBMS-SessionInfo-r19

MBMS-SessionInfo-r19 ::=			SEQUENCE {

tmgi-r9								TMGI-r9,

sessionId-r9						OCTET STRING (SIZE (1))		OPTIONAL,	-- Need OR

logicalChannelIdentity-r9			INTEGER (0..maxSessionPerPMCH-1),

pmch-TimeInterleaving				SEQUENCE {

pmch-TimeInterleaving-N				ENUMERATED {n1, n2, n4, n8, n16},

pmch-TimeInterleaving-M				ENUMERATED {n4, n8, n16, n32}

},

...

}

Proposal 1: A new Rel-19 field PMCH-InfoListExt-r19 should be introduced to indicate the time interleaving parameters. The details, e.g. per PMCH or per session configuration, are pending RAN1 discussion.

For frequency interleaving:

Frequency interleaving distributes the transmission to different frequency locations to reduce the impact of selective fading in the frequency domain. RAN1 hasn’t decided the parameters for frequency interleaving are configurable, or fixed and specified directly in RAN1 specifications. If the parameters are configured for frequency interleaving, the presence or absence of the parameters can indicate whether frequency interleaving is enabled or not. If the parameters are fixed and specified directly in RAN1 spec, RAN2 needs to introduce an explicit indication to indicate whether frequency interleaving is enabled or not..

Proposal 2: UE should be indicated by eNB whether the frequency interleaving is enabled or not. The details, e.g., eNB indicates this explicitly or implicitly via the presence of frequency interleaving parameters, are pending RAN1 discussion.

## 2.2 Impact on cell reselection

In Rel-14, frequency priority is introduced for cell reselection in case a UE is receiving or interested to receive an MBMS service. Since now time-frequency interleaving has been introduced for better MBMS performance, it is beneficial if UE supporting time-frequency interleaving can prioritize the cells that enable time-frequency interleaving. And for legacy UEs, the cells that enable time-frequency interleaving shouldn’t be prioritized since legacy UEs cannot decode the MBMS in these cells.

Proposal 3: UE supporting time-frequency interleaving should prioritize the cells enabling time-frequency interleaving while legacy UE shouldn’t.

# 3	Conclusion

Based on the above discussion we have the following observation and proposals:

Signalling to support time-frequency interleaving：

Observation 1: The time interleaving is only applied to MTCH, not for MCCH and MSI MAC CE.

Observation 2: The eNB should indicate time interleaving parameters M and N to UE, where parameter N indicates the TBS scaling factor N, and parameter M indicates the interval between two transmissions of the same TB.

Proposal 1: A new Rel-19 field PMCH-InfoListExt-r19 should be introduced to indicate the time interleaving parameters. The details, e.g. per PMCH or per session configuration, are pending RAN1 discussion.

Proposal 2: UE should be indicated by eNB whether the frequency interleaving is enabled or not. The details, e.g., eNB indicates this explicitly or implicitly via the presence of frequency interleaving parameters, are pending RAN1 discussion.

Impact on cell reselection

Proposal 3: UE supporting time-frequency interleaving should prioritize the cells enabling time-frequency interleaving while legacy UE shouldn’t.

4	Reference

[1] RP-250794, “Revised WID on LTE-based 5G Broadcast Phase 2”, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 2025

Annex. RAN1 agreements

  - RAN1#120 meeting

Agreement

For TBS scaling by N, down-select one of the two options below:

- Option 1: Calculate the total number of PRBs NRBtotal as NRBtotal=NRBDL×N, wherein, NRBDL is system bandwidth;

  - If NRBtotal≤110, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBtotal, obtain the corresponding TBS;

  - Else, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBDL, obtain the corresponding TBS’; and calculate the final TBS as TBS=round{TBS'×N}

  - The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

- Option 2: Obtain the initial TBS (TBS') as per current specifications, and calculate the final TBS as TBS=round{TBS'×N}

  - The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

Agreement

At least values of N={1,4,8} are supported, where N=1 indicates time interleaving is disabled.

  - FFS: other values

Agreement

LBRM is applied for MCH (excluding MCCH and MSI) with time interleaving.

  - FFS spec impact

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is derived based on DL-SCH as per TS 36.306 set by UE category.

  - FFS when the UE category has multiple maximum TBS

Agreement

The maximum number of transport block bits received within a TTI defined by the UE category should include the number of bits for unicast and broadcast, where the number of bits for broadcast is the one after being scaled up by N.

Agreement

The basic transmission pattern of PMCH with time interleaving consists of a set of (M x N) consecutive MBSFN subframes, excluding MCCH and MSI, where two transmissions of the same TB are separated by (M-1) MBSFN subframes, excluding MCCH and MSI.

Agreement

Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Agreement

RAN1 to further study the potential issue of mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes.

Agreement

The equation of k0 for time interleaving is modified to avoid puncturing of systematic bits.

- Further study the exact equation for k0 for the n-th subframe of a transmission belonging to a same TB k0n, n∈{0, 1, 2, …, N-1}), including

  - Option 1: for each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe.

  - Option 2: For each CB of the TB, follow the principle of NR TBoMS to determine k0n

  - Option 1 and Option 2 assumes all CBs of the TB have the same RV index for all CBs of the TB, indexed by k0n

- FFS whether different CBs of the TB mapped to each subframe may have different RV indices (e.g., indexed by different k0n per CB)

Agreement

The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows

- Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol

  - FFS: How to determine X

- Append y with <NULL> elements, as required, to obtain yapp

- Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0.

- FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations

- Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s.

- The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211.

FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

Agreement

The granularity of the frequency interleaver is 1 RE.

Agreement

For TBS scaling by N, the following is clarified for the rounding operation in option 1 and option 2 in previous agreement:

- In case two TBSs are at the same distance of TBS'×N, the larger TBS is chosen.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Agreement

Values of M = 16 and M = 8 are supported.

- FFS other values

- Note: some UE categories and depending on factors such as (value of N, bandwidth, MCS) may not be able to receive an interleaved MTCH with M=16.

Agreement

The previous agreement is modified as follows:

The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows

- Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol

  - FFS: How to determine X

    - Option 1: X is equal to the number of codeblocks that are mapped to the OFDM symbol

    - Option 2: X=32

    - Other options are not precluded.

- Append y with <NULL> elements, as required, to obtain yapp

- Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0.

- FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations

- Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s.

- The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211.

FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all

OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

  - RAN1#120-bis meeting

Agreement

Support at least the following values of time interleaver depth:

- Values M={4,8,16,32} and N={1,2,4,8,16} are supported.

  - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI.

- FFS: larger values, including M=64, N={32,64}.

Agreement

In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern:

- Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations.

  - May include the case of scheduling different M/N within the same MSI periodicity

  - FFS the values of the new periodicities.

- RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity

  - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity

  - Option 2: Drop the time-interleaved TBs which could not be successfully decoded.

    - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour)

  - Other Options not precluded

Agreement

The number of columns, C, of the frequency interleaver is C= #CBs Nsymb ×m, where #CBs denotes the number of CBs in the TB (including scaling, if any), Nsymb denotes the number of OFDM symbols of the PMCH,

- Working assumption:

  - m = Nsymbgcd(Nsymb,#CBs), and gcd(.) denotes the operation to compute the greatest common divisor.

Agreement

The following steps are applied to the rows of the frequency interleaver:

- Perform inter-row permutation with a reorder of the row indexes Ri', which is calculated as:

Ri'=R'i-1+d########################,          R'i-1≤R-d-1 #i×dR#,                 else.

With, 1≤i≤R-1, R'0=0, and d=R/R/RC-C+1.

- Perform a cyclic shift to the elements in the j-th row with an offset S(j) defined as:

Sj=modj+12×-1j,C, j=0,1,2,…,R-1

Agreement

LBRM parameters (e.g. NIR) are consistent between the broadcast network and the (multiple) UEs that are receiving the broadcast transmission.

- These multiple UEs may belong to different UE categories, thus having different values of Nsoft, NIR.

- FFS: how the consistent NIR and NCB is calculated.

Agreement

The agreement in the previous meeting is updated as follows:

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per PMCH configuration, e.g., via pmch-Config

- Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

  - Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching.

  - Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

Agreement

Obtain the initial TBS (𝑇𝐵𝑆′) as per current specifications, and calculate the final TBS as 𝑇𝐵𝑆=𝑟𝑜𝑢𝑛𝑑{𝑇𝐵𝑆′×𝑁}.

- The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.