3GPP TSG-RAN WG2 Meeting #130	R2-2504091

Malta, Malta, 19th – 23rd May, 2025

Agenda item:	8.9.3

Source:	Samsung

Title:	On procedures and open issues for CB-Msg3-EDT

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where Capacity enhancements for uplink:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In this contribution, we provide some discussions on the procedures and open issues on the objective of Support of Capacity enhancements for the uplink.

# Discussion

## Msg4 monitoring window and RNTI

In the last meeting it was discussed how the Msg4 monitoring window shall be designed. The discussion centered around two main proposals. First proposal proposed single Msg4 window and starting the window based on the end of the CB-Msg3 transmission window, and second proposal proposed multiple windows which are started from the end of each CN-Msg3 transmission:

-	session chair suggests to limit the discussion on FFS whether

A) the Msg4 monitoring always starts at the end of Msg3 transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not) or

B) We allow the network to configure one of the two options:

1)	The Msg4 monitoring starts at the end of Msg3 transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

2) 	The Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time) and is restarted after each replica plus UE-eNB RTT (FFS NW/UE processing time).  If the Msg4 monitoring window is overlapping with replica, FFS if the UE prioritize the replica transmission or monitoring

-	Nokia, ZTE and CMCC think that option 2 should be considered

-	Apple thinks we should not specify two options.

-	Ericsson thinks we should go for the simpler option (A). ESA agrees

-	HW also supports option B

-	NEC thinks A is sufficient. Oppo agrees. Viasat also agrees

The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

It was agreed in RAN2#129bis that at least the approach of single Msg4 window started based on the end of the CB-Msg3 transmission window shall be introduced. Then the question is whether the second option may also be introduced.

The main issue of the second option has to do with the half duplex issue. For the half-duplex issue, there will be a significant issue where a replica transmission will collide with the downlink reception. If the UE prioritizes the uplink replica transmission, then the network will not know when the UE is actually monitoring for Msg4, making the scheduling difficult. In fact the most reliable network implementation would be to transmit after the network can be certain that all of the replicas have been transmitted and configuring a long Msg4 window. This would then in fact be similar if not identical to the operation of starting the Msg4 monitoring window at the end of the CB-Msg3 transmission window, with the added drawback that the UE may have to monitor for CB-Msg4 for a lot longer durations. If the UE prioritizes monitoring for Msg4, then it is questionable whether the UE is actually performing DSA, since the UE will not really randomly select the CB-Msg3 replicas. Due to all of these limitations and drawbacks we believe that there is very little use of starting the CB-Msg4 monitoring window at the end of the CB-Msg3 uplink transmission.

Proposal 1: Do not introduce Msg4 monitoring window starting in the subframe containing the last (N)PUSCH repetition (plus UE-eNB RTT).

How to derive the RNTI was discussed in the last meeting. Some companies proposed that the RNTI is fixed for all transmission windows. While in principle it may work, there is the issue that CB-Msg4 monitoring windows from different Msg3 transmission windows overlap. For CB-Msg3-EDT this does not present as big of a problem as for random access as the UE determines whether a response is addressed to it based on the CRID, rather than the RAPID. However, it may cause issues if for instance fallback or Backoff Indication is introduced. In this case, the network may accidentally address UEs from CB-Msg3 transmission windows.

Proposal 2: Same RNTI is used for CB-Msg3 and Msg4 and it is derived based on the transmission window.

Furthermore, we do not see a reason why the CB-Msg4 monitoring window should not be a configurable timer as for RAR and for Msg4.

Proposal 3: CB-Msg4 monitoring window is a configurable timer. FFS on the value range.

## Various open issues

MAC-14:  FFS for the detail of HARQ operation on CB-Msg4.

In legacy random access, the UE will ACK or NACK the Msg4. This is required as otherwise the network will not know whether the Msg4 reception was successfully received, which is used whether the UE may be scheduled or whether for instance scheduling request resources will start to be used. The HARQ scheduling is based on PDCCH sent in Msg4.

For CB-Msg4, the network may schedule multiple UEs. This means that the UEs cannot all follow the HARQ scheduling in the PDCCH, as then there will be collision. This means that the HARQ ACK and NACK needs to be configured in the CB-Msg4 MAC message itself. This further implies that the Msg4 can only be acknowledged.

In 2-step random access, multiple UEs may also be scheduled and the HARQ resources are configured in the Success RAR. Therefore we believe that something similar can be pursued for CB-Msg3-EDT.

Proposal 4: The UEs will send HARQ ACK for CB-Msg4 based on scheduling in MAC subheader.

MAC-11: Whether a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in NB-IoT CP solution

In RAN2#129bis the following was agreed:

4.	RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT

-	For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject

-	For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject.

In EDT for both CP and UP solution, the EDT procedures are essentially RRC procedures that are completed based on receiving the appropriate RRC message. Last meeting it was proposed that in some cases only a L2 ACK may be able to complete the RRC procedure. For user plane, this would be difficult to achieve as the Msg4 will normally apply AS security and the associated RRC messages are complicated. For control plane, this may however be plausible as similar mechanism exist for PUR, whereby if Layer 1 ACK is received, then the UE completes the RRC procedures as if an empty RRCEarlyDataComplete is received. Thus it could be that if the UE receives a matching CRID and there being no RRC message that the EDT procedures can be completed. However, we do not think that the UE can only receive a CRID to complete the procedures. For instance, the CB-Msg4 message also needs to be acknowledged etc.

Proposal 5: For CB-Msg3-EDT CP solution, the RRC procedures can be completed (in MAC and RRC) if UE receives a matching CRID without an RRC message in a CB-Msg4. The CB-Msg4 still needs to be acknowledged.

Proposal 6: For CB-Msg3-EDT UP solution, the legacy EDT RRC procedures are followed, i.e the procedures can only be completed with an RRC message.

### CB-Msg4 header and payload design

There are a lot of similarities to CB-Msg3-EDT and 2-step random access. In 2-step random access, multiple UEs may be scheduled in the MsgB. If the MsgA (PRACH and PUSCH) of the 2-step random access is correctly received, the gNB will respond with a MAC subheader called successRAR. This successRAR can be seen in figure 2 below.

Figure 1.

Looking at the successRAR for 2-step random access many fields would be applicable to CB-Msg4:

- UE CRID. This is needed for CB-Msg4 as well.

- Channel Access-CPext. This is not needed to CB-Msg4.

- HARQ feedback Timing Indicator and PUCCH Resource Indicator. This gives the resources for the HARQ ACK for the MsgB. The HARQ resources also needs to be configured for CB-Msg4, but the exact fields may need to be confirmed by RAN1 and may also be different for NB-IoT and eMTC.

- Timing Advance Command. This is need for both the purpose of the UE moving to RRC connected as well for transmitting the HARQ ACK. This field occupies 12 bits in 5G NR, but in LTE RAR the field only occupies 11 bits.

- C-RNTI. This is mainly needed if the UE moves to RRC connected, but can also be used for contention resolution only.

There may be some optimizations, for instance introducing multiple subheaders for different cases, for instance if C-RNTI is present or not etc. However, we do not think that this would prove very useful, as then there would need to be an indication whether one or the other subheader is present.

Therefore, we propose that the RAN2 introduces a single MAC subheader which is used for all “successful cases”, i.e for completion of RRC procedure without RRC message for CP solution, for completing RRC procedure with RRC message, for falling back to RRC connected and for rejecting the UE. This subheader can be called CB-Success which contain the following fields:

Proposal 7: RAN2 introduces a single MAC subheader for the case of (MAC-layer) successful CB-Msg3 EDT procedure in CB-Msg4 with at least the following:   
    - UE CRID
    - HARQ Feedback resource configuration
    - Timing Advance Command
    - C-RNTI

Proposal 8: This single MAC subheader is used for all successful cases, i.e completing EDT RRC procedure without RRC message (CP solution), completing EDT RRC procedures with RRC procedure with RRC message, falling back to RRC connected and for rejecting the UE.

For the HARQ feedback resource configuration, for 2-step random access this field was designed by RAN1. We propose that RAN2 checks with RAN1 regarding how define this field.

Proposal 9: Send LS to RAN1 regarding design of the HARQ feedback resource configuration field.

RAN2 has currently not started designing the PDU format of the CB-Msg4 message. The MAC PDU format for DL-SCH and UL-SCH can be seen in the Figure 2 below. In LTE (as opposed to NR), the subheader for a MAC SDU is contained in the MAC header, which is in the beginning of the MAC PDU. We believe that this principle should be followed for CB-Msg4.

Figure 2.

For a successful CB-Msg4 case, we note that there may the following options for MAC SDUs CB-Msg4, which is different from 2-step random access:

- UE receives a matching CRID with no MAC SDUs (if proposal X is agreed).

- UE receives a matching CRID with one MAC SDU. This can be in the case of CP-solution or for UP solution which does not contain downlink data.

- UE receives a matching CRID with two MAC SDUs. This can be for UP-solution where CB-MSg4 contains the RRCConnectionResume and the downlink data.

For each MAC SDUs, a subheader indicating the length and LCID would required.

Observation 1: A CB-Msg4 may contain 0, 1 or 2 MAC SDUs addressed to it.

Furthermore, we also assume that other indications such as a Backoff indication may be sent in a CB-Msg4.

Proposal 10: CB-Msg4 may contain a backoff indication sub-header, which only contains the Backoff Indication.

We believe that a suitable design of the MAC PDU format for CB-Msg4 can be seen in Figure 3.

Figure 3.

Some of the key aspects of the MAC PDU for CB-Msg4 include:

- MAC header consists of optional Backoff indication, CB-Success and 0-2 sub-headers for MAC SDUs. The sub-headers for the MAC SDU for a UE follows the CB-Success subheader.

- MAC payload consist of MAC subPDUs for each UE along with padding. Each MAC subPDU consists of one or two MAC SDUs addressed to one UE.

Therefore we propose:

Proposal 11: MAC PDU for CB-Msg4 is specified as follows as a baseline (according to Figure 3):

- MAC header consists of optional Backoff indication subheader, CB-Success and 0-2 subheaders for LCID and length indicators for MAC SDUs. The sub-headers for the MAC SDU for a UE follows after the CB-Success.

- MAC payload consists of MAC subPDUs for each UE along with padding. Each MAC subPDU consists of one or two MAC SDUs addressed to one UE.

### Modelling of the CB-Msg3-EDT procedures

RRC-7) RAN2 to further discuss whether to differentiate CB-Msg3 EDT for CP solution and UP solution in the procedure.

During the CR reviews, there have been some discussions regarding how to model the CB-Msg3-EDT procedure. We believe that in essence, the CB-Msg3-EDT procedure is a special case or an enhancement of the EDT procedures. This is because the (MT-)EDT (CP-EDT and UP-EDT) procedures are specified across NAS, RRC and MAC. For instance, in Stage 2, the EDT procedures are mostly described from the perspective of S1/NG and RRC procedures of a single UE. However, there may be some challenges in some cases, as EDT is already referenced in some lower layer configurations. For instance UP-EDT or CP-EDT should be able to be a CB-Msg3-EDT or EDT with RACH procedure. This is also reasonable as the MAC layers reference UP-EDT and CP-EDT only in some specific cases, otherwise it only references EDT. Therefore we propose guiding proposal:

Proposal 12: The guiding principle of modelling CB-Msg3-EDT in relation to legacy EDT is that CB-Msg3-EDT is a lower layer enhancement or lower layer special case of EDT.

## CQI reporting in Msg3

CQI reporting in Msg3 during RRC resume and EDT is supported for both eMTC and NB-IoT. This is useful in order to schedule Msg4 and to assess the coverage situation if the UE is moved to RRC_CONNECTED. In NB-IoT it is supported by including the cqi-NPDCCH in RRCConnectionResumeRequest and in RRCEarlyDataRequest if cqi-Reporting is configured in SIB2. For eMTC it is supported by including the DCQR and AS RAI MAC CE which is configured by the mpdcch-CQI-Reporting in SIB2. For these cases, the CQI is measured on Msg2.

CQI reporting may also be useful during the contention-based Msg3, similar to how it is already useful during EDT. In particular it may be useful for scheduling Msg4 to multiple UEs, as it can indicate whether two UEs sending Msg4 during the same CB-Msg3 opportunity operate with different CQI. For NB-IoT it will not occupy further bits as the bits are already occupied in the RRCConnectionResumeRequest and RRCEarlyDataRequest, however for eMTC it may occupy further bits as it is included in a MAC CE. Therefore we propose the following:

Proposal 13: RAN2 to confirm CQI reporting in CB-Msg3 for CB-Msg3-EDT is supported.

# Conclusion

In this contribution we discussion further issues on contention-based Msg3:

Observation 1: A CB-Msg4 may contain 0, 1 or 2 MAC SDUs addressed to it.

Proposal 1: Do not introduce Msg4 monitoring window starting in the subframe containing the last (N)PUSCH repetition (plus UE-eNB RTT).

Proposal 2: Same RNTI is used for CB-Msg3 and Msg4 and it is derived based on the transmission window.

Proposal 3: CB-Msg4 monitoring window is a configurable timer. FFS on the value range.

Proposal 4: The UEs will send HARQ ACK for CB-Msg4 based on scheduling in MAC subheader.

Proposal 5: For CB-Msg3-EDT CP solution, the RRC procedures can be completed (in MAC and RRC) if UE receives a matching CRID without an RRC message in a CB-Msg4. The CB-Msg4 still needs to be acknowledged.

Proposal 6: For CB-Msg3-EDT UP solution, the legacy EDT RRC procedures are followed, i.e the procedures can only be completed with an RRC message.

Proposal 7: RAN2 introduces a single MAC subheader for the case of (MAC-layer) successful CB-Msg3 EDT procedure in CB-Msg4 with at least the following:   
    - UE CRID
    - HARQ Feedback resource configuration
    - Timing Advance Command
    - C-RNTI

Proposal 8: This single MAC subheader is used for all successful cases, i.e completing EDT RRC procedure without RRC message (CP solution), completing EDT RRC procedures with RRC procedure with RRC message, falling back to RRC connected and for rejecting the UE.

Proposal 9: Send LS to RAN1 regarding design of the HARQ feedback resource configuration field.

Proposal 10: CB-Msg4 may contain a backoff indication sub-header, which only contains the Backoff Indication.

Proposal 11: MAC PDU for CB-Msg4 is specified as follows as a baseline (according to Figure 3):

- MAC header consists of optional Backoff indication subheader, CB-Success and 0-2 subheaders for LCID and length indicators for MAC SDUs. The sub-headers for the MAC SDU for a UE follows after the CB-Success.

- MAC payload consists of MAC subPDUs for each UE along with padding. Each MAC subPDU consists of one or two MAC SDUs addressed to one UE.

Proposal 12: The guiding principle of modelling CB-Msg3-EDT in relation to legacy EDT is that CB-Msg3-EDT is a lower layer enhancement or lower layer special case of EDT.

Proposal 13: RAN2 to confirm CQI reporting for CB-Msg3-EDT is supported.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

R2-2502773, Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek), Mediatek, RAN2#129bis, Wuhan, China, April 2025.