3GPP TSG-RAN WG2 Meeting #130	R2-2504092

Malta, Malta, 19th – 23rd May, 2025

Agenda item:	8.9.4

Source:	Samsung, Iridium, Vivo, Thales

Title:	Acceptable cell camping for NB-IoT

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where the PWS component is the following:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome. Note: PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported.

In this contribution, we provide some discussions related to PWS broadcasting for NB-IoT.

# Discussion

## PWS broadcasting

Public Warning System broadcasting is a feature in 4G LTE and 5G NR that allows a network to broadcast public warning notifications. PWS supports Earthquake and Tsunami Warning System (ETWS) and Commercial Mobile Alert System (CMAS) messages which are sent over a set of System Information Blocks. ETWS uses the SIB10 and SIB11 in LTE and SIB6 and SIB7 in NR while CMAS uses SIB12 in LTE and SIB8 in NR. The network notifies UEs using Paging messages, which configures the UEs to acquire the related system information. The eNB or gNBs are configured via S1 or NG-AP to start broadcasting the PWS broadcasting.

## Idle mode operation for PWS broadcasting

- In eMTC and normal LTE, there are different types categories of cells that the UE may camp on. These are acceptable cells, suitable cells, barred cells and reserved cells, which can be summarized as follows:

- Acceptable cell:

  - Is a cell that UE may camp on for limited service. Limited service is for emergency calls and to receive PWS signalling. The only criteria is that the cell is not barred and that the cell selection criteria is fulfilled.

- Suitable cell:

  - The cell is a part of the UEs PLMNs. This is the most common category of cells that a UE will camp on. The cell cannot be barred and the cell selection criteria need to be fulfilled.

- Barred cell:

  - A barred cell, which may be according to system information or due to other procedures. UE cannot in any circumstances camp on the cell.

- Reserved cell:

  - May be indicated by system information.

- The procedure is usually that the UE searches for suitable cells and if the UE is unable to camp on a suitable cell, then the UE may search for acceptable cells. These acceptable cells are cells that are not within the UEs pre-configured PLMNs. The procedure to camp on any cell (acceptable cell) can be seen as part of the LTE idle mode procedures in Figure 1. This is a very important part of cellular operation that users rely on daily.

Observation 1: In eMTC and normal LTE, a UE can camp on an acceptable cell to receive PWS broadcasting and originate emergency calls. This is a crucial part of cellular operation that users rely on daily.

Figure 1. Procedure of going from camping on normal cell to camping on acceptable cell as part of idle mode procedures (from 5.2.2-1 from 36.304).

- As mentioned above, the cell category acceptable cell is for the purpose of camping on a cell for originating emergency calls and to receive PWS signalling. In NB-IoT, there are no standardized support for PWS signalling or emergency calls, as this was not part of the imagined NB-IoT use cases when NB-IoT was standardized in Release 13.

Observation 2: NB-IoT did not introduce camping on any cell or an acceptable cell category as emergency calls or PWS signalling was never supported.

- However, now that PWS signalling is being introduced for emergency scenarios, the original rationale for not introducing camping on any cell or acceptable cells no longer hold true. Furthermore, there could be a real use of having the ability for users who are not a part of the cells PLMN to camp on an NB-IoT NTN cell to receive PWS signalling – in fact this can potentially save lives in emergency situations. Without this, a large amount of NB-IoT UEs that are capable of receiving PWS signalling will not be able to receive it as the UEs are unable to even camp on the cell. Therefore we propose:

Proposal 1: A UE shall be able to camp on an NB-IoT NTN cell for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs.

Proposal 2: RAN2 to discuss introducing acceptable cell category for NB-IoT NTN.

# Conclusion

In this contribution, we discussed issues related to PWS signaling for NB-IoT.

Observation 1: In eMTC and normal LTE, a UE can camp on an acceptable cell to receive PWS broadcasting and originate emergency calls. This is a crucial part of cellular operation that users rely on daily.

Observation 2: NB-IoT did not introduce camping on any cell or an acceptable cell as emergency calls or PWS signalling was never supported.

Proposal 1: A UE shall be able to camp on an NB-IoT NTN cell for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs.

Proposal 2: RAN2 to discuss introducing acceptable cell category for NB-IoT NTN.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.