3GPP TSG-RAN WG2 Meeting #130	R2-2504102

Saint Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.1.2.3

Source:	Nokia

Title:	Reporting of applicability and inference configurations

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

While RAN2 has agreed on the baseline LCM procedures for positioning Case 1 (see the agreements below), several aspects such as the actual content of the messages, e.g., with regards to inference configurations and their impact to applicable functionality reporting, and the expected validity of the applicable functionality reports are still open.

In this paper, we discuss the issues due to these open aspects and provide our proposals for discussion and decision.

Agreements (RAN2#127-bis): 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Agreements (RAN2#129)

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

# 2	Discussion

## 2.1	Reporting of applicable functionality and inference configuration

In the LCM procedures for positioning Case 1, according to current understanding in RAN2, it is not clear how long the applicable functionality reported by the UE would remain unchanged. Furthermore, applicable functionality may depend on the information provided by LMF, e.g., in Step 3 (ProvideAssistanceData) and Step 5 (RequestLocationInformation) of the agreed procedures, whose contents are, however, still open.

To illustrate, the UE may determine that it has a suitable AI/ML model for the DL PRS configuration indicated in Step 3 (ProvideAssistanceData), and report applicable functionalities to LMF in Step 4 (ProvideCapabilities). Later, however, after receiving LPP RequestLocationInformation in Step 5, UE may realize that the AI/ML model it has identified may not actually support the requested measurement configuration, QoS requirement, or reporting configuration it received now in LPP RequestLocationInformation.

UE would then need to find and identify another model that is applicable for the requested configuration and expected to satisfy the indicated QoS requirements. Accordingly, UE would need to update LMF by sending yet another applicable functionality report.

Therefore, such an occurrence of change in applicable functionality after Step 5 (RequestLocationInformation) results in additional signaling overhead and delays in obtaining the location information, which may result in not meeting the scheduled location time for the positioning request. In other words, if the LPP ProvideAssistanceData in Step 3 and LPP RequestLocationInformation in Step 5 contain similar information as in legacy positioning, LCM becomes unstable and inefficient, resulting in signaling overhead, and hindering timely positioning of the UE, due to any changes in applicable functionalities.

To cope with this problem, LMF, in Step 3 (ProvideAssistanceData), in addition to the provided assistance data such as containing the DL PRS configuration, may already provide the UE with a set of candidate inference configurations, each indicating the requested location information with QoS requirements (e.g., accuracy, response time, etc.), requested measurements (if any), reporting configuration (e.g., reporting periodicity). Note that the final content of the inference configuration would depend on RAN1, however RAN2 can already agree on the signaling framework related to inference configurations. With this additional information relating to inference, UE could more accurately determine the applicable functionalities based on the candidate inference configurations and report the applicable inference configuration(s) to LMF in Step 4 (ProvideCapabilities). Accordingly, LMF, in Step 5 (RequestLocationInformation), can simply indicate the selected applicable inference configuration(s) to be used for inference. Optionally, when requesting inference, LMF may indicate an order of preference among the selected applicable inference configurations, in case an indicated configuration becomes later non-applicable at the UE (e.g., due to UE-side internal conditions).

In an alternative scenario, if UE is able to proactively determine which inference configurations it may apply before step 3 (ProvideAssistanceData), UE may also proactively inform LMF with potential inference configurations it is ready to apply, already in Step 2 (ProvideCapabilities). In this case, UE can indicate a set of potential inference configurations consisting of applicable PRS configurations, measurements, if any, reporting configurations, expected QoS, etc., to assist LMF determining the final assistance data and the location request it may send to the UE.

To further assist the LMF for performing LCM decisions, UE may provide additional information to LMF together with its applicable functionality report, such as indicating how long the applicable functionality is expected to be valid or remain unchanged (e.g., based on UE-side conditions), expected performance (i.e., expected accuracy performance of the model) and monitoring requirements (e.g., how frequently the monitoring data needs to be provided for performance monitoring depending on the expected model performance/accuracy) of the applicable functionalities. For example, UE may indicate the expected positioning accuracy under certain network conditions, or how often or under which conditions a performance monitoring needs to be performed so as to ensure desired performance.

Proposal 1: LMF may provide UE with candidate inference configuration(s), optionally with preference order, in Step 3 (ProvideAssistanceData) of the LCM procedure, based on which UE determines and reports applicable functionalities along with applicable inference configurations. LMF could then request inference in Step 5 (RequestLocationInformation) using one or more of the applicable inference configurations received in Step 4 (ProvideCapabilities). 
FFS: the content of a candidate inference configuration, which would depend on RAN1 (e.g., requested location information with QoS requirements (e.g., accuracy, response time, etc.), requested measurements (if any), reporting configuration (e.g., reporting periodicity), in addition to the assistance data such as containing the DL PRS configuration or any NW-side additional conditions).

Proposal 2: UE may proactively inform LMF with potential inference configuration(s) it may apply, in Step 2 (ProvideCapabilities) of the LCM procedure, before receiving any assistance data or inference configuration from LMF.

Proposal 3: In the LCM procedure, when reporting applicability, UE may provide additional information such as the expected validity of the report, and expected QoS or monitoring requirements of the functionality, to assist LMF for making LCM decisions.

# 3	Conclusion

This document has proposed the following:

Proposal 1: LMF may provide UE with candidate inference configuration(s), optionally with preference order, in Step 3 (ProvideAssistanceData) of the LCM procedure, based on which UE determines and reports applicable functionalities along with applicable inference configurations. LMF could then request inference in Step 5 (RequestLocationInformation) using one or more of the applicable inference configurations received in Step 4 (ProvideCapabilities). 
FFS: the content of a candidate inference configuration, which would depend on RAN1 (e.g., requested location information with QoS requirements (e.g., accuracy, response time, etc.), requested measurements (if any), reporting configuration (e.g., reporting periodicity), in addition to the assistance data such as containing the DL PRS configuration or any NW-side additional conditions).

Proposal 2: UE may proactively inform LMF with potential inference configuration(s) it may apply, in Step 2 (ProvideCapabilities) of the LCM procedure, before receiving any assistance data or inference configuration from LMF.

Proposal 3: In the LCM procedure, when reporting applicability, UE may provide additional information such as the expected validity of the report, and expected QoS or monitoring requirements of the functionality, to assist LMF for making LCM decisions.