3GPP TSG-RAN WG2 Meeting #130						        											R2-2504112

St.Julians, Malta, May 19 – 23, 2025

Source: 	ZTE Corporation, Sanechips, Continental Automotive, Tejas Networks, Nokia, Ofinno, Samsung, Fujitsu, Kyocera, Sony, ETRI, III, Ericsson, Nordic Semiconductor ASA

Title: 	Details of option A for overall A-IoT MAC procedure (Open Issue 1-1)

Agenda item:	8.2.2

Document for:	Discussion and Decision

# Introduction

At RAN2#129-bis, the device behaviour if it gets a new service request while one procedure is still ongoing was discussed (R2-2502211). Two options were considered:

Option A: Ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure

Option B: terminate the ongoing procedure and respond to the latest request.

It was agreed that the R2D message design will be extensible to accommodate devices and features of future release. However, the solution for device behaviour was left as FFS. This contribution discusses the details of the device behaviour and proposes a way forward.

# Details of option A with CBRA

The agreed RAN2 framework for A-IoT is based on the existence of a transaction ID in the paging message for CBRA. Although there is an implicit assumption that there may be an ongoing procedure for A-IoT either for inventory or for command, there was no agreement on when the device shall consider the ongoing procedure is considered to be complete. First, we need to agree on an end condition hence for the ongoing procedure. Multiple options were discussed for the end-of procedure indication. These include:

- Using the trigger message as an implicit end indicator

- Defining a new end-of-procedure message

- Using a timer (either on its own or in combination with the above options)

Although all options are feasible in theory, option 1) above seems to be the most promising one because of the following:

- In case of CBRA trigger message(s) are anyway sent by the reader to device when the reader moves on to the next (set of) device(s) – so, this is a natural end point

- Trigger messages are light weight and hence minimize the overhead

- Multiple trigger messages can be transmitted by the reader (in CBRA) which would improve the robustness of the procedure against missed messages

Based on the above, considerations, we focus mainly on option 1) for the further analysis in this document and propose the following general approach for option A:

Proposal 1a: When a device has an ongoing A-IoT procedure, it ignores any paging request until the A-IoT procedure is considered completed (i.e. option A).

Proposal 1b: The R2D trigger message is used as an implicit end indicator for an ongoing procedure

Given that the main point of option A is that the device only responds to ongoing procedure, the trigger message should carry an indication of the ongoing procedure, i.e. the transaction ID. This can be included either explicitly (as a MAC payload) or preferably (considering the overhead), in some lower layer signalling implicitly (e.g. either in the CRC or in the R2D preamble etc – up to RAN1).

Proposal 2: The R2D trigger message includes the transaction ID either explicitly as MAC payload or preferably, implicitly in physical layer signalling (e.g. mask the CRC with transaction ID or include it implicitly in the R2D preamble if feasible – details up to RAN1 – send an LS to RAN1 asking them to implement this)

# Inventory procedure with option A

With the above framework in place, in this section we look at the overall procedure for inventory. The overall procedure is depicted in Figure 1 below.

Figure 1: Option A: Inventory procedure

Based on the above, the following proposals are made for inventory procedure:

Proposal 3: In case of inventory procedure for CBRA:

- The device assumes that the inventory procedure is ongoing once MSG1 is transmitted and the procedure is considered complete upon receiving a subsequent R2D trigger with the same transaction ID as the one included in the paging message that triggered CBRA

- Upon receiving the subsequent R2D trigger message after transmitting MSG1:

  - the inventory procedure is considered to be completed successfully if msg3 was transmitted and no NACK was received. In this case the device will not respond to the same transaction ID (i.e. no re-access)

  - The inventory procedure is considered to be completed unsuccessfully for all other cases (e.g. MSG1 failure, MSG2 failure, MSG3 failure). In this case the device will respond to paging for any transaction ID (including the same transaction ID – i.e. re-access)

# Command procedure with option A

Whilst inventory procedure is fairly simple as noted in section 3.1, command procedure needs a bit more analysis because of the latest framework agreed in SA2. Specifically, SA2 agreed the so called 2-step command procedure as follows:

Step 1:

- CN first sends inventory request to the reader

- Reader sends UE ID(s) – inventory response

Step 2:

- CN then sends commands (unicast) to each of the device that successfully inventoried

Any option for command over the A-IoT radio interface hence, should fit within the above framework. Thus, there are the following options for the reader to execute the command procedure.

Command Option A-1: Finish inventory and command between two consecutive trigger messages:

Command Option A-1 is depicted in Figure 2 below. In this option, the whole procedure is completed between two trigger messages. The advantage of this option is that this doesn’t need the device to be aware of the difference between command and/or inventory. The reader will simply process devices in groups between two trigger messages. The total number of devices that are processed between two trigger messages depend on the number of access occasions (i.e. time x frequency resources) for MSG1 between two trigger messages. The drawback of this approach is that there one a CN round trip delay between every set of trigger messages. So, this approach may not be suitable if there is a very large number of MSG1 time slots (distributed across a large number of trigger messages).

Figure 2: Command option A-1

Command Option A-2: Finish the inventory first and then transmit the command later:

In this option, the inventory procedure is completed first and the device may need to retain the ASID until the command procedure is complete. Once there is a subsequent trigger message after the command, the device will consider the overall procedure to be completed. This approach is depicted in Figure 3 below.

Figure 3: Command option A-2

Given the different use cases for command options A-1 and A-2 (based on the number of devices), it is proposed to enable both options. However, it should be noted that command option A-1 comes for free once option A-2 is enabled – i.e. option A-1 is a reader implementation choice. Based on this, the following proposals are made:

Proposal 4: RAN2 to discuss and choose between the following for “Inventory + Command”:

- Option A-1: Inventory + Command procedure is completed between two consecutive R2D triggers (see Figure 2) – command indication is not needed in paging message for this option: or

- Option A-2: Inventory step is completed first (between two consecutive R2D triggers) and command is transmitted afterwards (see Figure 3) – a command indication is needed in the paging message for this option: or

- Support both options: Support both Option A-1 and Option A-2

Note: detailed proposals for the above options are provided in Annex A.

# Handling error cases

One advantage of using the trigger message as the end-of-procedure indication is that in CBRA, there are multiple trigger messages and even if the device misses one of these, there is inherent robustness due to the fact that there will always be a subsequent message. This however may not apply to the devices that respond towards the end of the CBRA procedure (i.e. during the last MSG1 slot). This may be problematic if the reader then moves on to a different service (i.e. transaction ID). However, to mitigate the risk of this happening, the reader can send one (or more) R2D trigger messages to ensure that no stuck devices are left, especially before moving to a new transaction ID.

Of course, it may be argued that regardless of any number of repeated R2D triggers being sent, the device may still miss them all and hence remain stuck. In this case, the reader may still adopt other means such as turning the CW node off to prevent any hanging devices.

If everything fails, the hanging devices will still be back online once the transaction ID space wraps around (and the device will eventually see a new trigger with the relevant transaction ID).

Thus, it can be left to reader implementation to avoid the hanging devices. If RAN2 thinks that such possibility of stuck devices (however remote it is) is not acceptable, then the final option would be to have a timer at the device side. However, this option should only be considered if it is deemed that none of the above options are reasonable.

Observation 1:

In order to minimize the risk of the devices missing one or more R2D trigger messages and being “stuck”, the reader may adopt multiple strategies:

- Send one or more redundant R2D trigger messages with or without a subsequent paging with the same transaction ID before moving to a new transaction ID

- Turning the CW off to clear the device states before moving to a new transaction ID

- Rely on the transaction ID wrap around

Proposal 5: It is up to the reader implementation to handle the “stuck” devices (see observation 1). If none of the above options in observation 1 are deemed to be sufficient, RAN2 may discuss whether a device based solution, (e.g. using a timer) may be used

# Details of option A with CFRA

Firstly, the CFRA procedure comes with dedicated resources in the paging message. Thus, the CFRA procedure may be considered complete once inventory response is transmitted for inventory case and once command response is transmitted for inventory+command case.

Proposal 6. The procedure initiated for CFRA can be considered to be complete once inventory response/command response is transmitted by the device for inventory/inventory+command case respectively.

# Issues with option B

RAN2 agreed that devices will listen to DL all the time as long as there is sufficient energy. With this, the assumption is that any R2D message may be received and processed by the device. With this framework, we have to consider how option A and option B work. Whilst option B may look simple at the outset, there are a few issues with this option that need to be considered:

- For Rel-19:

  - An ongoing procedure from one reader may be interrupted by another reader.

    - This will result in race conditions where the procedure initiated by one reader may never terminate if another reader is in the range of the same device.

    - Since the coverage of the Rel-19 is small, in order to perform a given service in a given area, a typical scenario is to employ multiple readers and to initiate the A-IoT procedure to speed-up the overall process and if the procedures from the readers are interrupted by other then there may be cases where the procedures may never successfully complete. The only option then would be essentially to perform the service with one reader at a time and this will prolong the overall service completion times.

  - SA2 have agreed a 2-step command procedure where the command is delivered after the inventory report is received. This means that there is a CN round trip time between the inventory report and command and this wait time increases the likelihood of the ongoing procedure being interrupted by another reader

  - Since MSG2 transmission and any retransmission of MSG2 can happen at any point in time (until next paging message), the device (even if MSG3 was successfully received earlier) will retransmit MSG3 as long as the RN16 value matches in MSG2 and hence cause MSG3 collisions. The probability of this MSG3 collision will be worse towards the end of the paging round – thus MSG2 reception would benefit from a predefined end point similar to option A in any case.

  - The same issue is also applicable to NACK for MSG3 (i.e. any device that receives NACK with a matching device ID – i.e. RN16 – will retransmit MSG3 after subsequent paging if NACK is received – even if its MSG3 transmission was successful) and the probability of this issue happening increases towards the end of the paging round when any device that would have transmitted the MSG3 with the same device ID will respond to the NACK.

  - If the device(s) miss the subsequent paging message, these issues will extend to the subsequent paging round and devices will interfere and cause contention in the in this paging round too.

- For Rel-20 and beyond:

  - In case of topology 2, the typical use case would be to receive a service request from the CN and to configure multiple UE based readers with the service request. The readers then go off and perform the procedure and report the results. Thus, the likelihood of multiple readers performing services in a given area will actually be a typical case and option B is not compatible with this in topology 2.

  - Outdoor deployments are also being discussed for Rel-20 and beyond and this further increases the cases where the devices will be in range of multiple readers.

- System performance implications:

  - In general, system performance is degraded as energy consumption increases due to the A-IoT procedures being prolonged by multiple readers

  - Radio resources are wasted since a procedure initiated and about to be completed by one reader is terminated by another reader. The reader may have to schedule resources for D2R transmission multiple times before it can conclude that the device might have left for another procedure.

Observation 2:

If the device is allowed to terminate the ongoing procedure upon receiving any new request from any reader, it results in the following issues:

- Race conditions where procedure initiated by one reader is terminated (repeatedly) by another

- Issues with MSG3 collision and NACK collision which gets progressively worse towards the end of the paging round and may extend to next paging round if some of the devices miss the subsequent paging message

- Prolong the overall service duration as essentially a single reader can perform the service in a given region at a given time

- Interruption to command service is even more likely due to the extended duration between the inventory report and command with the 2-step command procedure agreed by SA2

- Not forward compatible as the above multi reader issues are even more problematic with the typical use cases in topology 2 as well as outdoor deployments

- System performance is degraded due to increased energy consumption (not only from network but also from device perspective), wasted radio resources, and increased implementation complexity

# Proposals and observations

The following observations and proposals (all related to open issue 1-1) are made:

Option A overview

Proposal 1a: When a device has an ongoing A-IoT procedure, it ignores any paging request until the A-IoT procedure is considered completed (i.e. option A).

Proposal 1b: The R2D trigger message is used as an implicit end indicator for an ongoing procedure

Proposal 2: The R2D trigger message includes the transaction ID either explicitly as MAC payload or preferably, implicitly in physical layer signalling (e.g. mask the CRC with transaction ID or include it implicitly in the R2D preamble if feasible – details up to RAN1 – send an LS to RAN1 asking them to implement this)

Inventory procedure

Proposal 3: In case of inventory procedure for CBRA:

- The device assumes that the inventory procedure is ongoing once MSG1 is transmitted and the procedure is considered complete upon receiving a subsequent R2D trigger with the same transaction ID as the one included in the paging message that triggered CBRA

- Upon receiving the subsequent R2D trigger message after transmitting MSG1:

  - the inventory procedure is considered to be completed successfully if msg3 was transmitted and no NACK was received. In this case the device will not respond to the same transaction ID (i.e. no re-access)

  - The inventory procedure is considered to be completed unsuccessfully for all other cases (e.g. MSG1 failure, MSG2 failure, MSG3 failure). In this case the device will respond to paging for any transaction ID (including the same transaction ID – i.e. re-access)

Command procedure

Proposal 4: RAN2 to discuss and choose between the following for “Inventory + Command”:

- Option A-1: Inventory + Command procedure is completed between two consecutive R2D triggers (see Figure 2) – command indication is not needed in paging message for this option: or

- Option A-2: Inventory step is completed first (between two consecutive R2D triggers) and command is transmitted afterwards (see Figure 3) – a command indication is needed in the paging message for this option: or

- Support both options: Support both Option A-1 and Option A-2

Handling stuck devices

Observation 1:

In order to minimize the risk of the devices missing one or more R2D trigger messages and being “stuck”, the reader may adopt multiple strategies:

- Send one or more redundant R2D trigger messages with or without an a subsequent paging with the same transaction ID before moving to a new transaction ID

- Turning the CW off to clear the device states before moving to a new transaction ID

- Rely on the transaction ID wrap around

Proposal 5: It is up to the reader implementation to handle the “stuck” devices (see observation 1). If none of the above options in observation 1 are deemed to be sufficient, RAN2 may discuss whether a device based solution (e.g. using a timer) may be used

CFRA

Proposal 6. The procedure initiated for CFRA can be considered to be complete once inventory response/command response is transmitted by the device for inventory/inventory+command case respectively.

Issues with option B

Observation 2:

If the device is allowed to terminate the ongoing procedure upon receiving any new request from any reader, it results in the following issues:

- Race conditions where procedure initiated by one reader is terminated (repeatedly) by another

- Issues with MSG3 collision and NACK collision which gets progressively worse towards the end of the paging round and may extend to next paging round if some of the devices miss the subsequent paging message

- Prolong the overall service duration as essentially a single reader can perform the service in a given region at a given time

- Interruption to command service is even more likely due to the extended duration between the inventory report and command with the 2-step command procedure agreed by SA2

- Not forward compatible as the above multi reader issues are even more problematic with the typical use cases in topology 2 as well as outdoor deployments

- System performance is degraded due to increased energy consumption (not only from network but also from device perspective), wasted radio resources, and increased implementation complexity

# References

- RAN2#129-bis, Chair notes

# Annex A – Detailed proposals

If only option A-1 is supported:

- The entire procedure is completed between two consecutive triggers. So, the device procedure for inventory+command is exactly same as the procedure for inventory case (Proposal 3 then covers all cases)

If only option A-2 is supported:

- The reader indicates command type in the paging message

- If inventory step is considered to be successful (see P3), the device stores ASID and waits for command (if inventory step is unsuccessful, the device performs reaccess – again see P3)

- if a subsequent R2D trigger message (with the same transaction ID) is received after the R2D command message, the ongoing (inventory+command) procedure is considered to be complete

If both option A-1 and A-2 are supported (i.e. it is up to the reader which option to use):

- The reader indicates command type in the paging message

- Once MSG1 is transmitted, if a subsequent R2D trigger message (with the same transaction ID) is received before the R2D command message:

  - the device stores the ASID if inventory procedure is successful (see P3) and waits for the command message

  - the device performs reaccess if inventory step is considered unsuccessful (see P3)

- Once MSG1 is transmitted, if a subsequent R2D trigger message (with the same transaction ID) is received after the R2D command message, the ongoing (inventory+command) procedure is considered to be complete.